Acceptance Test Procedure for the MFD-268C4B Display Head Module and Control Card

Document Number:	946-4DC0-001

Revision:	-

CAGE Code:	0EFD0

Rockwell Collins

The contents of this document are proprietary to Rockwell Collins and shall not be disclosed, disseminated, copied, or used except for purposes expressly authorized in writing by Rockwell Collins.

2026 Rockwell Collins, All rights reserved.

|  | NAME | TITLE | APPROVAL |
| --- | --- | --- | --- |
| Prepared by: | Shaun C. Ochsner | Preparer | N/A |
| Approved by: | Alan Kuefler | Electrical Engineer | On File |
| Approved by: | Paul C. Smith | Technical Project Manager | On File |
| Approved by: | David F. Dickie | System Engineer | On File |
| Approved by: |  | Design Assurance Center Engineer | On File |

Revision History

| REV | DESCRIPTION | DATE | APPROVED |
| --- | --- | --- | --- |
| - | Initial Release |  | S.C.Ochsner |

# Contents

1	Scope	6

2	Reference Information	7

2.1	Reference Information—UUT	7

2.1.1	Specifications	7

2.1.2	Drawings	7

2.1.3	Abbreviations	8

2.2	Test Conditions and Special Procedures	8

2.2.1	Environmental	8

2.2.2	Power Requirements	9

2.2.3	Grounding Requirements	9

2.2.4	Operational Duty Cycle	9

2.2.5	Warm-up Period	9

2.2.6	Test Condition and Tolerances	9

2.2.7	Default Input Conditions	10

2.2.8	Recommended Power Up Procedure	10

2.2.9	Recommended Power Down Procedure	11

2.3	Data Archive	11

2.4	Formulas	12

2.4.1	Chroma Formula	12

2.4.2	LED Current Formula	12

2.4.3	A/D Converter Formulas	13

2.4.4	D/A Converter Formulas	14

2.4.5	Etched ITO Temperature Sensor Formulas	14

2.4.6	Temperature Sensor Formulas	16

2.4.7	LCD Heater Formulas	16

2.5	Serial Communications	17

2.6	Calibration Data Load	17

2.7	Vesta FPGA Programming	18

2.8	Memory Reads/Writes	18

3	Connector Interfaces	19

3.1	Functional Connectors	19

3.1.1	REIM Interface Power Connector	19

3.1.2	REIM Interface I/O Connector	19

3.1.3	LCD Connector	19

3.1.4	LED Backlight Connectors	19

3.1.5	Bezel Connector	19

3.1.6	LCD Heater Connectors	19

3.1.7	Fan Connector	19

3.2	Test Connectors	20

3.2.1	Main Test Connector	20

4	Test Specifications	22

4.1	Control Card Test Specifications	22

4.1.1	Test Local Power Supply Operation	23

4.1.2	Test Reference Voltages	23

4.1.3	Test Holdup Supply	24

4.1.4	Test SYS_RST_F	25

4.1.5	Program Vesta FPGA	25

4.1.6	Load Default Calibration Data	25

4.1.7	Test Discrete Inputs	26

4.1.8	Test Discrete Outputs	27

4.1.9	Test Backlight Power Supply	28

4.1.10	Test Analog to Digital Conversion Circuitry	30

4.1.11	Test Digital to Analog Conversion Circuitry	31

4.1.12	Test UNIT_VOLTAGE/UNIT_CURRENT	33

4.1.13	Test Manual Brightness Analog Input	34

4.1.14	Test PFM Control	34

4.1.15	Test A429 Input/Output	36

4.1.16	Test Control Card Temperature Sensor	37

4.1.17	Test LCD Power Switch	37

4.1.18	Test LCD Gate Feedback Hardware Monitor	38

4.2	Display Head Test Specifications	39

4.2.1	Test ITO Temperature Sensors	39

4.2.2	Test LCD Heater	41

4.2.3	Test Pixel Bus and LCD	45

4.2.4	Test DVI Input (987-8392-050 only)	46

4.2.5	Test Keypanel	47

4.2.6	Test Backlight Temperature Sensor	50

4.2.7	Test Fan	50

4.2.8	Test Chromaticity	51

4.2.9	Test Backlight Luminance	55

4.2.10	Test Contrast	58

4.2.11	Load DHM CPN	59

4.2.12	Test Faults	60

5	Key Characteristics	61

# Figures

Figure 2-1 Calibration File Example	17

Tables

Table 1-1, Applicable Hardware	6

Table 2-1, Power Requirements	9

Table 4-1, 987-8392-050 Chromaticity Targets	51

Table 4-2, 987-8392-054 Chromaticity Targets	52

Table 4-3, Display Head Part Number Memory Format	59

Equations

Equation 1, Chroma Tolerance	12

Equation 2, White LED Peak Current (per string)	12

Equation 3, Red LED Peak Current (per string)	12

Equation 4, Blue LED Peak Current (per string)	12

Equation 5, Positive A/D Measurements	13

Equation 6, Negative A/D Measurements	13

Equation 7, A/D Scale Factor	13

Equation 8, DAC Voltage Conversion	14

Equation 9, DAC Setting Conversion	14

Equation 10, ITO Sensor Slope	14

Equation 11, Sensor Resistance at 25°C	14

Equation 12, ITO Sensor Offset	15

Equation 13, Positive A/D Measurements (upper 8 bits)	15

Equation 14, Negative A/D Measurements (upper 8 bits)	15

Equation 15, ITO Sensor Look Up Table Entry Calculation	16

Equation 16, Temperature Sensor Conversion	16

Equation 17, ITO Heater Current Conversion	16

Equation 18, Heater Power Calculation	16

# Scope

These test specifications were written specifically for the MFD-268C3A, MFD-268C4A and MFD-268C6 display head and control card version indicated in Table 1-1. This specification may be utilized for other display head and control card versions not specifically indicated in this document as appropriate.

This document’s purpose is to provide the test specifications for the control card functional tests and the display head level functional tests and calibration.

The figures contained within this document are for reference only.

Table 1-1, Applicable Hardware

| CPN | Unit Type | Comment |
| --- | --- | --- |
| 987-8392-050 [6] | MFD-268C3A/C6 | Both MFD’s use identical DHM hardware and calibration data |
| - 966-7753-002/003 [7] | MFD-268C3A/C6 | Control card assembly |
| - 987-8392-054 [11] | MFD-268C4A | Display Head Assembly |
| - 966-7753-102/103 [12] | MFD-268C4A | Control card assembly |

# Reference Information

## Reference Information—UUT

### Specifications

MFD-268C4B Display Control Card Hardware Design Document (RCPN 946-44A4-001)

Internal Handling Procedure for Electrostatic Discharge Sensitive Devices and Assemblies (RCPN 489-0004-001)

Interface Control Document for the Standard Head Interface Protocol for Display Head Modules (RCPN 963-0981-001)

Hera2 FPGA Reference Manual (RCPN 946-48N6-001)

Acceptance Test Method (ATM) for the MFD-2684C4B Display Head Module and Control Card (RCPN 683-XXXX-001)

### Drawings

Display Head Assembly, MFD-268C4B (RCPN 987-1003-101)

Circuit Card Assembly, Control Card, MFD-268C4B (RCPN 828-4D89-001)

Circuit Card Assembly, LED Backlight, MFD-268C4B (RCPN 828-4D91-001)

Schematic Diagram, Control Card, MFD-268C4B (RCPN 828-4D90-001)

Schematic Diagram, LED Backlight, MFD-268C4B (RCPN 828-4D92-001)

### Abbreviations

The following abbreviations are used within this document:

| Term | Definition |
| --- | --- |
| ADC ARINC ATM | Analog to Digital Converter Aeronautical Radio, Incorporated Acceptance Test Method |
| ATP | Acceptance Test Procedure |
| CCA CPN | Circuit Card Assembly Collins Part Number |
| DAC DEC2HEX DHM | Digital to Analog Converter Decimal to Hexadecimal Display Head Module |
| DVI EAR | Digital Visual Interface Export Administration Regulation |
| ESD FPGA | Electrostatic Discharge Field Programmable Gate Array |
| GND | Ground |
| Hz | Hertz |
| I/O ITO JTAG | Input / Output Indium Tin Oxide Joint Test Action Group |
| LCD LED | Liquid Crystal Display Light Emitting Diode |
| MFD | Multi-Function Display |
| N/A NVG | Not Applicable Night Vision Goggle |
| NVIS NVM PFM | Night Vision Imaging System Non Volatile Memory Pulse Frequency Modulation |
| PWM | Pulse Width Modulation |
| PWR | Power |
| RCPN | Rockwell Collins Part Number |
| REIM SHIP | Rear Electronics Interface Module Standard Head Interface Protocol |
| TTL UXGA | Transistor Transistor Logic Ultra Extended Graphics Array |
| UUT | Unit Under Test |
| VAC | Volts Alternating Current |
| VDC XGA | Volts Direct Current Extended Graphics Array |

## Test Conditions and Special Procedures

There is no security requirement associated with the testing of this assembly.

Unless otherwise stated, all tests shall be performed under the conditions defined in the following subsections.

### Environmental

Temperature, humidity, and atmospheric pressure will be normal factory ambient.

The MFD display head has a cooling fan. No external cooling air is required.

### Power Requirements

The unit under test (UUT) requires the input voltages identified in Table 2-1.

Table 2-1, Power Requirements

| Description | Voltage | Comment |
| --- | --- | --- |
| +28VDC_BL | +29 VDC ± 2 VDC, 2.0 A (except where specified by test) Adjustable from 14.0 VDC to 33.0 VDC |  |
| +28VDC_HTR | +29 VDC ± 2 VDC, 4 A (except where specified by test) Adjustable from 16.0 VDC to 33.0 VDC |  |
| +5VDC | +5 VDC ± 0.25 VDC, 2.0 A |  |
| +12VDC | +12 VDC ± 0.6 VDC, 0.5 A |  |
| -12VDC | -12 VDC ± 0.6 VDC, 0.5 A |  |
| 5VAC_H/L-A | 5 VAC ± 0.25 VAC, 400 ± 40 Hz, 50 mA Adjustable from 0.0 to 5.0 VAC | Keypanel uses LED lighting, 5VAC_H/L-A is used as a dimming control analog, 5VAC_H/L-B is not used |
| 115VAC_H/L | 115 VAC ± 5.75 VAC, 400 ± 40 Hz, Single phase, 2.0 A | Cooling fan power |

### Grounding Requirements

There is no ground isolation in the UUT design. All signals are referenced to a common ground.

The assembly under test contains electrostatic discharge (ESD) sensitive devices which require handling in accordance with the internal handling procedure [2].

### Operational Duty Cycle

Continuous.

### Warm-up Period

No warm-up period is required before verifying the requirements in this document.

### Test Condition and Tolerances

Unless otherwise specified, all applied voltages (excluding those in Table 2-1) shall have a tolerance of ±2%.

For optical tests, the photometer shall be at the specified angle +/- 2.0 degrees.

Unless otherwise specified, TTL logic low is defined as less than or equal to 0.8 V and logic high is defined as greater than or equal to 2.4 V but less than 5.2 V. An OPEN state is defined as 100 kohm or greater with respect to ground. A GND state is defined as 50 ohms or less with respect to ground.

Minimum input impedance for equipment measuring voltages with test connector shall be 1 Mohm.

### Default Input Conditions

Unless otherwise indicated, discrete inputs should be in the following states by default:

- PWR_WARN_F = GND

- DHM_LOAD_F = GND

- HEATER_ENABLED = GND

- DHM_SHOP_F = GND

- NVIS_ENB_F = OPEN

Unless otherwise indicated, 5 VAC keypanel dimming voltage should be off or 0 VAC.

Unless otherwise indicated, +28VDC_HTR should be off or 0 VDC.

### Recommended Power Up Procedure

The low voltage power voltages +12VDC, -12VDC, and +5VDC should be powered up together. Low voltage power voltages (including test equipment logic drive circuitry) should be powered up at the same time or before backlight input power. Test equipment circuitry used to drive the TTL inputs of the control card should be powered up at the same time as the low voltage power is applied to the control card. This may include +3.3 VDC and/or other voltages depending on test equipment circuitry. Heater input power should be powered up at the same time or after the low voltage power voltages.

The following procedure is recommended to power up the control card/display head:

- Set the display discrete inputs to their default states

- Apply power to +12VDC, -12VDC, +5VDC, and test equipment logic drive circuitry together

- Apply nominal power to +28VDC_BL (and +28VDC_HTR if needed)

- Set PWR_WARN_F to OPEN

### Recommended Power Down Procedure

Heater and backlight power should be powered down at the same time or before the low voltage power voltages. The low voltage power voltages (including test equipment logic drive circuitry) should be powered down together. Test equipment circuitry used to drive the TTL inputs of the control card should be powered down at the same time as the low voltage power is removed from the control card. This may include +3.3 VDC and/or other voltages depending on test equipment circuitry. To ensure the LCD shuts down properly, display head level tests should always set PWR_WARN_F to GND prior to removing low voltage power. Failure to do so could cause the LCD to flicker at the next power up.

The following procedure is recommended to power down the control card/display head:

- Set PWR_WARN_F to GND

- Power down +28VDC_BL (and +28VDC_HTR)

- Power down +12VDC, -12VDC, +5VDC, and test equipment logic drive circuitry

- Set discrete inputs back to the default states

## Data Archive

It is suggested that at least the following data should be recorded and stored in a retrievable and sortable format (Note: archiving is not required when the tests are performed manually):

Pass/fail status and measurements for all tests performed.

Calibration date and time.

CCA part number and serial number.

Display head part number and serial number when performing display head level test and calibration.

Calibration data file part number.

Minimum and maximum luminance for day and NVIS mode

Power supply currents for +5 VDC, +12 VDC, and -12 VDC.

Chromaticity u’ and v’ for colors: white, red, green, blue.

LED currents at maximum day luminance for all of the operating LED colors

## Formulas

### Chroma Formula

Equation 1 is used to calculate the chroma tolerance.

Equation 1, Chroma Tolerance

Radius

where:

u’erp = 1976 CIE-UCS u’- nominal u’

v’erp = 1976 CIE-UCS v’- nominal v’

- DELTA C*

where:

u’erp = 1976 CIE-UCS u’- nominal u’

v’erp = 1976 CIE-UCS v’- nominal v’

where Yn=measured white luminance.

### LED Current Formula

The LED peak current for white LED color can be calculated using Equation 2 where x is the 12 bit DAC setting.

Equation 2, White LED Peak Current (per string)

PeakCurrent=21*x4095+4

### A/D Converter Formulas

Converting A/D readings

For A/D data between 0H and 7FFH (inclusive), the measurement is positive and should be converted to a voltage value using Equation 3 where x is the decimal equivalent of the converted 12 bits.

Equation 3, Positive A/D Measurements

A2D_input_voltage=5*x2048

For A/D data between 800H and FFFH (inclusive), the measurement is negative and should be converted to a voltage value using Equation 4 where x is the decimal equivalent of the 2’s complement of the converted 12 bits.

Equation 4, Negative A/D Measurements

A2D_input_voltage=(4096-x)*-52048

- An individual scale factor is applied to the voltage measured at each input to determine the final input voltage as defined by Equation 5. Refer to Table 5-1 in section 5.6 of the hardware design document [1].

Equation 5, A/D Scale Factor

Volts=A2D_input_voltage*scale_factor

### D/A Converter Formulas

DAC settings can be converted to a voltage using Equation 6. Note that the polarity of the voltage for a specific DAC channel depends on the polarity of the reference voltage for that channel. The polarity of the DAC output voltage is the opposite of the DAC reference.

Equation 6, DAC Voltage Conversion

DAC_Output_voltage=±5*x4095

DAC settings for a desired voltage can be calculated using Equation 7 where |x| is the absolute value of the DAC output voltage.

Equation 7, DAC Setting Conversion

DAC_Setting(hex)=DEC2HEX|x|*40955

### Etched ITO Temperature Sensor Formulas

The following equation is used to calculate the etched ITO temperature sensor resistance slope:

Equation 8, ITO Sensor Slope

mx=HRx-LRxT2-T1

Where:

mx = sensor resistance slope

HRx = sensor resistance at upper temperature

T2 = upper temperature

LRx = sensor resistance at lower temperature

T1 = lower temperature

The following equation is used to calculate the etched ITO temperature sensor resistance at 25°C:

Equation 9, Sensor Resistance at 25°C

R25=LRx+mx25-T1

R25 = sensor resistance at 25°C

LRx = sensor resistance at lower temperature

mx = sensor resistance slope

T1 = lower temperature

The following equation is used to calculate the etched ITO temperature sensor calibration offset:

Equation 10, ITO Sensor Offset

VOFFSETx=-R4R5VT-R4VREF1R3-R25+mxT-25R1R2   (a)

Solved for T:

T=25-R25mx+R1R2mxR3+R1R2mxVREFVTR5+VOFFSETxR4      (b)

Where:

VOFFSETx = ITO sensor calibration offset

VT = voltage indicating the measured temperature (set to 0 VDC when calculating offset voltage)

VREF = 5

mx = sensor resistance slope

T = measured temperature in degrees Celsius (set to 25 when calculating offset voltage)

R25 = sensor resistance at 25°C

R1 = 20000

R2 = 20000

R3 = 15000

R4 = 30100

R5 = 221000

Since the temperature lookup table only has 256 entries, only the upper 8 bits of the A/D data is used to index into the table. For A/D data between 0H and 7FH (inclusive), the measurement is positive and should be converted to a voltage value using Equation 11 where x is the decimal equivalent of the converted 8 bits.

Equation 11, Positive A/D Measurements (upper 8 bits)

A2D_input_voltage=5*x128

For A/D data between 80H and FFH (inclusive), the measurement is negative and should be converted to a voltage value using Equation 12 where x is the decimal equivalent of the 2’s complement of the converted 8 bits.

Equation 12, Negative A/D Measurements (upper 8 bits)

A2D_input_voltage=(256-x)*-5128

The following equation is used to calculate the ITO sensor look up table which is an 8 bit two’s complement representation of the temperature in degrees Celsius

Equation 13, ITO Sensor Look Up Table Entry Calculation

If 0 <= T <= 127 C then

Entry = DEC2HEX(T)

If T > 127 C then

Entry = 7Fh

If T < 0 C then

Entry = DEC2HEX(T + 256)

Where:

T is the temperature value calculated with Equation 10 (b)

### Temperature Sensor Formulas

The display head reports the measured temperature in degrees Celsius as an 8 bit 2’s complement number for the control card, backlight card, and ITO temperature sensors.

Equation 14, Temperature Sensor Conversion

If x < 128 then

Temperature (Celsius) = x

If x ≥ 128 then

Temperature (Celsius) = x - 256

Where:

x is the decimal equivalent of the 8 bit value reported

### LCD Heater Formulas

The following equation is used to calculate the heater current in Amps.

Equation 15, ITO Heater Current Conversion

If V < 0,

IHTR = 0 (A)

If V > 0,

IHTR = V / 0.03 / 25.098  (A)

Where:

V = Voltage measured on ITO_CURRENT (ADC channel 46, upper 8 bits only), converted using Equation 11 or Equation 12

The following equation is used to create the Heater Power lookup table in Watts using the calculated heater resistance and the measured heater current.

Equation 16, Heater Power Calculation

Power = IHTR2 * RHTR   (W)

Where:

RHTR is the calculated heater resistance in Ohms

Note: Calculated power greater than 255 W is set to 255d in the table since the entries are 8 bits

## Serial Communications

The display head has two different RS485 serial bus interfaces. A full duplex RS485 interface is used in the field to communicate with an external user (i.e. Rear Electronics Interface Module). A half duplex 485 interface is used in a production test environment. The serial bus interface protocol for the display head is described in the Standard Head Interface Protocol 4.0 (SHIP 4.0) [3]. Any deviations from this protocol are described in the Vesta reference manual [4].

The display head will communicate on the half duplex RS485 interface at 460.8 Kbps data rate, odd parity, 8 data bits, and 1 stop bit. A DHM ID of Ah must be used to establish communication with the display head.

In general, the half duplex is the primary bus used during production test. The full duplex interface is used only to the extent necessary to verify the bus is operational.

## Calibration Data Load

The default calibration file has two output files for use in loading the memory settings for different circumstances. For a unit that hasn’t been calibrated, the full calibration text file should be used. This will load all of the memory settings needed for correct operation. This includes data that will eventually be overwritten during calibration.

For a unit which has been calibrated, the no-cal text file should be used. This file will update most memory settings but will not overwrite the portions of memory which have the unit specific calibration data stored in them.

The calibration output files are formatted such that the alpha characters “ADDRESS” precede a numerical character group which indicates the start address for the data. Following the start address, each numerical character group indicates the successive data for each address beginning with the start address. Each numerical character group increments the contents address until the next alpha character is parsed which indicates the data block is complete and another valid “ADDRESS” (address is not case sensitive) word is required to indicate a new start address.

Figure 2-1 is an example calibration/configuration file format. All numbers are formatted in decimal and addresses are the virtual address for the memory as defined in SHIP 1.0 [3] and the Vesta reference manual [4]. The green highlighted text is the virtual address and the blue highlighted text is the data for the address and successive addresses. The calibration files end with the phrase: A   leave these characters here. Note the three spaces following the letter “A”.

Figure 2-1 Calibration File Example

## Hera2 FPGA Programming

The JTAG interface for programming the Hera2 FPGA is available via the Programming Test Connector J7. The FPGA can be programmed using a FlashPro 3.0, FlashPro 4.0, or other equivalent JTAG programming hardware.

Programming files and software load procedures are called out on the assembly drawing of the control card.

## Microcontroller Programming

The JTAG interface for programming the microcontroller is available via the Programming Test Connector J7. The FPGA can be programmed using a FlashPro 3.0, FlashPro 4.0, or other equivalent JTAG programming hardware.

Programming files and software load procedures are called out on the assembly drawing of the control card.

## Memory Reads/Writes

The non-volatile memory device is accessed as an 8 bit memory space. SHIP does have 16 bit read/write commands (Read 16 and Write 16). When a 16 bit access is indicated, it will be indicated at an even address (ex. 7000h). The least significant byte is read/stored at the even address (ex. 7000h) and the most significant byte is read/stored at the next higher address (ex. 7001h).

ADC and DAC data is stored as 16 bit data but only the upper 12 bits are used. The lower 4 bits are don’t care but are typically loaded with 0’s.

# Connector Interfaces

All externally accessible display head connectors are on the display head control card. There are multiple connectors including those used for operation in a higher level assembly (functional connectors) and those dedicated for test only (test connectors). This section provides a brief description of each connector’s use and some information on the interconnect signals. More detailed information and signal descriptions are available in the hardware design document [1].

## Functional Connectors

### REIM Interface Power Connectors

Connector J1 provides main, backlight, and heater power interface for the control card. REM_+12VDC supplies power to the control card circuitry. +28VDC_BL supplies power to the backlight power supply. +28VDC_HTR supplies power to the LCD heater. In a top level display this connector interfaces to the backplane card via a discrete wire cable. Connector J2 includes 115 VAC which powers the internal cooling fan. This connector also includes a 5 VAC input used as a dimming analog to control the keypanel LED lighting brightness.

### REIM Interface I/O Connector

Connector J3 is the main input/output and low voltage power connector for the control card. The connector includes DVI, A818, and MV3 inputs for video, discrete inputs, RS485 REM serial bus,  and RS485 shop serial bus. In a top level display, this connector interfaces with the backplane card via a discrete wire cable.

### LCD Connector

Connector J4 provides the interface between the control card and LCD when installed in a display head assembly. It includes the LCD power supply voltage, LVDS image bus, control discretes (outputs to LCD), gate feedback signals (inputs from the LCD), an I2C bus interface to the LCD driver chip, and a SPI bus interface to the LCD configuration EEPROM. The control card interfaces with the LCD via a flex circuit card.

### LED Backlight Connectors

The display head uses a direct lit LED backlight. J11 contains the LED power and returns for the LED strings, backlight photodiode interface, and an analog temperature sensor interface. The backlight card and the control card both have a female connector that connects to each other via a double sided two row pin header.

### Bezel Connector

The keypanel interface connector is J9. The 987-1003-106/7 keypanel 24 pushbuttons and three rocker switches with an I2C interface. The keypanel lighting is LED based and is powered by the REIM +5 VDC bus. The 5 VAC input bus is used as a dimming analog to control the keypanel lighting brightness and draws negligible current.

### LCD Heater Connectors

Connectors J5 and J6 provide the LCD heater interface. J5 contains heater power and the high sides of three etched temperature sensors. P3 contains the heater power return and the low sides of three etched temperature sensors. The LCD heater assembly has two flex circuit boards which come through the display head chassis and plug on to the control card.

### Fan Connector

J8 is the fan connector. The discrete wire cable from the cooling fan engages this connector.

## Test Connectors

### Main Test Connector

The main test connector (J10) provides access to power supply voltages. There are also several discrete inputs and outputs that help assess the proper function of the card.

- +3P3VDC_TST

This signal is connected to the +3.3VDC local supply output through a series resistor. During normal operation, the nominal voltage is +3.3 VDC.

- +2P5VDC_TST

This signal is connected to the +2.5VDC local supply output through a series resistor. During normal operation, the nominal voltage is +2.5 VDC.

- +1VDC_TST

This signal is connected to the +1.0VDC local supply output through a series resistor. During normal operation, the nominal voltage is +1.0 VDC.

- +12VDC_TST

This signal is connected to the +12VDC local supply output through a series resistor. During normal operation, the nominal voltage is +12.0 VDC.

- -12VDC_TST

This signal is connected to the -12VDC local supply output through a series resistor. During normal operation, the nominal voltage is -12.0 VDC.

- +25_HLDUP_TST

This signal is connected to the +25VDC_Holdup local supply output through a series resistor. During normal operation, the nominal voltage is +25.0 VDC.

- +1P8VDC_TST

This signal is connected to the +1.8VDC local supply output through a series resistor. During normal operation, the nominal voltage is +1.8 VDC.

- +5VDC_TST

This signal is connected to the +5V_Main local supply output through a series resistor. During normal operation, the nominal voltage is +5.0 VDC.

- LCD_SCL/LCD_SDA

This is the I2C interface to the LCD driver chips.

- +4P5VDC_HLDUP_TST

This signal is connected to the +4P5VDC holdup supply output through a series resistor. During normal operation, the nominal voltage is +4.5 VDC.

- +5V_VAUX_TST

This signal is connected to the +5V_VAUX through a series resistor. During normal operation, the nominal voltage is +5.0 VDC.

- +2.5V_AUX_REF_TST

This signal is connected to the +2.5_AUX_REF through a series resistor. During normal operation, the nominal voltage is +2.5 VDC.

- VREF_2.5_TST

This signal is connected to the VREF_2.5 through a series resistor. During normal operation, the nominal voltage is +2.5 VDC.

- DSBL_GATE_FB_A

This signal when pulled high, simulates a gate feedback failure from the LCD.

- DSBL_GATE_FB_B

This signal when pulled high, simulates a gate feedback failure from the LCD.

- SYS_RST_F

This is the main system reset generated by the control card. This is an open drain signal pulled up to +3.3VDC.

- LCL_PWR_GOOD_TST

This is an active high signal that indicates when the +3.3 VDC, +2.5 VDC, and +1.2 VDC local power supplies are operating within tolerance. This signal is pulled up to +3.3 VDC.

- LED_PWM_TST

This is the TTL level PWM signal used by the control card FPGA to set the duty cycle of the backlight.

- GMON_DSBL_F

This input is used to disable the LCD gate feedback hardware monitor. The LCD gate feedback hardware monitor will latch the backlight off if gate feedback pulses aren’t received at least every 200 ms. Disabling the hardware monitor may be necessary to test some control card functionality.

- LCD_EEP_DI, LCD_ATREN, LCD_EEP_CS, LCD_EEP_EN, LCD_EEP_DO, LCD_EEP_CLK

This is the SPI bus interface to the LCD configuration PROM. This PROM is located on the LCD source driver board. This interface can be used to change the default settings of the LCD.

- +3.3VDC_MON_TST

This signal is used of engineering test only.

- +5VDC_MON_TST

This signal is used of engineering test only.

- +12VDC_MON_TST

This signal is used of engineering test only.

### Programming Test Connector

Test Connector J7 The JTAG interface for programming the Hera2 FPGA and microcontroller. The devices can be programmed using a FlashPro 3.0, FlashPro 4.0, or other equivalent JTAG programming hardware.

# Test Specifications

This section defines the production test requirements for the control card and display head. Tests that are considered optional are labeled with “should”. Most control card level tests can be run at the display head assembly level. Some control card level tests may require disconnecting the LCD when run at the display head level. If control card level requirements are tested at a display head level, the recommendation is to disconnect the LCD and LCD heater from the control card to minimize potential damage to the LCD stack until initial circuit tests are performed but this isn't a requirement.

Suggested test procedures are for reference only and are not considered test requirements. They are provided to clarify the intent of the test requirements and to facilitate development of the actual production test procedures documented in the Acceptance Test Method (ATM) document. The ATM test procedures do not have to match the reference test procedures in this document.

## Control Card Test Specifications

This section contains specifications for the control card level tests.

Unless otherwise noted, the Hera2 FPGA and microcontroller must be programmed and the default calibration data must be loaded to test a given function. Programming data and procedures are called out on the control card assembly drawing.

Unless otherwise noted, test ranges are inclusive of the end points.

### Test Local Power Supply Operation

Purpose

This test verifies the tolerance of the local power supply voltages.

Acceptance Criteria

#### The production test shall verify +3.3VDC is within the range [3.15, 3.42] VDC.

#### The production test shall verify +2.5VDC is within the range [2.4, 2.6] VDC.

#### The production test shall verify +1.0VDC is within the range [0.95, 1.05] VDC.

#### The production test shall verify +12VDC is within the range [11.4, 13.8] VDC.

#### The production test shall verify -12VDC is within the range [-13.1, -10.6] VDC.

#### The production test shall verify +1.8VDC is within the range [1.75, 1.85] VDC.

#### The production test shall verify +5VDC is within the range [4.8, 5.2] VDC.

Product State or Condition

Note: These test steps may be conducted with the Vesta FPGA and configuration/calibration data unprogrammed.

- Power up the control card (except backlight power)

- Measure the voltage on +3P3VDC_TST and verify it’s within the required range

- Measure the voltage on +2P5VDC_TST and verify it’s within the required range

- Measure the voltage on +1VDC_TST and verify it’s within the required range

- Measure the voltage on +12VDC_TST and verify it’s within the required range

- Measure the voltage on -12VDC_TST and verify it’s within the required range

- Measure the voltage on +1P8VDC_TST and verify it’s within the required range

- Measure the voltage on +5VDC_TST and verify it’s within the required range

### Test Reference Voltages

Purpose

This test verifies the tolerance of the reference voltages.

Acceptance Criteria

#### The production test shall verify +5VREF is within the range [4.9, 5.1] VDC.

#### The production test shall verify -5VREF is within the range [-5.1, -4.9] VDC.

#### The production test shall verify +10VREF is within the range [9.8, 10.2] VDC.

Product State or Condition

Note: These test steps may be conducted with the Vesta FPGA and configuration/calibration data unprogrammed.

- Power up the control card (except backlight power)

- Measure the voltage on +5VREF_TST and verify it’s within the required range

- Measure the voltage on -5VREF_TST and verify it’s within the required range

- Measure the voltage on +10VREF_TST and verify it’s within the required range

### Test Holdup Supply

Purpose

This test verifies the tolerance of the holdup supply output voltage and that +12 VDC remains powered for a minimum amount of time after REM low voltage power is removed.

Acceptance Criteria

#### The production test shall verify +4P5VDC_HLDUP is within the range [4.37, 4.64] VDC.

#### The production test shall verify +25VDC_HLDUP is within the range [24.7, 26.75] VDC when the local power supply is powered up.

#### The production test shall verify +3.3VDC is within the range [3.17, 3.43] VDC for at least 110 ms after REM_+12VDC is powered down.

Product State or Condition

Note: These test steps may be conducted with the Hera2 FPGA, microcontroller, and configuration/calibration data unprogrammed.

- Power up the control card (except backlight power)

- Measure the voltage on +4P5VDC_HLDUP _TST and verify it’s within the required range

- Measure the voltage on +25VDC_HLDUP _TST and verify it’s within the required range

- Monitor +3P3VDC_TST and REM_+12VDC with a scope

- Power down the control card

- Trigger on the falling edge of REM_+12VDC at a threshold of 10 VDC

- Measure the time until +3.3VDC_TST drops below 3.17 VDC and verify it meets the requirement

### Test SYS_RST_F

Purpose

This test verifies SYS_RST_F is held low for a minimum time period after +3.3VDC reaches a minimum voltage threshold.

Acceptance Criteria

#### The production test shall verify SYS_RST_F is logic low for at least 1.0 ms after +3.3VDC reaches +3.0 VDC at power up.

Product State or Condition

Note: These test steps may be conducted with the Hera2 FPGA, microcontroller, and configuration/calibration data unprogrammed.

- Monitor SYS_RST_F and +3P3VDC_TST with a scope

- Power up the control card (except backlight power)

- Trigger on the rising edge of +3P3VDC_TST at a threshold of 3.0 VDC

- Measure the time until SYS_RST_F transitions high and verify it meets the requirement

### Program Microcontroller

Purpose

This test programs the microcontroller.

Acceptance Criteria

#### The production test shall program the microcontroller in accordance with the load procedure indicated on the control card assembly drawing.

Product State or Condition

- Program the microcontroller in accordance with the software load procedure called out on the control card assembly drawing

### Program Hera2 FPGA

Purpose

This test programs the Hera2 FPGA.

Acceptance Criteria

#### The production test shall program the Hera2 FPGA in accordance with the load procedure indicated on the control card assembly drawing.

Product State or Condition

- Program the Hera2 FPGA in accordance with the software load procedure called out on the control card assembly drawing

### Load Default Calibration Data

Purpose

This test loads the default calibration data into memory and verifies proper operation of the RS232 shop serial interface.

Acceptance Criteria

#### The production test shall verify the default calibration data can be loaded into memory without error via the RS485 shop serial interface.

Product State or Condition

- Power up the control card (except backlight power)

- Establish communication with the control card via RS485

- Load the default calibration data indicated in the Acceptance Test Method [5] into memory. Refer to section 2.6. Verify the data load completes without error.

### Test Discrete Inputs

Purpose

This test verifies the discrete inputs operate correctly in each logic state.

Acceptance Criteria

#### The production test shall verify HTR_ENABLE correctly detects both OPEN and GND logic states.

#### The production test shall verify REM_DATALOAD_F correctly detects both OPEN and GND logic states.

#### The production test shall verify REM_SHOP_F correctly detects both OPEN and GND logic states.

#### The production test shall verify PSIO_WARN_F correctly detects both OPEN and GND logic low states.

#### The production test shall verify DISCRETE_IN_0..2 correctly detects both OPEN and GND logic low states.

Product State or Condition

- Power up the control card (except backlight power)

- Establish communication with the control card via RS485

- Set HTR_ENABLE to GND

- Read the REG_DISCRETE_INPUTS Register Number 065 and verify bit 0 is logic low

- Set HTR_ENABLE to OPEN

- Read the REG_DISCRETE_INPUTS Register Number 065 and verify bit 0 is logic high

- Set REM_DATALOAD_F to GND

- Read the REG_DISCRETE_INPUTS Register Number 065 and verify bit 2 is logic low

- Set REM_DATALOAD_F to OPEN

- Read the REG_DISCRETE_INPUTS Register Number 065 and verify bit 2 is logic high

- Set REM_SHOP_F to GND

- Read the REG_DISCRETE_INPUTS Register Number 065 verify bit 3 is logic low

- Set PSIO_WARN_F to GND

- Read the REG_DISCRETE_INPUTS Register Number 065 and verify bit 4 is logic low

- Set PSIO_WARN_F to OPEN

- Read the REG_DISCRETE_INPUTS Register Number 065 and verify bit 4 is logic high

- Set REM_SHOP_F to OPEN

- Verify communication is lost with the control card

### Test Backlight Power Supply

Purpose

This test verifies the backlight power supplies operate correctly.

Acceptance Criteria

Note: The tests in this section cannot be run at the display head level with the control card connected to a backlight. If running card level tests at the display head level, the tests in this section can be reserved for more detailed debug if a display head fails a backlight related test.

#### The production test should verify WHT_VOLT_1 is within the range [61, 67] VDC with +28VDC_BL at 29 VDC +/- 1 VDC.

#### The production test should verify WHT_VOLT_2 is within the range [61, 67] VDC with +28VDC_BL at 29 VDC +/- 1 VDC.

#### The production test should verify WHT_VOLT_3 is within the range [61, 67] VDC with +28VDC_BL at 29 VDC +/- 1 VDC.

#### The production test should verify WHT_VOLT_1 is within the range [13.5, 14.5] VDC with +28VDC_BL at 14.5 VDC nominal.

#### The production test should verify WHT_VOLT_2 is within the range [13.5, 14.5] VDC with +28VDC_BL at 14.5 VDC nominal.

#### The production test should verify WHT_VOLT_3 is within the range [13.5, 14.5] VDC with +28VDC_BL at 14.5 VDC nominal.

Product State or Condition

- Power up the control card with +28VDC_BL set to 29 VDC

- Drive the pixel bus with an XGA image

- Establish communication with the control card via RS485

- Set GMON_DSBL_F to logic low to hold the hardware gate feedback monitor in reset

- Write FFh to address 763Ah to set the fault monitor override

- Send Luminance/Gamma Control SHIP command with data 1h to change the luminance to Serial control mode

- Send a SHIP Set Brightness command with data FFH to set luminance to maximum

- Measure the voltage at WHT_VOLT_1 and verify it meets the requirement

- Measure the voltage at WHT_VOLT_2 and verify it meets the requirement

- Measure the voltage at WHT_VOLT_3 and verify it meets the requirement

- Decrease the voltage on +28VDC_BL to 14.5 VDC

- Measure the voltage at WHT_VOLT_1 and verify it meets the requirement

- Measure the voltage at WHT_VOLT_2 and verify it meets the requirement

- Measure the voltage at WHT_VOLT_3 and verify it meets the requirement

### Test Analog to Digital Conversion Circuitry

Purpose

This test verifies the analog to digital converter circuitry operates correctly.

Acceptance Criteria

#### The production test shall verify the analog to digital converter circuitry measures VREF_2.5 within the range [2.4, 2.6] VDC.

Product State or Condition

- Power up the control card (including backlight power)

- Drive the pixel bus with an XGA image

- Establish communication with the control card via RS485

- Read the 2’s complement value from the upper 12 bits of Address 757Ah

- Convert the 2’s complement value to a voltage using Equation 3 or Equation 4 and multiply by 2

- Verify the VREF_2.5 converted value meets the requirement

### Test Digital to Analog Conversion Circuitry

Purpose

This test verifies the digital to analog converter circuitry operates correctly.

Acceptance Criteria

#### The production test shall verify BRT_CMD_COARSE is within the range [-0.6, -0.4] VDC when set to -0.5 VDC.

#### The production test shall verify BRT_CMD_FINE is within the range [-1.2, -0.8] VDC when set to -1.0 VDC.

#### The production test shall verify RED_CHROM_CNTRL is within the range [-3.25, -2.75] VDC when set to -3.0 VDC. (966-7753-002/003 only)

#### The production test shall verify BLUE_CHROM_CNTRL is within the range [-3.75, -3.25] VDC when set to -3.5 VDC. (966-7753-002/003 only)

Product State or Condition

- Power up the control card (including backlight power)

- Drive the pixel bus with an XGA image

- Establish communication with the control card via RS485

- Write FFh to address 7637h to enable manual control of the DAC outputs

- Write 16 bit value 19A0h to address 7590h to set BRT_CMD_COARSE to -0.5 VDC

- Read 2’s complement analog to digital data from the upper 12 bits of Address 7558h

- Convert the 2’s complement value to a voltage using Equation 3 or Equation 4

- Verify the voltage for BRT_CMD_COARSE meets the requirement

- Write 16 bit value 3330h to address 7592h to set BRT_CMD_FINE to -1.0 VDC

- Read 2’s complement analog to digital data from the upper 12 bits of Address 755Ah

- Convert the 2’s complement value to a voltage using Equation 3 or Equation 4

- Verify the voltage for BRT_CMD_FINE meets the requirement

- Write 0h to address 7637h to disable manual control of the DAC outputs

### Test RS485 Input/Output

Purpose

This test verifies the RS485 input port correctly receives data.

Acceptance Criteria

#### The production test shall verify RS485 formatted data is received correctly on RS485_HI/LO at full RS485 voltage levels.

#### The production test shall verify RS485 formatted data is transmitted correctly on RS485_HI/LO at full RS485 voltage levels.

Product State or Condition

- Set DHM_SHOP_F to OPEN

- Power up the control card (including backlight power)

- Drive the pixel bus with an XGA image

- Send a synchronization packet to the display head at a 10 Hz rate or faster

  - Synchronization packet consists of 5 consecutive ARINC 429 labels, all with SDI = 00, SSM = 11, and odd parity with ARINC word bits 29 down to 11 as follows:

    - Label 300 => 000-0000-0000-1111-1110

    - Label 301 => 000-0000-0000-1111-0001

    - Label 302 => 000-0000-0000-1111-0000

    - Label 303 => 000-0000-0000-1111-0000

    - Label 304 => 000-0000-0000-0000-1101

- Transmit a SHIP Loopback command A429 word at a 10 Hz rate or faster to DHM_SERIAL_IN_0_H/L with data bits 18:11 = A1h with SDI = 00, SSM = 11, and odd parity with bits 29 down to 11 as follows:

  - Label 202 => 000-0000-0000-1010-0001

- Verify that the SHIP Loopback response A429 word is transmitted out DHM_SERIAL_OUT_0_H/L with data bits 29 down to 11 matching the SHIP Loopback command A429 word sent to the DHM

  - Label 202 => 000-0000-0000-1010-0001

### Test Control Card Temperature Sensor

Purpose

This test verifies the control card temperature sensor operates correctly.

Acceptance Criteria

#### The production test shall verify the control card temperature sensor indicates a temperature within the range [20, 40] degrees C at factory ambient.

Product State or Condition

- Power up the control card (including backlight power)

- Drive the pixel bus with an XGA image

- Establish communication with the control card via RS232

- Read the 2’s complement value from address 7627h

- Convert the 2’s complement value to an integer to obtain the temperature in degrees Celsius. See Equation 16.

- Verify the computed control card temperature meets the requirement

### Test LCD Power Switch

Purpose

This test verifies the LCD power switch can be enabled and disabled.

Acceptance Criteria

#### The production test shall verify LCD_VCC_FILT is within the range [3.17, 3.43] VDC when the LCD power switch (U107) is enabled.

#### The production test shall verify LCD_VCC_FILT is within the range [-0.45, 0.45] VDC when the LCD power switch (U107) is disabled.

Product State or Condition

- Power up the control card (including backlight power)

- Drive the pixel bus with an XGA image

- Establish communication with the control card via RS232

- Read address 7620h and verify bit 7 is logic ‘1’ (backlight ON)

- Measure LCD_VCC_FILT and verify it’s within the required range

- Set PWR_WARN_F to GND

- Measure LCD_VCC_FILT and verify it’s within the required range

### Test LCD Gate Feedback Hardware Monitor

Purpose

This test verifies the gate feedback hardware monitor can detect inactivity on the gate feedback pulses from the LCD.

Acceptance Criteria

#### The production test shall verify GMON_GOOD is logic low when GATE_FB_A is not toggling.

#### The production test shall verify GMON_GOOD is logic low when GATE_FB_B is not toggling.

Product State or Condition

Note: If this test is run at a display head level, the LCD flex will likely need to be disconnected to get access to the gate feedback signals to test the monitor.

- Set GMON_DSBL_F to logic low

- Power up the control card (including backlight power in order to enable LCD_VCC and power the monitor circuitry)

- Drive the pixel bus with an XGA image

- Establish communication with the control card via RS232

- Verify GMON_GOOD is logic high (Read address 79D0h bit 2)

- Toggle GATE_FB_A and GATE_FB_B with a 50% duty cycle 60 Hz signal with the logic high state +10.0 +/- 0.5 VDC and the logic low state -10 +/- 0.5 VDC

- Set GMON_DSBL_F to logic high

- Verify GMON_GOOD is logic high (Read address 79D0h bit 2)

- Hold GATE_FB_A to logic low

- Verify GMON_GOOD is logic low (Read address 79D0h bit 2)

- Set GMON_DSBL_F to logic low

- Toggle GATE_FB_A and GATE_FB_B with a 50% duty cycle 60 Hz signal with the logic high state +10.0 +/- 0.5 VDC and the logic low state -10 +/- 0.5 VDC

- Set GMON_DSBL_F to logic high

- Hold GATE_FB_B to logic low

- Verify GMON_GOOD is logic low (Read address 79D0h bit 2)

## Display Head Test Specifications

### Test ITO Temperature Sensors

Purpose

This test calibrates the ITO temperature sensors and verifies proper operation.

Acceptance Criteria

#### The production test shall calculate the offset voltage and lookup table values for the Top ITO Temperature Sensor based on the characterization data on the heater assembly flex and store the data in non-volatile memory.

#### The production test shall calculate the offset voltage and lookup table values for the Center ITO Temperature Sensor based on the characterization data on the heater assembly flex and store the data in non-volatile memory.

#### The production test shall calculate the offset voltage and lookup table values for the Bottom ITO Temperature Sensor based on the characterization data on the heater assembly flex and store the data in non-volatile memory.

#### With the LCD heater disabled, the production test shall verify the Center ITO Temperature Sensor indicates 25 +/- 10 degrees Celsius.

#### With the LCD heater disabled, the production test shall verify the Top ITO Temperature Sensor indicates a temperature reading within +/- 5 degrees Celsius of the Center ITO Temperature Sensor reading.

#### With the LCD heater disabled, the production test shall verify the Bottom ITO Temperature Sensor indicates a temperature reading within +/- 5 degrees Celsius of the Center ITO Temperature Sensor reading.

Product State or Condition

- Power up the control card

- Drive the pixel bus with an XGA image

- Establish communication with the control card via RS232

- Write FFh to address 763Ah to set the fault monitor override

- Using the 10-60 characterization data for the ITO sensors shown on the LCD heater flex, use Equation 12 (a) to calculate the offset voltages for the Top, Center, and Bottom ITO sensors.

- Use Equation 9 to calculate DAC settings for the calculated offset voltages

- Write the Top ITO offset setting to the 12 most significant bits of address 3A14h

- Write the Center ITO offset setting to the 12 most significant bits of address 3A16h

- Write the Bottom ITO offset setting to to the 12 most significant bits of address 3A18h

- Compute the 256 entry temperature lookup table for the Top ITO sensor with index 0 to 255

  - The upper 8 bits of ADC measured voltage (ADC channel 37) (two’s complement) is used as the index to the lookup table (0 to 255)

  - Use Equation 13 or Equation 14 to calculate the voltage VT for each entry 0-255

  - Use Equation 12 (b) solved for T to calculate the temperature value for each entry. Use the offset voltage calculated earlier for VOFFSETx.

  - Use Equation 15 to calculate the 8 bit two’s complement representation of the temperature value for each entry to be stored in memory

- Store the lookup table for the Top ITO sensor at address 0800h to 08FFh

- Compute the 256 entry temperature lookup table for the Center ITO sensor with index 0 to 255

  - The upper 8 bits of ADC measured voltage (ADC channel 38) (two’s complement) is used as the index to the lookup table (0 to 255)

  - Use Equation 13 or Equation 14 to calculate the voltage VT for each entry 0-255

  - Use Equation 12 (b) solved for T to calculate the temperature value for each entry. Use the offset voltage calculated earlier for VOFFSETx.

  - Use Equation 15 to calculate the 8 bit two’s complement representation of the temperature value for each entry to be stored in memory

- Store the lookup table for the Center ITO sensor at address 0900h to 09FFh

- Compute the 256 entry temperature lookup table for the Bottom ITO sensor with index 0 to 255

  - The upper 8 bits of ADC measured voltage (ADC channel 39) (two’s complement) is used as the index to the lookup table (0 to 255)

  - Use Equation 13 or Equation 14 to calculate the voltage VT for each entry 0-255

  - Use Equation 12 (b) solved for T to calculate the temperature value for each entry. Use the offset voltage calculated earlier for VOFFSETx.

  - Use Equation 15 to calculate the 8 bit two’s complement representation of the temperature value for each entry to be stored in memory

- Store the lookup table for the Bottom ITO sensor at address 0A00h to 0AFFh

- Read the Top ITO temperature from address 7623h

- Convert the 2’s complement data into a temperature using Equation 16

- Verify the temperature reading meets the requirement

- Read the Center ITO temperature from address 7624h

- Convert the 2’s complement data into a temperature using Equation 16

- Verify the temperature reading meets the requirement

- Read the Bottom ITO temperature from address 7625h

- Convert the 2’s complement data into a temperature using Equation 16

- Verify the temperature reading meets the requirement

### Test LCD Heater

Purpose

This test verifies the LCD heater, voltage monitor, and current monitor circuits operate correctly. This test creates the heater power table and stores the table in non-volatile memory.

Acceptance Criteria

#### The production test shall verify ITO_CURRENT measures within the range [-0.200, 0.200] ADC when the LCD heater is disabled.

#### The production test shall verify ITO_CURRENT measures within the range [2.0, 4.0] ADC when the LCD heater is enabled and +28VDC_HTR is 28.0 +/- 1.0 VDC.

#### The production test shall calculate the ITO heater resistance based on the +28VDC_HTR voltage and the ITO_CURRENT measurement, generate the heater power lookup table, and store the table in non-volatile memory.

#### The production test shall verify HEATER_PWR_GOOD is logic high when +28VDC_HTR is 28.0 +/- 1.0 VDC.

#### The production test shall verify the Center ITO Temperature Sensor temperature reading increases at least 5 degrees when the LCD heater is enabled for at least 30 seconds.

#### The production test shall verify the reported heater power is within +/-15% of the power being delivered by the test equipment to the +28VDC_HTR power input of the display head.

#### The production test shall verify the LCD heater shuts off when enabled and +28VDC_HTR is increased above 33 VDC.

#### The production test shall verify HEATER_PWR_GOOD is logic low when +28VDC_HTR is increased above 33 VDC.

#### The production test shall verify the LCD turns on when enabled and +28VDC_HTR is decreased from out of range high to 28.0 +/- 1.0 VDC.

#### The production test shall verify the LCD shuts off when enabled and +28VDC_HTR is decreased below 20 VDC.

#### The production test shall verify HEATER_PWR_GOOD is logic low when +28VDC_HTR is decreased below 20 VDC.

#### The production test shall verify the LCD heater turns on when enabled and +28VDC_HTR is increased from out of range low to 28.0 +/- 1.0 VDC.

#### The production test shall verify the LCD heater shuts off when enabled and HEATERS_ENABLED is set to GND.

#### The production test shall verify the LCD heater shuts off when enabled and PWR_WARN_F is set to GND.

#### The production test shall verify the LCD heater shuts off when enabled and ITO_CTR_TEMP indicates greater than 85 degrees Celsius.

#### The production test shall verify the LCD heater shuts off when enabled and ITO_TOP_TEMP indicates greater than 85 degrees Celsius.

#### The production test shall verify the LCD heater shuts off when enabled and ITO_BOT_TEMP indicates greater than 85 degrees Celsius.

Product State or Condition

- Set HEATERS_ENABLED to GND

- Power up the control card (including backlight power)

- Drive the pixel bus with an XGA image

- Establish communication with the control card via RS232

- Write FFh to address 763Ah to set the fault monitor override

- Apply 28.0 VDC to +28VDC_HTR

- Read 2’s complement analog to digital data from the upper 12 bits of address 755Ch

- Convert the 2’s complement value to voltage using Equation 5 or Equation 6

- Convert the voltage to current using Equation 17

- Verify the measured current meets the requirement when the heater is disabled

- Set HEATERS_ENABLED to OPEN

- Enable the heater by sending a SHIP Thermal Control command with data 14h to force the ITO heater on

- Read 2’s complement analog to digital data from the upper 12 bits of address 755Ch

- Convert the 2’s complement value to voltage using Equation 3 or Equation 4

- Convert the voltage to current using Equation 15

- Verify the measured current meets the requirement when the heater is enabled

- Measure the voltage applied to +28VDC_HTR while the heater is enabled

- Disable the heater by sending a SHIP Thermal Control command with data 4h to disable the ITO heater

- Use Ohm’s law to calculate the ITO heater resistance based on the measured current when the heater is enabled and the voltage applied to +28VDC_HTR

- Compute the 256 entry Heater Power lookup table with index 0 to 255

  - The index to the lookup table is the upper 8 bits of the 2’s complement ADC measurement of ITO_CURRENT (ADC channel 42)

  - Compute the ITO heater current for each entry using Equation 13 or Equation 14 and Equation 17, followed by the heater power for each entry using Equation 18.

- Store the heater power lookup table at address 3800h to 38FFh

- Read address 7639h and verify bit 5 is logic ‘1’ (HEATER_PWR_GOOD)

- Read the Center ITO temperature from address 7624h

- Convert the 2’s complement data into a temperature reading using Equation 16

- Enable the heater by sending a SHIP Thermal Control command with data 14h to force the ITO heater on

- Wait at least 30 seconds

- Read the Center ITO temperature from address 7624h

- Convert the 2’s complement data into a temperature reading using Equation 16

- Verify the temperature has increased in accordance with the requirement

- Calculate the heater power delivered to the display head by test equipment based on the +28VDC_HTR voltage and current of the test equipment power supply

- Read the display head calculated heater power from address 762Fh and verify it meets the requirement

- Increase +28VDC_HTR to 33.0 VDC

- Read address 763Bh and verify bit 3 is logic ‘0’ (heater disabled)

- Read address 7639h and verify bit 5 is logic ‘0’ (HEATER_PWR_GOOD)

- Reduce +28VDC_HTR to 28.0 VDC

- Read address 763Bh and verify bit 3 is logic ‘1’ (heater enabled)

- Read address 7639h and verify bit 5 is logic ‘1’ (HEATER_PWR_GOOD)

- Reduce +28VDC_HTR to 20.0 VDC

- Read address 763Bh and verify bit 3 is logic ‘0’ (heater disabled)

- Read address 7639h and verify bit 5 is logic ‘0’ (HEATER_PWR_GOOD)

- Increase +28VDC_HTR to 28.0 VDC

- Read address 763Bh and verify bit 3 is logic ‘1’ (heater enabled)

- Set HEATERS_ENABLED to GND

- Read address 763Bh and verify bit 3 is logic ‘0’ (heater disabled)

- Set HEATERS_ENABLED to OPEN

- Set PWR_WARN_F to GND

- Read address 763Bh and verify bit 3 is logic ‘0’ (heater disabled)

- Set PWR_WARN_F to OPEN

- Set HEATERS_ENABLED to GND

- Wait 30 seconds to allow heater and LCD stack to cool off

Test Top ITO Sensor monitor

- Read the 16 bit value at address 3A14h and save for later

- Write 16 bit value D000h to address 3A14h to force ITO Top sensor to read higher

  - Adjust the value at address 3A14h to get a temperature reading of approximately 85 C. Adding or subtracting 20h to the value adjusts the value by approximately +/-1 degree C. The temperature reading is available at address 7623h

- Set HEATERS_ENABLED to OPEN

- Read address 763Bh and verify bit 3 is logic ‘0’ (heater disabled)

- Restore the original 16 bit value to address 3A14h

- Set HEATERS_ENABLED to GND

Test Center ITO Sensor monitor

- Cycle power to the display head (since the high side switch was disabled when forcing the heater on, an ITO Heater fault was set which can only be cleared by a power cycle)

- Establish communication with the control card via RS232

- Write FFh to address 763Ah to set the fault monitor override

- Enable the heater by sending a SHIP Thermal Control command with data 14h to force the ITO heater on

- Read the 16 bit value at address 3A16h and save for later

- Write 16 bit value D000h to address 3A16h to force ITO Center sensor to read higher

  - Adjust the value at address 3A16h to get a temperature reading of approximately 85 C. Adding or subtracting 20h to the value adjusts the value by approximately +/-1 degree C. The temperature reading is available at address 7624h

- Set HEATERS_ENABLED to OPEN

- Read address 763Bh and verify bit 3 is logic ‘0’ (heater disabled)

- Restore the original 16 bit value to address 3A16h

- Set HEATERS_ENABLED to GND

Test Bottom ITO Sensor monitor

- Cycle power to the display head (since the high side switch was disabled when forcing the heater on, an ITO Heater fault was set which can only be cleared by a power cycle)

- Establish communication with the control card via RS232

- Write FFh to address 763Ah to set the fault monitor override

- Enable the heater by sending a SHIP Thermal Control command with data 14h to force the ITO heater on

- Read the 16 bit value at address 3A18h and save for later

- Write 16 bit value D000h to address 3A18h to force ITO Center sensor to read higher

  - Adjust the value at address 3A18h to get a temperature reading of approximately 85 C. Adding or subtracting 20h to the value adjusts the value by approximately +/-1 degree C. The temperature reading is available at address 7625h

- Set HEATERS_ENABLED to OPEN

- Read address 763Bh and verify bit 3 is logic ‘0’ (heater disabled)

- Restore the original 16 bit value to address 3A18h

- Set HEATERS_ENABLED to GND

- Cycle power to the display head (since the high side switch was disabled when forcing the heater on, an ITO Heater fault was set which can only be cleared by a power cycle)

### Test Pixel Bus and LCD

Purpose

This test verifies input pixel data is correctly received and displayed on the LCD and that no related faults are detected.

Acceptance Criteria

#### The production test shall verify that XGA video data received on the input pixel bus is correctly displayed on the LCD.

#### The production test shall verify the LCD does not indicate self-protect mode when powered up and displaying an image.

#### The production test shall verify the LCD does not indicate a fail condition when powered up and displaying an image.

#### The production test shall verify that the hardware gate feedback monitor doesn’t indicate a fault when powered up and displaying an image.

Product State or Condition

- Power up the control card

- Drive the pixel bus with an XGA image

- Establish communication with the control card via RS232

- Write FFh to address 763Ah to set the fault monitor override

- Drive the pixel bus with an XGA vertical color bar pattern

- Verify the pattern is correctly displayed on the LCD

- Read address 79D0h bit 0 and verify it is logic ‘1’ (LCD_FAIL_F = Not Fail)

- Read address 79D0h bit 1 and verify it is logic ‘0’ (SELF_PROTECT = Not Self Protect)

- Read address 79D0h bit 2 and verify it is logic ‘1’ (GMON_GOOD = Good)

### Test DVI Input (987-8392-050 only)

Purpose

This test verifies DVI image data is correctly received and displayed on the LCD and that no related faults are detected.

Acceptance Criteria

#### The production test shall verify that UXGA video data received on the DVI input bus is correctly displayed on the LCD.

#### The production test shall verify the LCD does not indicate self-protect mode when powered up and displaying an image.

#### The production test shall verify the LCD does not indicate a fail condition when powered up and displaying an image.

#### The production test shall verify that no active UXGA Clock, UXGA DE, UXGA Hsync, or UXGA Vsync faults are present when the display head receives valid pixel bus data.

Product State or Condition

- Drive a UXGA resolution image to the MFD input of the tester card

- Set tester card switch S6-3 to GND to drive the DVI input to the display head

- Power up the control card

- Establish communication with the control card via RS232

- Write FFh to address 763Ah to set the fault monitor override

- Write 6h to address 79A2h to select the DVI input for display

- Drive the DVI input with a UXGA vertical color bar pattern

- Verify the pattern is correctly displayed on the LCD

- Read address 79C6h

  - Verify Bit 0 = 0 (UXGA Clock Fault status)

  - Verify Bit 1 = 0 (UXGA DE Fault status)

  - Verify Bit 2 = 0 (UXGA Hsync Fault status)

  - Verify Bit 3 = 0 (UXGA Vsync Fault status)

- Drive the DVI input with an UXGA horizontal color bar pattern

- Verify the pattern is correctly displayed on the LCD

- Read address 79C6h

  - Verify Bit 0 = 0 (UXGA Clock Fault status)

  - Verify Bit 1 = 0 (UXGA DE Fault status)

  - Verify Bit 2 = 0 (UXGA Hsync Fault status)

  - Verify Bit 3 = 0 (UXGA Vsync Fault status)

### Test Keypanel

Purpose

This test verifies the keypanel pushbuttons and rocker switches operate correctly. This test also verifies the keypanel lighting operates correctly.

Acceptance Criteria

#### The production test shall verify NVIS_ENB_F correctly detects both OPEN and GND logic states. (987-8392-050 only)

#### The production test shall verify no button press is indicated when no keypanel buttons or rocker switches are being pressed.

#### The production test shall verify that the correct button press is detected when each of the 32 keypanel buttons is individually pressed. (987-8392-050 only)

#### The production test shall verify that the correct button press is detected when each of the 24 keypanel buttons is individually pressed. (987-8392-054 only)

#### The production test shall verify that the correct button press is detected when each half of the Video Brightness and Video Contrast rocker switches is individually pressed.

#### The production test shall verify the backlight brightness can be adjusted up and down using the Backlight Brightness rocker switch.

#### The production test shall verify the Power/Backlight Mode knob operates correctly in all three positions. (987-8392-050 only)

#### The production test shall verify the Power On/Off toggle switch operates correctly in both positions. (987-8392-054 only)

#### The production test shall verify all keypanel bezel buttons and the rocker switch are illuminated when 5 +/- 0.1 VAC 400 Hz is applied between 5VAC_H-A and 5VAC_L-A.

#### The production test shall verify the keypanel illumination varies with the voltage applied between 5VAC_H-A and 5VAC_L-A

Product State or Condition

987-8392-050 only

- Power up the control card

- Drive the pixel bus with an XGA image

- Establish communication with the control card via RS232

- Write FFh to address 763Ah to set the fault monitor override

- Set the Power/Backlight Mode knob to the NORM position

- Set NVIS_ENB_F to GND

- Read Address 7639h and verify bit 0 is logic low

- Set NVIS_ENB_F to OPEN

- Read Address 7639h and verify bit 0 is logic high

- With no keypanel buttons being pressed, use the SHIP Read Bezel Buttons command to verify there are no indicated button presses

- For each of the 32 keypanel buttons, press the button and use the SHIP Read Bezel Buttons command to verify that only the appropriate button press is indicated and all of the other button/rocker switches indicate no press

- Press the upper and lower halves of the Video Brightness and Video Contrast rocker switches

  - Use the SHIP Read Bezel Discrete command to verify the appropriate key press is indicated and the other keypanel buttons indicate no press

- Set the Power/Backlight Mode knob to the NORM position

- Verify PWR_CNTRL_OUT is in the OPEN state

- Press and hold the lower half of the Backlight Brightness rocker and verify the backlight brightness decreases steadily

- Press and hold the upper half of the Backlight Brightness rocker and verify the backlight brightness increases steadily

- Set the Power/Backlight Mode knob to the NVG position

- Verify the backlight dims

- Verify PWR_CNTRL_OUT is in the OPEN state

- Set the Power/Backlight Mode knob to the OFF position

- Verify PWR_CNTRL_OUT is in the GND state

- Apply 5 VAC between 5VAC_H-A and 5VAC_L-A

- Verify all keypanel push buttons and rocker switches are illuminated

- Adjust the keypanel lighting voltage between 0 and 5 VAC and verify the keypanel illumination follows the input voltage level

987-8392-054 only

- Power up the control card

- Drive the pixel bus with an XGA image

- Establish communication with the control card via RS232

- Write FFh to address 763Ah to set the fault monitor override

- Set NVIS_ENB_F to GND

- Read Address 7639h and verify bit 0 is logic low

- Set NVIS_ENB_F to OPEN

- Read Address 7639h and verify bit 0 is logic high

- With no keypanel buttons being pressed, use the SHIP Read Bezel Buttons command to verify there are no indicated button presses

- For each of the 24 keypanel buttons, press the button and use the SHIP Read Bezel Buttons command to verify that only the appropriate button press is indicated and all of the other button/rocker switches indicate no press

- Press the upper and lower halves of the Video Brightness and Video Contrast rocker switches

  - Use the SHIP Read Bezel Discrete command to verify the appropriate key press is indicated and the other keypanel buttons indicate no press

- Press and hold the lower half of the Backlight Brightness rocker and verify the backlight brightness decreases steadily

- Press and hold the upper half of the Backlight Brightness rocker and verify the backlight brightness increases steadily

- Set the Power On/Off toggle switch to the On position

- Verify PWR_CNTRL_OUT is in the OPEN state

- Set the Power On/Off toggle switch to the Off position

- Verify PWR_CNTRL_OUT is in the GND state

- Apply 5 VAC between 5VAC_H-A and 5VAC_L-A

- Verify all keypanel push buttons and rocker switches are illuminated

- Adjust the keypanel lighting voltage between 0 and 5 VAC and verify the keypanel illumination follows the input voltage level

### Test Backlight Temperature Sensor

Purpose

This test verifies the backlight temperature sensor operates correctly.

Acceptance Criteria

#### With the backlight off, the production test shall verify the backlight temperature sensor indicates a temperature within the range [20, 40] degrees C at factory ambient.

Product State or Condition

- Power up the control card

- Drive the pixel bus with an XGA image

- Establish communication with the control card via RS232

- Read address 7626h

- Convert the 2’s complement data into a signed integer representing temperature in degrees Celsius (see Equation 16)

- Verify the computed backlight temperature meets the requirement

### Test Fan

Purpose

This test verifies the display head fan operates properly.

Acceptance Criteria

#### The production test shall verify FAN_PWR is logic ‘0’ (fan voltage not detected) when 115VAC is not applied to the control card.

#### The production test shall verify FAN_CUR_F is logic ‘1’ (fan current not detected) when 115VAC is not applied to the control card.

#### The production test shall verify FAN_PWR is logic ‘1’ when 115VAC is applied between 115VAC_H and 115VAC_L and the fan is commanded off.

#### The production test shall verify FAN_CUR_F is logic ‘1’ (fan current not detected) when 115VAC is applied between 115VAC_H and 115VAC_L and the fan is commanded off.

#### The production test shall verify the fan runs when commanded on.

#### The production test shall verify FAN_PWR is logic ‘1’ (fan power detected) when the fan is running.

#### The production test shall verify the direction of airflow is into the fan when the fan is on.

#### The production test shall verify FAN_CUR_F is logic ‘0’ (fan current detected) when the fan is commanded on and the fan is running.

#### With 115VAC applied to the display head, the production test shall verify the fan turns off when commanded off.

Product State or Condition

- Power up the control card

- Drive the pixel bus with an XGA image

- Establish communication with the control card via RS232

- Write FFh to address 763Ah to set the fault monitor override

- Ensure 115VAC is off

- Send a SHIP Thermal Control command for manual fan control and disable the fan

- Read bit 6 at address 7639h (FAN_PWR) and verify it is logic ‘0’

- Read bit 7 at address 7639h (FAN_CUR_F) and verify it is logic ‘1’

- Apply 115VAC to the display head

- Read bit 6 at address 7639h (FAN_PWR) and verify it is logic ‘1’

- Read bit 7 at address 7639h (FAN_CUR_F) and verify it is logic ‘1’

- Send a SHIP Thermal Control command to enable the fan

- Verify the fan is running and that airflow direction is into the fan

- Read bit 6 at address 7639h (FAN_PWR) and verify it is logic ‘1’

- Read bit 7 at address 7639h (FAN_CUR_F) and verify it is logic ‘0’

- Send a SHIP Thermal Control command with data 1h to manually disable the fan

- Read address 763Bh and verify Bit 1 = 0 (fan disabled)

### Test Chromaticity

Purpose

This test calibrates and checks the display chromaticity.

Acceptance Criteria

#### The production test shall adjust the red drive to achieve the color targets in Table 4-1 within a radius of 0.035. Refer to Equation 1 (a). (987-8392-050 only)

#### The production test shall adjust the blue drive to achieve the color targets in Table 4-1 within a radius of 0.035. Refer to Equation 1 (a). (987-8392-050 only)

Table 4-1, 987-8392-050 Chromaticity Targets

| Color | u’ | v’ |
| --- | --- | --- |
| White | 0.196 | 0.494 |
| Red | 0.420 | 0.520 |
| Green | 0.122 | 0.549 |
| Blue | 0.180 | 0.170 |

#### The production test shall verify the chromaticity of white, red, green, and blue are within a Delta C*(≤26.0) with respect to the color targets shown in Table 4-2. Refer to Equation 1 (b). (987-8392-054 only)

Table 4-2, 987-8392-054 Chromaticity Targets

| Color | u’ | v’ |
| --- | --- | --- |
| white | 0.187 | 0.478 |
| Red | 0.386 | 0.525 |
| Green | 0.123 | 0.566 |
| Blue | 0.172 | 0.197 |

#### The production test shall write the measured white chromaticity to the following addresses as indicated (e.g. 0.X1X2X3X4 ).

white u’

6613H	X1

6614H	X2

6615H	X3

6616H	X4

white v’

6617H	X1

6618H	X2

6619H	X3

661AH	X4

#### The production test shall write the measured red chromaticity to the following addresses as indicated (e.g. 0.X1X2X3X4 ).

red u’

661BH	X1

661CH	X2

661DH	X3

661EH	X4

red v’

661FH	X1

6620H	X2

6621H	X3

6622H	X4

#### The production test shall write the measured green chromaticity to the following addresses as indicated (e.g. 0.X1X2X3X4 ).

green u’

6623H	X1

6624H	X2

6625H	X3

6626H	X4

green v’

6627H	X1

6628H	X2

6629H	X3

662AH	X4

#### The production test shall write the measured blue chromaticity to the following addresses as indicated (e.g. 0.X1X2X3X4 ).

blue u’

662BH	X1

662CH	X2

662DH	X3

662EH	X4

blue v’

662FH	X1

6630H	X2

6631H	X3

6632H	X4

Product State or Condition

987-8392-050 DHM only

- Power up the control card

- Drive the pixel bus with an XGA image

- Establish communication with the control card via RS232

- Write FFh to address 763Ah to set the fault monitor override

- Send a SHIP Display Mode command with data C0h to enable the backlight and display a solid grayscale pattern

- Write FFh to address 7628h, 7629h, and 762Ah to command a white screen

- Adjust the backlight brightness to approximately 100 fL

- Write FFh to address 7628h and 00h to addresses 7629h and 762Ah to command a red screen

- Measure the u’ and v’ color coordinates of the display

  - If u’ is less than .3930, increase RED_CHROM_CNTRL value (the upper 12 bits of the 16 bit value) at address 3A0Ah until u’ is greater than .3930

  - If u’ is not greater than 0.3970 then proceed to the next step. If u’ is greater than .3970, decrease RED_CHROM_CNTRL value (the upper 12 bits of the 16 bit value) at address 3A0Ah until u’ is less than .3970

  - Verify u’ is still greater than .3930

  - Write the 16 bit value at address 3A0Ah to address 3A08h (NVIS chrom setting)

- Write FFh to address 762Ah and 00h to addresses 7628h and 7629h to command a blue screen

- Measure the u’ and v’ color coordinates of the display

  - If v’ is less than .1800, decrease BLUE_CHROM_CNTRL value (the upper 12 bits of the 16 bit value) at address 3A0Eh until v’ is greater than .1800

  - If v’ is not greater than 0.1840 then proceed to the next step. If v’ is greater than .1840, increase BLUE_CHROM_CNTRL value (the upper 12 bits of the 16 bit value) at address 3A0Eh until v’ is less than .1840

  - Verify u’ is greater than .1800

  - Write the 16 bit value at address 3A0Eh to address 3A0Ch (NVIS chrom setting)

- Write FFh to address 7628h, 7629h, and 762Ah to command a white screen

- Measure and record the u’ and v’ color coordinates of the display for a white screen

- Decrease the blue setting at address 762A until u’ and v’ are at a radius of ≤ 0.030.

- Write the blue setting to address 6F12h

- Verify u’ and v’ are within the target radius for white

- Store the u’ and v’ values for white in memory per the requirement

- Write FFh to address 7629h and 00h to addresses 7628h and 762Ah to command a green screen

- Verify u’ and v’ are within the target radius for green

- Store the u’ and v’ values for green in memory per the requirement

- Write FFh to address 762Ah and 00h to addresses 7628h and 7629h to command a blue screen

- Verify u’ and v’ are within the target radius for blue

- Store the u’ and v’ values for blue in memory per the requirement

- Write FFh to address 7628h and 00h to addresses 7629h and 762Ah to command a red screen

- Verify u’ and v’ are within the target radius for red

- Store the u’ and v’ values for red in memory per the requirement

Note: Spectrometer is measuring the center of the display at 0 degrees vertical.

987-8392-054 only

- Power up the control card

- Drive the pixel bus with an XGA image

- Establish communication with the control card via RS232

- Write FFh to address 763Ah to set the fault monitor override

- Send a SHIP Display Mode command with data C0h to enable the backlight and display a solid grayscale pattern

- Write FFh to address 7628h, 7629h, and 762Ah to command a white screen

- Adjust the backlight brightness to approximately 100 fL

- Measure the u’ and v’ color coordinates of the display

- Verify u' and v' are within the target Delta C* for white

- Write FFh to address 7628h and 00h to addresses 7629h and 762Ah to command a red screen

- Measure the u’ and v’ color coordinates and the luminance of the display

- Verify u' and v' are within the target Delta C* for red

- Write FFh to address 762Ah and 00h to addresses 7628h and 7629h to command a blue screen

- Measure the u’ and v’ color coordinates and the luminance of the display

- Verify u' and v' are within the target Delta C* for blue

- Write FFh to address 7629h and 00h to addresses 7628h and 762Ah to command a green screen

- Measure the u’ and v’ color coordinates and the luminance of the display

- Verify u' and v' are within the target Delta C* for green

- Store the u’ and v’ values for white, red, blue, green in memory per the requirement

Note: Spectrometer is measuring the center of the display at 0 degrees vertical.

### Test Backlight Luminance

Purpose

This test calibrates and checks the display backlight luminance.

Acceptance Criteria

#### The production test shall calibrate the maximum Day mode luminance within the range [324, 334] fL  for the 987-8392-050 and [235, 245] fL for the 987-8392-054.

#### The production test shall calibrate the minimum Day mode luminance within the range [0.025, 0.045] fL.

#### The production test shall calibrate the maximum NVIS mode luminance within the range [2.50, 2.65] fL.

#### The production test shall calibrate the minimum NVIS mode luminance within the range [0.012, 0.014] fL for the 987-8392-050 and [0.02, 0.045] fL for the 987-8392-054.

#### The production test shall write the calibration date to the following addresses in ASCII format (e.g. 30h is the ASCII character 0) as follows:

6F0BH	M

6F0CH	M

6F0DH	D

6F0EH	D

6F0FH	Y

6F10H	Y

#### The production test shall write the Day maximum luminance in fL to the following addresses in ASCII format (e.g. 30h is the ASCII character 0; e.g. X1X2X3.X4 X5 fL) as follows:

6600H	X1

6601H	X2

6602H	X3

6603H	X4

6604H	X5

#### The production test shall write the Day minimum luminance in fL to the following addresses in ASCII format (e.g. 30h is the ASCII character 0; e.g. X1X2X3.X4 X5 fL) as follows:

6605H	X1

6606H	X2

6607H	X3

6608H	X4

6609H	X5

#### The production test shall write the NVIS maximum luminance in fL to the following addresses in ASCII format (e.g. 30h is the ASCII character 0; e.g. X1X2X3.X4 fL) as follows:

660AH	X1

660BH	X2

660CH	X3

660DH	X4

#### The production test shall write the NVIS minimum luminance in fL to the following addresses in ASCII format (e.g. 30h is the ASCII character 0; e.g. X1X2X3.X4 X5 fL) as follows:

660EH	X1

660FH	X2

6610H	X3

6611H	X4

6612H	X5

Product State or Condition

- Power up the control card

- Drive the pixel bus with an XGA image

- Establish communication with the control card via RS232

- Write FFh to address 763Ah to set the fault monitor override

- Set the Power/Backlight Mode knob to the NORM position

- Set NVIS_ENB_F to GND

- Send a SHIP Display Mode command with data C0h to enable the backlight and display a solid grayscale pattern

- Write FFh to address 7628h, 7629h, and 762Ah to command a white screen

NVIS Maximum Luminance

- Send Luminance/Gamma Control SHIP command with data 1h to change the luminance to Serial control mode

- Send a SHIP Set Brightness command with data FFH to set luminance to maximum

- Adjust the upper 12 bits of the 16 bit NVIS SYS_LUM_CAL value at address 3B0Ah to reach the target maximum luminance

  - Note: Reducing NVIS SYS_LUM_CAL increases luminance

- Write the measured NVIS maximum luminance to memory in accordance with the requirements

NVIS Minimum Luminance

- Send a SHIP Set Brightness command with data 00h to set luminance to minimum

- Toggle PWR_WARN_F low then back high to clear the error amplifier

  - Make sure the ERROR_AMP_OUTPUT voltage is less than -50 mVDC. ERROR_AMP_OUT is the 2’s complement value stored at 756Eh (ADC channel 55). Use Equation 5 or Equation 6 to convert to a voltage and multiply by 2.5 scale factor

  - If necessary, adjust the upper 12 bits of the 16 bit NVIS ERROR_AMP_OFFSET at address 3B0Eh until ERROR_AMP_OUT is ([-100, -50] mVDC.

- Note: It is important when adjusting the MIN_PWM in the next step that the pulse width circuit be up against the minimum limiter (MIN_PWM) that is being adjusted to set the minimum luminance. This is assured by making sure that the ERROR_AMP_OUTPUT voltage stays negative until after the adjustment process is complete.Adjust the upper 12 bits of the 16 bit MIN_PWM value at address 3B10h to achieve the target minimum luminance

- Adjust the upper 12 bits of the 16 bit NVIS ERROR_AMP_OFFSET at address 3B0Eh until ERROR_AMP_OUT is within the range [-30, -10] mV.

  - ERROR_AMP_OUT is the 2’s complement value stored at 756Eh (ADC channel 55). Use Equation 5 or Equation 6 to convert to a voltage and multiply by 2.5 scale factor

- Write the measured NVIS minimum luminance to memory in accordance with the requirements

Day Maximum Luminance

- Set NVIS_ENB_F to OPEN

- Send a SHIP Set Brightness command with data FFh to set luminance to maximum

- Adjust the upper 12 bits of the 16 bit Day SYS_LUM_CAL value at address 3B08h to reach the target maximum luminance

  - Note: Reducing Day SYS_LUM_CAL increases luminance

- Write the measured Day maximum luminance to memory in accordance with the requirements

- Read the 2’s complement value from the upper 12 bits of Address 7574h (ADC channel 58, WHT_CUR_CNTRL)

- Convert the 2’s complement value to a voltage using Equation 5 or Equation 6 and multiply by 2

- Multiply the WHT_CUR_CNTRL voltage measurement by 1.62

- Use Equation 9 to calculate the maximum DAC setting value for the calculated maximum WHT_CUR_CNTRL voltage and write that value to the upper 12 bits of address 3B16h

Day Minimum Luminance

- Send a SHIP Set Brightness command with data 00h to set luminance to minimum (00h)

- Adjust the upper 12 bits of the 16 bit Day ERROR_AMP_OFFSET value at address 3B0Ch to achieve the target minimum luminance

- Write the measured Day minimum luminance to memory in accordance with the requirements

- Write the calibration date to memory in accordance with the requirements

Note: Photometer is measuring the center of the display at 0 degrees vertical.

### Test Contrast

Purpose

This test verifies the display contrast meets requirements.

Acceptance Criteria

#### The production test shall verify the display contrast ratio is at least 200:1.

Product State or Condition

- Power up the control card

- Drive the pixel bus with an XGA image

- Establish communication with the control card via RS232

- Write FFh to address 763Ah to set the fault monitor override

- Set the Power/Backlight Mode knob to the NORM position

- Send a SHIP Display Mode command with data C0h to enable the backlight and display a solid grayscale pattern

- Write FFh to address 7628h, 7629h, and 762Ah to command a white screen

- Adjust the backlight brightness to approximately 200 fL

- Measure and record the luminance

- Write 00h to address 7628h, 7629h, and 762Ah to command a black screen

- Measure and record the luminance

- Calculate the contrast ratio by dividing the white luminance by the black luminance and verify the result meets the requirement

### Load DHM CPN

Purpose

This test loads the display head part number into memory.

Acceptance Criteria

#### The production test shall load the 10 digit display head part number into memory.

Product State or Condition

- Power up the control card

- Drive the pixel bus with an XGA image

- Establish communication with the control card via RS232

- Load the 10 digit part number without dashes “ABC-DEFG-HIJ” using the ASCII code of each digit as indicated in Table 4-3

Table 4-3, Display Head Part Number Memory Format

| address | data |
| --- | --- |
| 6f00H | 30H + “dIGIT A” |
| 6f01H | 30H + “dIGIT b” |
| 6f02H | 30H + “dIGIT C” |
| 6f03H | 30H + “dIGIT D” |
| 6f04H | 30H + “dIGIT E” |
| 6f05H | 30H + “dIGIT F” |
| 6f06H | 30H + “dIGIT G” |
| 6f07H | 30H + “dIGIT H” |
| 6f08H | 30H + “dIGIT I” |
| 6f09H | 30H + “dIGIT J” |

### Test Faults

Purpose

This test verifies there are no active faults and clears the fault log.

Acceptance Criteria

#### The production test shall verify there are no unexpected active faults indicated.

#### The production test shall clear the fault log.

Product State or Condition

- Power up the control card (including backlight power)

- Drive the pixel bus with an XGA image

- Establish communication with the control card via RS232

- Write FFh to address 763Ah to set the fault monitor override

- Re-calculate NVM parity and update memory to clear NVM Parity Fault

- Set the Power/Backlight Mode knob to the NORM position

- Write 8Dh to address 3B05h to disable the Pixel Data MSB Fail and Pixel Data MID Fail Faults

- Write 0h to address 763Ah to disable the fault monitor override

- Drive the pixel bus with an XGA vertical color bar pattern

- Read the fault status from address 7400h to 7403h and verify there are no active faults indicated with the following exception:

  - MFD Fail Fault (7400h Bit 0)

  - NVM Parity Fault (7401h Bit 7)

- Write BDh to address 3B05h to re-enable the Pixel Data MSB Fail and Pixel Data MID Fail Faults

- Verify the NVM Parity Fault is no longer active (7401h Bit 7)

- Write FFh to address 763Ah to set the fault monitor override

- Clear the fault log by writing 0h to address 7000h through 71FFh

- Power down the display head

# Key Characteristics

N/A