U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

Acceptance Test Procedure (ATP) for the MFD-268C4B

Document Number 946-4C45-001A Revision -

CAGE Code 0EFD0

Rockwell Collins

Contract Number: N/A

NOTICE: The contents of this document are proprietary to Rockwell Collins and shall not be disclosed, disseminated, copied, or used except for purposes expressly authorized in writing by Rockwell Collins.

U.S. Export Classification: EAR 7E994

© 2026 Rockwell Collins. All rights reserved.

|  | NAME | TITLE | Approval |
| --- | --- | --- | --- |
| Prepared By: | David Sherman | Sr Principal Engineer | N/A |
| Approved By: | Shaun Ochsner | Cognizant Engineer | N/A |
| Approved By: | Nicole Sierra | Test Engineer | N/A |
| Approved By: | Joel Sauser | Manufacturing Engineer | N/A |
| Approved By: | Ashley B. Large | Design Quality Assurance | N/A |

946-4C45-001A Rev -	Rockwell Collins Proprietary Information	Page 1

REVISION HISTORY

Rockwell Collins Proprietary Information

U.S. Export Classification: EAR 9E610.a

946-4C45-001A Rev -

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

|  |  |  |  |  |  |  | -001A | VER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | - | REV |
|  |  |  |  |  |  |  | Initial Release | DESCRIPTION |
|  |  |  |  |  |  |  | 2026-04-20 | DATE |
|  |  |  |  |  |  |  | Shaun Ochsner | APPROVED |

Page 2

STATE 3 - ENGINEERING RELEASE 2026-05-06

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

Table of Contents

- SCOPE	7

  - Applicability	7

  - Electrostatic Discharge and Handling Instructions	7

  - Inspection Verification	7

  - Critical Test Order	7

  - Safety Related Tests	7

- REFERENCE INFORMATION	8

  - Specifications	8

  - Requirements Traceability	8

  - Acceptance Test Method	8

  - Other Relevant Procedures	8

  - Definitions (Acronyms and Terms)	8

- TEST CONDITIONS	10

  - Environmental Test Conditions	10

  - Security Requirements	10

  - Initial Alignments	10

  - Product Configuration/State Types	10

    - Standard External Power Supply Requirements	10

    - Initial Power Up	11

    - Initial Power Down	11

- ENVIRONMENTAL TEST	12

  - Vibration Tests	12

  - Thermal Tests	12

  - HASS Tests	12

    - Power Consumption	12

    - BIT and CBIT Results	12

    - Video Path Verification	14

    - Ground/Open Discrete Verification	14

    - 28V/Open Discrete Verification	15

    - 5V/G Discrete Verification	16

    - Differential Analog Verification	16

    - Single Ended Analog Verification	17

    - Analog Output Verification	17

    - Bezel Function	17

    - A429 Tx and Rx Function	17

    - Serial Ports RS422/RS485 Function	19

    - Ethernet Port Function	19

    - 1553 Bus Function	19

    - Temperature Monitor Function	20

  - Other Environmental Tests	22

- FUNCTIONAL TEST	23

  - Power Up Test	23

    - Continuity	23

    - On and Off Limit Test (for DC products)	23

    - Hold Up Test	24

    - Warm and Cold Start	24

    - Heater and Fan Power Consumption	24

  - Internal Tests	25

    - BIT Status	25

    - Video Path Verification	25

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

Discrete In	26

    - Discrete In Current Sourcing	26

    - Discrete In Low Test	27

    - Discrete In High Test	28

  - Discrete In V/O	29

    - Discrete In 28V/O Current Sourcing	29

    - Discrete In 28V/O Low Test	30

    - Discrete In 28V/O High Test	30

    - Discrete In 5V/G Current Sourcing	31

    - Discrete In 5V/G Low Test	31

    - Discrete In 5V/G High Test	31

  - Discrete Out	32

    - Discrete Out Open Test	32

    - Discrete Out Active Test	32

  - ARINC 429 TX	32

    - ARINC 429 TX Characteristics Test	32

  - ARINC 429 RX	34

    - ARINC 429 RX Worse Case Input	34

  - ARINC708	35

    - ARINC708 Input	35

    - ARINC708 Input termination	36

  - Ethernet	36

    - Ethernet Data	36

  - MIL-STD-1553	37

    - 1553 Data and Voltage	37

  - Video	38

    - A818 Fiber Video In Data	38

    - A818 Fiber Video Out Power and Data	38

    - SMPTE Video In	39

    - Missing Video Input Fault	39

    - Analog Video In – STANAG 3350	40

    - Analog Video In – NTSC/PAL	40

    - Analog Video Input termination	40

  - RS422	41

    - RS422 TX Voltage and Data	41

    - RS422 RX Data	41

    - RS422 RX Failsafe Resistor Check	42

  - Key Panel	43

    - Key Panel Test	43

    - Key Panel Light Test	44

  - Display	45

    - Test Pattern	45

    - Light Leak	45

    - Luminance Test	46

    - DEP Chroma Tolerance	46

  - Analog I/O Verification	48

    - Differential Analog Input Tests	48

    - Single Ended Analog Input Tests	49

    - Analog Output Tests	49

  - RS232	50

  - Temperature Monitor Function	50

- SUB ASSEMBLY TEST	52

- CONFIGURATION TEST	53

  - Configuration Verification	53

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

SUPPLEMENTAL INFORMATION	54

- TEST DATA SHEET	55

- INTERFACE TEST COVERAGE	56

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

List of Tables

###### Table 1 - Applicable Equipment	7

###### Table 2 - Critical Test Order	7

###### Table 3 - Referenced Specifications	8

###### Table 4 - Requirements Traceability	8

###### Table 5 - Acceptance Test Methods	8

###### Table 6 - Other Relevant Procedures	8

###### Table 7 - Definitions (Acronyms and Terms)	8

###### Table 8 - Standard Input Power	11

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

SCOPE

This Acceptance Test Procedure (ATP) defines the criteria for determining acceptance via test for each deliverable item listed in Table 1, regardless of assembly level. Some of the tests listed may only be able to be performed at a lower level or in-process assembly state and includes acceptance criteria for both the factory and return to service. Both are included and specified in this document.

  - Applicability

This ATP applies to the UUT(s) (Unit Under Test) listed in the table below, in both production and return to service environments:

Table 1 - Applicable Equipment

| Item | Part Number | Equipment Type | Title |
| --- | --- | --- | --- |
| 1 | 822-3828-901 | MFD-268C4B | Multi-Function Display (Red Label) |
| 2 | 822-3828-001 | MFD-268C4B | Multi-Function Display |

  - Electrostatic Discharge and Handling Instructions

The assembly being tested contains electrostatic discharge sensitive device(s) (ESDS); handle per 489-0004-001.

  - Inspection Verification

Verify that all relevant UUT inspections have been completed prior to starting any test.

  - Critical Test Order

The tests listed in this document shall be performed in the defined in Table 2 - Critical Test Order

Table 2 - Critical Test Order

| Critical Test Order | Test Paragraph and Description |
| --- | --- |
| 1 | 5.1.1 Power Up Tests |
| 2 | All remaining functional tests |

  - Safety Related Tests

#### Tests associated with safety requirements/functions are noted. Any changes to safety related tests require Safety Engineering approval.

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

REFERENCE INFORMATION

  - Specifications

The table below lists all the specifications referenced in this document.

##### Table 3 - Referenced Specifications

| Part Number | Description |
| --- | --- |
| 946-46U3-001 | Platform Requirements Specification for the Multifunction Display (MFD) 268C4B |
| 946-4C44-001 | Acceptance Test Requirements for the MFD-268C4B |
| 489-0004-001 | Internal Handling Procedure for Electrostatic Discharge Sensitive Devices and Assemblies |
| 946-2F03-002 | Inspection Procedure for the 10.0" Liquid Crystal Display (LCD) |

  - Requirements Traceability

The table below correlates the tests in this ATP (sections 4, 5 and/or 6) to a requirements source or identified as derived (if there isn’t a requirements source).

Table 4 - Requirements Traceability

| ATP Paragraph (Test Step) | Requirement source |
| --- | --- |
| Sections 4, 5 | ATR Requirements: 946-4C44-001 |

  - Acceptance Test Method

The table below list the associated Acceptance Test Method (ATM) for each ATP paragraph (test step).

##### Table 5 - Acceptance Test Methods

| ATP Paragraph (Test Step) | Part Number | Description |
| --- | --- | --- |
| 4 ENVIRONMENTAL TEST | 683-2520-(XXX) | Environmental Acceptance Test Method |
| 5 FUNCTIONAL TEST | 683-2521-(XXX) | Top-Level Acceptance Test Method |

  - Other Relevant Procedures

The table below lists relevant procedures that are formally documented, specifically those that alter the configuration of the product such as software loading or detailed mechanical adjustments. Note, anything that changes the configuration of the product should also be listed in the product bill of material.

##### Table 6 - Other Relevant Procedures

| Part Number | Description |
| --- | --- |
| 946-4DN2-XXX | Test Software Interface Control Document |
| 946-45G7-001 | Environmental Stress Screening Procedure |

  - Definitions (Acronyms and Terms)

The table below defines the acronyms and terms used in this document.

##### Table 7 - Definitions (Acronyms and Terms)

| Acronym or Term | Definition |
| --- | --- |
| Applicable | As used in the Test Name/Test ID table, is used to indicate when a test shall be done. “ALL” in this column for all units from Table 1. |
| ATM | Acceptance Test Method |

| Acronym or Term | Definition |
| --- | --- |
| ATP | Acceptance Test Procedure |
| PTS | Production Test Software |
| LSL | Lower Spec Limit |
| N/A | Not Applicable |
| RCPN | Rockwell Collins Part Number |
| RTS | Return to Service |
| SN | Serial Number |
| USL | Upper Spec Limit |
| UUT | Unit Under Test |
| *Input Signal*_F | Input signal is defined as active Low |
| *Input Signal*_T | Input Signal is defined as active High |
| ADC | Analog to Digital Convertor |
| ARINC | Aeronautical Radio, Incorporated |
| ATM | Acceptance Test Method |
| ATR | Acceptance Test Requirements |
| BIT | Built In Test |
| CAGE | Commercial and Government Entity |
| CBIT | Continuous Built in Test |
| CRC | Cyclic Redundancy Check |
| DC | Direct Current |
| DHM | Display Head Module |
| ESD | Electrostatic Discharge |
| ESDS | Electrostatic Discharge Sensitive |
| ESSP | Environmental Stress Screening Procedure |
| GND | Ground |
| ICD | Interface Control Document |
| ID | Identification |
| IO | Input/Output |
| LCD | Liquid Crystal Display |
| LSD | Lower Specified Limit |
| Max | Maximum |
| MCN | Manufacturing Control Number |
| MFD | Multi-Function Display |
| NTSC | National Television System Committee |
| NVIS | Night Vision Imaging System |
| ODL | Optical Display Link |
| PAL | Phase Alternating Line |
| PTS | Production Test Software |
| RTN | Return |
| RTS | Return to Service |
| RX | Receive |
| SMPTE | Society of Motion Picture and Television Engineers |
| SN | Serial Number |
| TBD | To Be Determined |
| TX | Transmit |
| UDP | User Datagram Protocol |
| UUT | Unit Under Test |
| VAC | Volts Alternating Current |
| VDC | Volts Direct Current |

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

TEST CONDITIONS

Unless otherwise specified, all tests shall be performed under the following conditions.

  - Environmental Test Conditions

This section is intended to specify typical environmental conditions (not part of environmental testing), such as temperature, humidity, etc.

Normal factory environmental conditions except as specified below.

  - Security Requirements

This section is intended to specify security requirements or constraints (Secret, Top Secret, etc.) Normal factory security.

  - Initial Alignments

This section describes initial alignments that affect the acceptance testing of the unit. An alignment is a modification (hardware/software) of adjustable parameters that affect performance of the UUT.

Not applicable.

  - Product Configuration/State Types

This section specifies the different types of configurations or states of the product that are required to perform testing (Power up sequence, power off sequence, service mode, wheels on ground, etc).

    - Standard External Power Supply Requirements

Refer to Table 8 for pinout definition of the UUT input power connector signals.

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

Table 8 - Standard Input Power

| SIGNAL NAME | PIN | VOLTAGE | SURGE PEAK CURRENT (MAX) |
| --- | --- | --- | --- |
| 28VDC | J4-A | 28 V Power | 20 Amps (long ~20 ms) 30 Amp (short ~5 ms) |
| 28V RETURN | J4-E | 28 V Power | N/A |
| SAFETY GROUND | J4-D | Safety Ground | N/A |
| 115VAC_L | J1-13 | 115 VAC, 400 Hz, Single Phase Fan Power (Fan Low) | N/A |
| 115VAC_H | J1-27 | 115 VAC, 400 Hz, Single Phase Fan Power (Fan HI) | 40 W |
| DIMMING_IN_N (Dimming Low) | J1-7 J1-14 | DIMMING LOW (0 to 5 VDC LIGHTING LO) (Dual with J1-14) | N/A |
| DIMMING_IN_P (Dimming High) | J-121 J1-128 | DIMMING HIGH (0 to 5 VDC LIGHTING HI) (Dual with J1-128) | 1 W (0.2 A @ 5 VDC) |

    - Initial Power Up

#### How to power the UUT

#### Set Position and Slot straps to valid values for applicable test environment

#### Slot ID and Parity to an odd sum

#### Position ID and Parity to an odd sum

#### Set Power Rocker Switch to ON

- Set WOW_IN_F input as needed for applicable test environment

#### Apply 115 VAC, 400 Hz single phase power to 115VAC_L/H Fan Power

#### Apply 28VDC power to the 28VDC input.

#### Wait for UUT to power up.

    - Initial Power Down

#### How to power down the UUT

#### Set Power Rocker Switch to OFF

#### Remove 28VDC power to the 28VDC input.

#### Remove 115 VAC, 400 Hz single phase power to 115VAC_L/H Fan Power

#### Wait for UUT to power down.

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

ENVIRONMENTAL TEST

This section describes the environmental tests that shall be performed. All tests listed within this section shall be performed on all units listed in Table 1 in the manufacturing environment unless otherwise specified.

  - Vibration Tests

This section describes vibration acceptance tests. For each test, the requirement, conditions and stimulus, the measurement, and pass/fail criteria are described as shown. Each vibration test will have its own section number 4.1.N and associated table.

Not applicable

  - Thermal Tests

This section describes thermal acceptance tests. For each test, the requirement, conditions and stimulus, the measurement, and pass/fail criteria are described as shown. Each thermal test will have its own section number 4.2.N and associated table.

Not applicable

  - HASS Tests

This section describes HASS environmental acceptance tests. The HASS environment and execution schedule are described in the ESSP (see Table 6). For each test, the requirement, conditions and stimulus, measurement, and pass/fail criteria are described as shown. Each environmental test has its own section number 4.3.N and associated table.

    - Power Consumption

Purpose

This test validates proper power consumption of the powered on UUT.

Safety Related: No

Product State or Condition

Apply power per section 3.4.2

Running HQS SW with all interfaces active (worst case power consumption)

Input(s) and Stimulus

N/A

| Test ID/Test Name | Measurement Description | LSL | USL | UNITS | Applicable |
| --- | --- | --- | --- | --- | --- |
| 4.3.1.1 Unit Power Consumption | Measure steady state UUT power (28VDC) consumed. | TBD | 335 | W | All |

*Lower limit is based on typical results

    - BIT and CBIT Results

Purpose

This test validates no unexpected test faults are detected in the UUT over temperature and vibration.

Safety Related: Yes

Product State or Condition

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

Apply power per section 3.4.2

Input(s) and Stimulus

N.A

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 4.3.2.1 BIT Faults | Monitor for BIT status from the UUT test | No BIT failures are generated | ALL |
| 4.3.2.2 CBIT Faults | Monitor UUT for CBIT faults after startup | No CBIT faults | ALL |
| 4.3.2.3 CRC Error Detection | Monitor UUT for CRC errors | No CRC errors | ALL |

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

Video Path Verification

Purpose

This test validates that UUT display inputs operate without error.

Safety Related: Yes

Product State or Condition

UUT powered on

Input(s) and Stimulus

Provide a valid video input for each available source Command UUT to cycle selection of each display source.

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 4.3.3.1 GE5 A video source | Detect input source CRC | No display CRC fault detected | ALL |
| 4.3.3.2 GE5 B video source | Detect input source CRC | No display CRC fault detected | ALL |
| 4.3.3.3 E9171 video source | Detect input source CRC | No display CRC fault detected | ALL |
| 4.3.3.4 E9171 video source image | Monitor for missing video source error | No reported error | ALL |
| 4.3.3.5 WXR video source 1 | Monitor for missing video source error | No display CRC fault or missing input detected | ALL |
| 4.3.3.6 WXR video source 2 | Monitor for missing video source error | No display CRC fault or missing input detected | ALL |
| 4.3.3.7 Analog video source 1 | Monitor for missing video source error | No reported error | ALL |
| 4.3.3.8 Analog video source 2 | Monitor for missing video source error | No reported error | ALL |
| 4.3.3.9 Analog video source 3 | Monitor for missing video source error | No reported error | ALL |
| 4.3.3.10 SMPTE video source 1 | Monitor for missing video source error | No display CRC fault or missing input detected | ALL |
| 4.3.3.11 SMPTE video source 2 | Monitor for missing video source error | No display CRC fault or missing input detected | ALL |
| 4.3.3.12 A818 video source 1 | Monitor for missing video source error | No display CRC fault or missing input detected | ALL |
| 4.3.3.13 A818 video source 2 | Monitor for missing video source error | No display CRC fault or missing input detected | ALL |
| 4.3.3.14 A818 video output 1 | Monitor the ARINC 818 video out 1 | Display output without errors | ALL |
| 4.3.3.15 A818 video output 2 | Monitor the ARINC 818 video out 2 | Display output without errors | ALL |

Note: Analog video input can be either RGB component or composite

    - Ground/Open Discrete Verification

Purpose

This test validates that UUT discrete inputs and outputs operates correctly.

Safety Related: Yes

Product State or Condition

UUT powered on Side configured 0 or 1

Slot values set odd value 1-17

Input(s) and Stimulus

Set the discrete inputs as specified.

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

Ground input is ≤ 0.1 VDC Open is input ≥ 14 VDC

Apply power per section 3.4.2.

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 4.3.4.1 AC_STRAP_1 | Set input to ground | Verify discrete reads True | ALL |
| 4.3.4.2 AC_STRAP_2 | Set input to open | Verify discrete reads False | ALL |
| 4.3.4.3 AC_STRAP_3 | Set input to ground | Verify discrete reads True | ALL |
| 4.3.4.4 AC_STRAP_4 | Set input to open | Verify discrete reads False | ALL |
| 4.3.4.5 AC_STRAP_5 | Set input to ground | Verify discrete reads True | ALL |
| 4.3.4.6 HEATER_DISABLE_IN_F | Set input to open | Verify discrete reads False | ALL |
| 4.3.4.7 NVIS_IN_F | Set input to open | Verify discrete reads False | ALL |
| 4.3.4.8 DATALOAD_IN_F | Set input to open | Verify discrete reads False | ALL |
| 4.3.4.9 DHM_SHOP_MODE_IN_F | Set input to open | Verify discrete reads False | ALL |
| 4.3.4.10 DHM_DATA_LOAD_IN_F | Set input to open | Verify discrete reads False | ALL |
| 4.3.4.11 HOMER_SIG_D | Toggle input states | Verify discrete responds | ALL |
| 4.3.4.12 ATT_REV | Toggle input states | Verify discrete responds | ALL |
| 4.3.4.13 HDG_REV | Toggle input states | Verify discrete responds | ALL |
| 4.3.4.14 ADC_REV | Toggle input states | Verify discrete responds | ALL |
| 4.3.4.15 FMS_REV | Toggle input states | Verify discrete responds | ALL |
| 4.3.4.16 DCV_REV | Toggle input states | Verify discrete responds | ALL |
| 4.3.4.17 ZEROIZE | Toggle input states | Verify discrete responds | ALL |
| 4.3.4.18 CUR_BUTN1 J1 | Toggle input states | Verify discrete responds | ALL |
| 4.3.4.19 CUR_BUTN1 J3 | Toggle input states | Verify discrete responds | ALL |
| 4.3.4.20 CUR_BUTN2 J1 | Toggle input states | Verify discrete responds | ALL |
| 4.3.4.21 CUR_BUTN2 J3 | Toggle input states | Verify discrete responds | ALL |
| 4.3.4.22 CUR_BUTN3 J1 | Toggle input states | Verify discrete responds | ALL |
| 4.3.4.23 CUR_BUTN3 J3 | Toggle input states | Verify discrete responds | ALL |
| 4.3.4.24 CUR_BUTN4 | Toggle input states | Verify discrete responds | ALL |
| 4.3.4.25 CUR_BUTN5 | Toggle input states | Verify discrete responds | ALL |
| 4.3.4.26 DISCIN_GND_OPEN_13 | Toggle input states | Verify discrete responds | ALL |
| 4.3.4.27 DISCIN_GND_OPEN_14 | Toggle input states | Verify discrete responds | ALL |
| 4.3.4.28 DISCIN_GND_OPEN_15 | Toggle input states | Verify discrete responds | ALL |
| 4.3.4.29 DISCIN_GND_OPEN_16 | Toggle input states | Verify discrete responds | ALL |
| 4.3.4.30 DISCIN_GND_OPEN_17 | Toggle input states | Verify discrete responds | ALL |
| 4.3.4.31 DISCIN_GND_OPEN_18 | Toggle input states | Verify discrete responds | ALL |
| 4.3.4.32 DISCIN_GND_OPEN_19 | Toggle input states | Verify discrete responds | ALL |
| 4.3.4.33 DISCIN_GND_OPEN_20 | Toggle input states | Verify discrete responds | ALL |
| 4.3.4.34 DISCIN_GND_OPEN_21 | Toggle input states | Verify discrete responds | ALL |
| 4.3.4.35 DISCIN_GND_OPEN_22 | Toggle input states | Verify discrete responds | ALL |
| 4.3.4.36 DISCIN_GND_OPEN_23 | Toggle input states | Verify discrete responds | ALL |
| 4.3.4.37 DISCIN_GND_OPEN_24 | Toggle input states | Verify discrete responds | ALL |
| 4.3.4.38 DISCIN_GND_OPEN_25 | Toggle input states | Verify discrete responds | ALL |
| 4.3.4.39 DISCIN_GND_OPEN_26 | Toggle input states | Verify discrete responds | ALL |
| 4.3.4.40 DISCIN_GND_OPEN_27 | Toggle input states | Verify discrete responds | ALL |
| 4.3.4.41 DISCIN_GND_OPEN_28 | Toggle input states | Verify discrete responds | ALL |
| 4.3.4.42 DISCOUT_GND_OPEN_0 | Toggle the output state | Verify discrete responds | ALL |
| 4.3.4.43 DISCOUT_GND_OPEN_1 | Toggle the output state | Verify discrete responds | ALL |
| 4.3.4.44 DISCOUT_GND_OPEN_2 | Toggle the output state | Verify discrete responds | ALL |
| 4.3.4.45 DISCOUT_GND_OPEN_3 | Toggle the output state | Verify discrete responds | ALL |
| 4.3.4.46 DISCOUT_GND_OPEN_4 | Toggle the output state | Verify discrete responds | ALL |
| 4.3.4.47 DISCOUT_GND_OPEN_5 | Toggle the output state | Verify discrete responds | ALL |
| 4.3.4.48 DISCOUT_GND_OPEN_6 | Toggle the output state | Verify discrete responds | ALL |
| 4.3.4.49 DISCOUT_GND_OPEN_7 | Toggle the output state | Verify discrete responds | ALL |

    - 28V/Open Discrete Verification

Purpose

This test validates that UUT 28V/O discretes operates correctly.

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

Safety Related: No

Product State or Condition

UUT powered on

Input(s) and Stimulus

Set the discrete inputs as specified. True: Input set to 14.5 VDC False: Open is 9.5 VDC

Apply power per section 3.4.2.

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 4.3.5.1 DISCIN_28V_OPEN_0 | Set input 28VDC | Verify discrete reads True | ALL |
| 4.3.5.2 DISCIN_28V_OPEN_1 | Set input open | Verify discrete reads False | ALL |
| 4.3.5.3 DISCIN_28V_OPEN_2 | Set input 28VDC | Verify discrete reads True | ALL |
| 4.3.5.4 DISCIN_28V_OPEN_3 | Set input open | Verify discrete reads False | ALL |
| 4.3.5.5 DISCIN_28V_OPEN_4 | Set input 28VDC | Verify discrete reads True | ALL |
| 4.3.5.6 DISCIN_28V_OPEN_5 | Set input open | Verify discrete reads False | ALL |

    - 5V/G Discrete Verification

Purpose

This test validates that UUT 5V/O discrete operates correctly over temperature and vibration.

Safety Related: no

Product State or Condition

UUT powered on

Input(s) and Stimulus

Set the discrete inputs as specified.

True: Input is 3.5 VDC Apply power per section 3.4.2.

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 4.3.6.1 RAD_ALT_VALID | Set input to 3.5 VDC (True) | Verify discrete reads True | ALL |

    - Differential Analog Verification

Purpose

This test validates that UUT differential analog inputs operate correctly over temperature and vibration.

Safety Related: No

Product State or Condition

Apply power per section 3.4.2

| Test ID/Test Name | Stimulus | Measurement | LSL | USL | Units | Applicable |
| --- | --- | --- | --- | --- | --- | --- |
| 4.3.7.1 HOMER_SIG 0.2V Differential Under Stress | 0.2V Differential | 0.2 | 0.160 | 0.240 | V | ALL |
| 4.3.7.2 HOMER_STEER -0.3V Differential Under Stress | -0.3V Differential | -0.3 | -0.340 | -0.260 | V | ALL |
| 4.3.7.3 CURSOR_DRV_IN 15V Differential Under Stress | 15.0V Differential | 15.0 | 14.85 | 15.15 | V | ALL |
| 4.3.7.4 RAD_ALT -14V Differential Under Stress | -14.0V Differential | -14.0 | -14.068 | -13.932 | V | ALL |

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

Note: The voltage values for the Homer signals are being driven through a 1KOhm resistor to create a current input

    - Single Ended Analog Verification

Purpose

This test validates that UUT single ended analog inputs operate correctly over temperature and vibration.

Safety Related: No

Product State or Condition

Apply power per section 3.4.2

| Test ID/Test Name | Stimulus | Measurement | LSL | USL | Units | Applicable |
| --- | --- | --- | --- | --- | --- | --- |
| 4.3.8.1 CURSOR_X J1 Single Ended -1.5V Under Stress | -1.5V | -1.5 | -1.575 | -1.425 | V | ALL |
| 4.3.8.2 CURSOR_Y J1 Single Ended 1.5V Under Stress | 1.5V | 1.5 | 1.425 | 1.575 | V | ALL |
| 4.3.8.3 CURSOR_X J3 Single Ended -1.5V Under Stress | -1.5V | -1.5 | -1.575 | -1.425 | V | ALL |
| 4.3.8.4 CURSOR_Y J3 Single Ended 1.5V Under Stress | 1.5V | 1.5 | 1.425 | 1.575 | V | ALL |
| 4.3.8.5 CURSOR_X_2 Single Ended -1.5V Under Stress | -1.5V | -1.5 | -1.575 | -1.425 | V | ALL |
| 4.3.8.6 CURSOR_Y_2 Single Ended 1.5V Under Stress | 1.5V | 1.5 | 1.425 | 1.575 | V | ALL |

    - Analog Output Verification

Purpose

This test validates that UUT analog output operates correctly over temperature and vibration.

Safety Related: No

Product State or Condition

Apply power per section 3.4.2

Apply a 125 Ohm resistor to ground on each pin

| Test ID/Test Name | Stimulus | Measurement | LSL | USL | Units | Applicable |
| --- | --- | --- | --- | --- | --- | --- |
| 4.3.9.1 MSC_PWR_P7_5V/MSC_PWR_N7_5V J1 | 15.0V Differential | 15.0 | 14.85 | 15.15 | V | ALL |
| 4.3.9.2 MSC_PWR_P7_5V/MSC_PWR_N7_5V J3 | 15.0V Differential | 15.0 | 14.85 | 15.15 | V | ALL |

    - Bezel Function

Purpose

This test validates that UUT bezel lighting functions correctly.

Safety Related: No

Product State or Condition

UUT powered on

Input(s) and Stimulus

Apply 5 VDC to differential input

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 4.3.10.1 Bezel 5VDC Lightning Max | Apply 5VDC to 5VDC lighting input | Verify the bezel has uniform lighting | ALL |

    - A429 Tx and Rx Function

Purpose

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

This test validates that UUT ARINC 429 inputs and outputs function over temperature and vibration.

Safety Related: No

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 4.3.11.1 A582_IN0_H A582_IN0_L | Monitor the received ARINC data | Verify the received ARINC data matches the expected value | ALL |
| 4.3.11.2 A429_IN1_H A429_IN1_L | Monitor the received ARINC data | Verify the received ARINC data matches the expected value | ALL |
| 4.3.11.3 A429_IN2_H A429_IN2_L | Monitor the received ARINC data | Verify the received ARINC data matches the expected value | ALL |
| 4.3.11.4 A429_IN3_H A429_IN3_L | Monitor the received ARINC data | Verify the received ARINC data matches the expected value | ALL |
| 4.3.11.5 A429_IN4_H A429_IN4_L | Monitor the received ARINC data | Verify the received ARINC data matches the expected value | ALL |
| 4.3.11.6 A429_IN5_H A429_IN5_L | Monitor the received ARINC data | Verify the received ARINC data matches the expected value | ALL |
| 4.3.11.7 A429_IN6_H A429_IN6_L | Monitor the received ARINC data | Verify the received ARINC data matches the expected value | ALL |
| 4.3.11.8 A429_IN7_H A429_IN7_L | Monitor the received ARINC data | Verify the received ARINC data matches the expected value | ALL |
| 4.3.11.9 A429_IN8_H A429_IN8_L | Monitor the received ARINC data | Verify the received ARINC data matches the expected value | ALL |
| 4.3.11.10 A429_IN9_H A429_IN9_L | Monitor the received ARINC data | Verify the received ARINC data matches the expected value | ALL |
| 4.3.11.11 A429_IN10_H A429_IN10_L | Monitor the received ARINC data | Verify the received ARINC data matches the expected value | ALL |
| 4.3.11.12 A429_IN11_H A429_IN11_L | Monitor the received ARINC data | Verify the received ARINC data matches the expected value | ALL |
| 4.3.11.13 A429_IN12_H A429_IN12_L | Monitor the received ARINC data | Verify the received ARINC data matches the expected value | ALL |
| 4.3.11.14 A429_IN13_H A429_IN13_L | Monitor the received ARINC data | Verify the received ARINC data matches the expected value | ALL |
| 4.3.11.15 A429_IN14_H A429_IN14_L | Monitor the received ARINC data | Verify the received ARINC data matches the expected value | ALL |
| 4.3.11.16 A429_IN15_H A429_IN15_L | Monitor the received ARINC data | Verify the received ARINC data matches the expected value | ALL |
| 4.3.11.17 A429_OUT0_H A429_OUT0_L | Monitor the transmitted ARINC data | Verify the output ARINC data matches the expected value | ALL |
| 4.3.11.18 A429_OUT1_H A429_OUT1_L | Monitor the transmitted ARINC data | Verify the output ARINC data matches the expected value | ALL |
| 4.3.11.19 A429_OUT2_H A429_OUT2_L | Monitor the transmitted ARINC data | Verify the output ARINC data matches the expected value | ALL |
| 4.3.11.20 A429_OUT3_H A429_OUT3_L | Monitor the transmitted ARINC data | Verify the output ARINC data matches the expected value | ALL |
| 4.3.11.21 A429_OUT4_H A429_OUT4_L | Monitor the transmitted ARINC data | Verify the output ARINC data matches the expected value | ALL |

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 4.3.11.22 A429_OUT5_H A429_OUT5_L | Monitor the transmitted ARINC data | Verify the output ARINC data matches the expected value | ALL |
| 4.3.11.23 A429_OUT6_H A429_OUT6_L | Monitor the transmitted ARINC data | Verify the output ARINC data matches the expected value | ALL |
| 4.3.11.24 A429_OUT7_H A429_OUT7_L | Monitor the transmitted ARINC data | Verify the output ARINC data matches the expected value | ALL |

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

*Inputs and Output functionality can be monitored using loopbacks

    - Serial Ports RS422/RS485 Function

Purpose

This test validates that UUT RS422 inputs and outputs function over temperature and vibration.

Safety Related: No

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 4.3.12.1 RS-422_485_RX1_H RS-422_485_RX1_L | Monitor the received RS422 data | Verify the received RS422 data matches the expected data | ALL |
| 4.3.12.2 RS-422_485_RX2_H RS-422_485_RX2_L | Monitor the received RS422 data | Verify the received RS422 data matches the expected data | ALL |
| 4.3.12.3 RS-422_485_RX3_H RS-422_485_RX3_L | Monitor the received RS422 data | Verify the received RS422 data matches the expected data | ALL |
| 4.3.12.4 RS-422_485_TX1_H RS-422_485_TX1_L | Monitor the transmitted RS422 data | Verify the output RS422 data matches the expected value | ALL |
| 4.3.12.5 RS-422_485_TX2_H RS-422_485_TX2_L | Monitor the transmitted RS422 data | Verify the output RS422 data matches the expected value | ALL |
| 4.3.12.6 RS-422_485_TX3_H RS-422_485_TX3_L | Monitor the transmitted RS422 data | Verify the output RS422 data matches the expected value | ALL |

*Inputs and Output functionality can be monitored using loopbacks

    - Ethernet Port Function

Purpose

This test validates that UUT ethernet ports function over temperature and vibration.

Safety Related: Yes

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 4.3.13.1 GB_ETH1 | Monitor the ethernet port data transmission | Verify the ethernet is transmitting/receiving data at 1 GbE | ALL |
| 4.3.13.2 GB_ETH2 | Monitor the ethernet port data transmission | Verify the ethernet is transmitting/receiving data at 1 GbE | ALL |

*Inputs and Output functionality can be monitored using loopbacks through an ethernet switch

    - 1553 Bus Function

Purpose

This test validates that UUT bezel lighting functions correctly.

Safety Related: No

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 4.3.14.1 MILBUS0_A_DP MILBUS0_A_DN | Monitor the 1553 Bus traffic | Verify the 1553 port is functioning | ALL |
| 4.3.14.2 MILBUS0_B_DP MILBUS0_B_DN | Monitor the 1553 Bus traffic | Verify the 1553 port is functioning | ALL |

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

*Inputs and Output functionality can be monitored using loopbacks

    - Temperature Monitor Function

Purpose

This test validates that UUT temperature monitor functions over temperature and vibration.

Safety Related: Yes

| Test ID/Test Name | Input(s) and Stimulus | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 4.3.15.1 LCD Top Temp Cold | Evaluate unit temperature monitor after cold soak power up. | Temperature sensor reads < -30 DegC | ALL |
| 4.3.15.2 LCD Center Cold | Evaluate unit temperature monitor after cold soak power up. | Temperature sensor reads < -30 DegC | ALL |
| 4.3.15.3 LCD Bottom Cold | Evaluate unit temperature monitor after cold soak power up. | Temperature sensor reads < -30 DegC | ALL |
| 4.3.15.4 LCD Board Cold | Evaluate unit temperature monitor after cold soak power up. | Temperature sensor reads < -30 DegC | ALL |
| 4.3.15.5 Backlight Board Cold | Evaluate unit temperature monitor after cold soak power up. | Temperature sensor reads < -30 DegC | ALL |
| 4.3.15.6 GPX MM Cold | Evaluate unit temperature monitor after cold soak power up. | Temperature sensor reads < -30 DegC | ALL |
| 4.3.15.7 GPX U21 Cold | Evaluate unit temperature monitor after cold soak power up. | Temperature sensor reads < -30 DegC | ALL |
| 4.3.15.8 GPX GPU Cold | Evaluate unit temperature monitor after cold soak power up. | Temperature sensor reads < -30 DegC | ALL |
| 4.3.15.9 SCX3 HM Cold | Evaluate unit temperature monitor after cold soak power up. | Temperature sensor reads < -30 DegC | ALL |
| 4.3.15.10 SCX3 CCA Cold | Evaluate unit temperature monitor after cold soak power up. | Temperature sensor reads < -30 DegC | ALL |
| 4.3.15.11 SCX3 T2080 Cold | Evaluate unit temperature monitor after cold soak power up. | Temperature sensor reads < -30 DegC | ALL |
| 4.3.15.12 IOX3 MM0 Cold | Evaluate unit temperature monitor after cold soak power up. | Temperature sensor reads < -30 DegC | ALL |
| 4.3.15.13 IOX3 MM1 Cold | Evaluate unit temperature monitor after cold soak power up. | Temperature sensor reads < -30 DegC | ALL |
| 4.3.15.14 IOX3 U47 Cold | Evaluate unit temperature monitor after cold soak power up. | Temperature sensor reads < -30 DegC | ALL |
| 4.3.15.15 IOX3 U48 Cold | Evaluate unit temperature monitor after cold soak power up. | Temperature sensor reads < -30 DegC | ALL |
| 4.3.15.16 IOX3 U50 Cold | Evaluate unit temperature monitor after cold soak power up. | Temperature sensor reads < -30 DegC | ALL |
| 4.3.15.17 IOX3 U51 Cold | Evaluate unit temperature monitor after cold soak power up. | Temperature sensor reads < -30 DegC | ALL |
| 4.3.15.18 VMX3 MM Cold | Evaluate unit temperature monitor after cold soak power up. | Temperature sensor reads < -30 DegC | ALL |
| 4.3.15.19 VMX3 U42 Cold | Evaluate unit temperature monitor after cold soak power up. | Temperature sensor reads < -30 DegC | ALL |
| 4.3.15.20 VMX3 U47 Cold | Evaluate unit temperature monitor after cold soak power up. | Temperature sensor reads < -30 DegC | ALL |
| 4.3.15.21 VMX3 U48 Cold | Evaluate unit temperature monitor after cold soak power up. | Temperature sensor reads < -30 DegC | ALL |
| 4.3.15.22 VMX3 U88 Cold | Evaluate unit temperature monitor after cold soak power up. | Temperature sensor reads < -30 DegC | ALL |

| Test ID/Test Name | Input(s) and Stimulus | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 4.3.15.23 VMX3 U89 Cold | Evaluate unit temperature monitor after cold soak power up. | Temperature sensor reads < -30 DegC | ALL |
| 4.3.15.24 VMX3 MRV Cold | Evaluate unit temperature monitor after cold soak power up. | Temperature sensor reads < -30 DegC | ALL |
| 4.3.15.25 RIO-M ODL Cold | Evaluate unit temperature monitor after cold soak power up. | Temperature sensor reads < -30 DegC | ALL |
| 4.3.15.26 RIO-M ADC Cold | Evaluate unit temperature monitor after cold soak power up. | Temperature sensor reads < -30 DegC | ALL |
| 4.3.15.27 RIO-D Cold | Evaluate unit temperature monitor after cold soak power up. | Temperature sensor reads < -30 DegC | ALL |
| 4.3.15.28 LCD Top Temp Hot | Evaluate unit temperature monitor after hot soak power cycle. | Temperature sensor reads > 50 DegC | ALL |
| 4.3.15.29 LCD Center Hot | Evaluate unit temperature monitor after hot soak power cycle. | Temperature sensor reads > 50 DegC | ALL |
| 4.3.15.30 LCD Bottom Hot | Evaluate unit temperature monitor after hot soak power cycle. | Temperature sensor reads > 50 DegC | ALL |
| 4.3.15.31 LCD Board Hot | Evaluate unit temperature monitor after hot soak power cycle. | Temperature sensor reads > 50 DegC | ALL |
| 4.3.15.32 Backlight Board Hot | Evaluate unit temperature monitor after hot soak power cycle. | Temperature sensor reads > 50 DegC | ALL |
| 4.3.15.33 GPX MM Hot | Evaluate unit temperature monitor after hot soak power cycle. | Temperature sensor reads > 50 DegC | ALL |
| 4.3.15.34 GPX U21 Hot | Evaluate unit temperature monitor after hot soak power cycle. | Temperature sensor reads > 50 DegC | ALL |
| 4.3.15.35 GPX GPU Hot | Evaluate unit temperature monitor after hot soak power cycle. | Temperature sensor reads > 50 DegC | ALL |
| 4.3.15.36 SCX3 HM Hot | Evaluate unit temperature monitor after hot soak power cycle. | Temperature sensor reads > 50 DegC | ALL |
| 4.3.15.37 SCX3 CCA Hot | Evaluate unit temperature monitor after hot soak power cycle. | Temperature sensor reads > 50 DegC | ALL |
| 4.3.15.38 SCX3 T2080 Hot | Evaluate unit temperature monitor after hot soak power cycle. | Temperature sensor reads > 50 DegC | ALL |
| 4.3.15.39 IOX3 MM0 Hot | Evaluate unit temperature monitor after hot soak power cycle. | Temperature sensor reads > 50 DegC | ALL |
| 4.3.15.40 IOX3 MM1 Hot | Evaluate unit temperature monitor after hot soak power cycle. | Temperature sensor reads > 50 DegC | ALL |
| 4.3.15.41 IOX3 U47 Hot | Evaluate unit temperature monitor after hot soak power cycle. | Temperature sensor reads > 50 DegC | ALL |
| 4.3.15.42 IOX3 U48 Hot | Evaluate unit temperature monitor after hot soak power cycle. | Temperature sensor reads > 50 DegC | ALL |
| 4.3.15.43 IOX3 U50 Hot | Evaluate unit temperature monitor after hot soak power cycle. | Temperature sensor reads > 50 DegC | ALL |
| 4.3.15.44 IOX3 U51 Hot | Evaluate unit temperature monitor after hot soak power cycle. | Temperature sensor reads > 50 DegC | ALL |
| 4.3.15.45 VMX3 MM Hot | Evaluate unit temperature monitor after hot soak power cycle. | Temperature sensor reads > 50 DegC | ALL |
| 4.3.15.46 VMX3 U42 Hot | Evaluate unit temperature monitor after hot soak power cycle. | Temperature sensor reads > 50 DegC | ALL |
| 4.3.15.47 VMX3 U47 Hot | Evaluate unit temperature monitor after hot soak power cycle. | Temperature sensor reads > 50 DegC | ALL |
| 4.3.15.48 VMX3 U48 Hot | Evaluate unit temperature monitor after hot soak power cycle. | Temperature sensor reads > 50 DegC | ALL |
| 4.3.15.49 VMX3 U88 Hot | Evaluate unit temperature monitor after hot soak power cycle. | Temperature sensor reads > 50 DegC | ALL |
| 4.3.15.50 VMX3 U89 Hot | Evaluate unit temperature monitor after hot soak power cycle. | Temperature sensor reads > 50 DegC | ALL |
| 4.3.15.51 VMX3 MRV Hot | Evaluate unit temperature monitor after hot soak power cycle. | Temperature sensor reads > 50 DegC | ALL |

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

| Test ID/Test Name | Input(s) and Stimulus | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 4.3.15.52 RIO-M ODL Hot | Evaluate unit temperature monitor after hot soak power cycle. | Temperature sensor reads > 50 DegC | ALL |
| 4.3.15.53 RIO-M ADC Hot | Evaluate unit temperature monitor after hot soak power cycle. | Temperature sensor reads > 50 DegC | ALL |
| 4.3.15.54 RIO-D Hot | Evaluate unit temperature monitor after hot soak power cycle. | Temperature sensor reads > 50 DegC | ALL |

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

  - Other Environmental Tests

Not applicable

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

FUNCTIONAL TEST

This section describes the functional tests to be performed in the manufacturing and return to service environments. If a test is for manufacturing only, return to service only or has different pass conditions, then it will be indicated as such in the LSL/USL or pass condition column. All tests listed within this section are to be performed on the units listed in Table 1 unless otherwise specified in the applicability column. Each functional test will have its own section number 5.N and associated table.

  - Power Up Test

    - Continuity

Purpose

Verify the power signals are not shorted or low resistance to ground, as well as ground not being open. This will reduce the risk of damaging equipment when power is applied.

Safety Related: No

Product State or Condition

No power applied

Input(s) and Stimulus

None

| Test ID/Test Name | Measurement Description | LSL | USL | UNITS | Applicable |
| --- | --- | --- | --- | --- | --- |
| 5.1.1.1 115VAC_L to SAFETY_GROUND | Measure resistance between 115_VAC_L to SAFETY_GROUND | 5M | N/A | Ohms | ALL |
| 5.1.1.2 115VAC_H to SAFETY_GROUND | Measure resistance between 115_VAC_H to SAFETY_GROUND | 5M | N/A | Ohms | ALL |
| 5.1.1.3 28VDC to SAFETY_GROUND | Measure resistance between 28VDC to SAFETY_GROUND | 5M | N/A | Ohms | ALL |
| 5.1.1.4 28V RETURN to SAFETY_GROUND | Measure resistance between 28V_RETURN to SAFETY_GROUND | 5M | N/A | Ohms | ALL |
| 5.1.1.5 SAFETY_GROUND to Chassis_Ground | Measure resistance between SAFETY_GROUND to Chassis_Ground | 0 | 1 | Ohms | ALL |

    - On and Off Limit Test (for DC products)

Purpose

This test is to verify the on/off threshold circuitry is defect free. It does this by checking the turn on and off voltage is between two thresholds.

Safety Related: No

Product State or Condition

No power applied

| Test ID/Test Name | Stimulus | Measurement | Pass Criteria | Applicable |
| --- | --- | --- | --- | --- |
| 5.1.2.1 28VDC Below OFF to ON Threshold | Apply 17VDC to 28V_INPUT | UUT power state | Verify UUT is OFF | ALL |
| 5.1.2.2 28VDC Above OFF to ON Threshold | Apply 18VDC to 28V_INPUT | UUT power state | Verify UUT is ON | ALL |
| 5.1.2.3 28VDC Above ON to OFF Threshold | UUT already powered on. Apply 17VDC to 28V_INPUT | UUT power state | Verify UUT is ON | ALL |

| Test ID/Test Name | Stimulus | Measurement | Pass Criteria | Applicable |
| --- | --- | --- | --- | --- |
| 5.1.2.4 28VDC Below ON to OFF Threshold | UUT Powered on with 17VDC to the 28V_INPUT Decrement power voltage by 0.1VDC until unit turns off | UUT power state | Verify UUT turns off at 16 VDC or below | ALL |

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

*Verifying UUT is on/off may be determined multiple acceptable ways, such as ability to communicate, current draw, ect.

    - Hold Up Test

Purpose

Verify the hold up circuit is defect free.

Safety Related: No

Product State or Condition

Apply power per section 3.4

Input(s) and Stimulus

See table

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 5.1.3.1 Holdup Playthrough | Write to a volatile register in the UUT. Interrupt power for 195±4ms. Verify the register contents remained. | Memory Contents not erased | ALL |
| 5.1.3.2 Intermittent Power | Write to a volatile register in the UUT. Interrupt power for 195±4ms and restore power at least 3 times within a 2.1-2.5 sec interval Verify the register contents remained. | Memory Contents not erased | ALL |

* There are different ways to verify a reset didn’t occur. Other methods are acceptable.

    - Warm and Cold Start

Purpose

Verify the warm/cold start circuit is defect free.

Safety Related: No

Product State or Condition

Apply power per section 3.4

Input(s) and Stimulus

See table

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 5.1.4.1 Warm Start | Power UUT in cold start state (off for >12 seconds) Interrupt power for 2 seconds Verify the UUT indicates a warm start | Warm start is indicated | ALL |
| 5.1.4.2 Cold Start | Power UUT in cold start state (off for >12 seconds) Verify the UUT indicates a cold start | Cold Start is indicated | ALL |

    - Heater and Fan Power Consumption

Purpose

Verify the heater, fan and their circuits are defect free.

Safety Related: No

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

Product State or Condition

Apply power per section 3.4

Input(s) and Stimulus

See table

| Test ID/Test Name | Measurement Description | LSL | USL | UNITS | Applicable |
| --- | --- | --- | --- | --- | --- |
| 5.1.5.1 Power Consumption No Heater | Using UUT communication, turn the heater off. Record the main power consumption of the powered UUT. | TBD | 195 | W | ALL |
| 5.1.5.2 Heater Current | Record the main power draw Using UUT communication, turn the heater on Record the main power draw Verify the difference in power consumption | TBD | 140 | W | ALL |
| 5.1.5.3 Fan Current | Using UUT communication, turn the fan on Verify the 115VAC power consumption | TBD | 40 | W | ALL |

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 5.1.5.4 Fan Disable | Using UUT communication, command the cooling fan off, Verify fan is disabled, then re-enable fan operation. | Fan turns off when commanded | ALL |

  - Internal Tests

    - BIT Status

Purpose

Verify there are no defects affecting internal functionality as well as provide additional fault isolation.

Safety Related: Yes

Product State or Condition

Apply power per section 3.4

Input(s) and Stimulus

See table

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 5.2.1.1 BIT Test | Verify BIT result is PASS | No unexpected BIT failures | ALL |
| 5.2.1.2 Configuration CRC Check | Verify configuration file and CRC errors are not present | No config or CRC errors | ALL |

    - Video Path Verification

Purpose

This test validates that UUT display inputs operate without error.

Safety Related: Yes

Product State or Condition

UUT powered on

Input(s) and Stimulus

PTS software to command a combination of 2D and 3D images.

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 5.2.2.1 GE5 A-1 Internal Video Path | Verify 2D test image presentation | No image errors | ALL |
| 5.2.2.2 GE5 A-2 Internal Video Path | Verify 2D test image presentation | No image errors | ALL |
| 5.2.2.3 GE5 B-1 Internal Video Path | Verify 2D test image presentation | No image errors | ALL |
| 5.2.2.4 GE5 B-2 Internal Video Path | Verify 2D test image presentation | No image errors | ALL |
| 5.2.2.5 E9171-1 Internal Video Path | Verify 3D test image presentation | No image errors | ALL |
| 5.2.2.6 E9171-2 Internal Video Path | Verify 3D test image presentation | No image errors | ALL |
| 5.2.2.7 E9171-3 Internal Video Path | Verify 3D test image presentation | No image errors | ALL |
| 5.2.2.8 E9171-4 Internal Video Path | Verify 3D test image presentation | No image errors | ALL |

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

Note: Images can be verified visually or via CRC checks

### Discrete In

### Discrete In Current Sourcing

Purpose

Verify the discrete inputs have no current sourcing (resistive) defects, that may be due to wrong/missing parts or shorts to other input/outputs.

Safety Related: No

Product State or Condition

Apply power per section 3.4

Input(s) and Stimulus

For each pin apply a 10k pulldown to ground

| Test ID/Test Name | Measurement Description | LSL | USL | UNITS | Applicable |
| --- | --- | --- | --- | --- | --- |
| 5.3.1.1 AC_STRAP_1 Current Sourcing | Verify voltage at the pin | 2.8 | 3.4 | V | ALL |
| 5.3.1.2 AC_STRAP_2 Current Sourcing | Verify voltage at the pin | 2.8 | 3.4 | V | ALL |
| 5.3.1.3 AC_STRAP_3 Current Sourcing | Verify voltage at the pin | 2.8 | 3.4 | V | ALL |
| 5.3.1.4 AC_STRAP_4 Current Sourcing | Verify voltage at the pin | 2.8 | 3.4 | V | ALL |
| 5.3.1.5 AC_STRAP_5 Current Sourcing | Verify voltage at the pin | 2.8 | 3.4 | V | ALL |
| 5.3.1.6 POSITION_IN_0 Current Sourcing | Verify voltage at the pin | 4.8 | 5.4 | V | ALL |
| 5.3.1.7 POSITION_IN_1 Current Sourcing | Verify voltage at the pin | 4.8 | 5.4 | V | ALL |
| 5.3.1.8 POSITION_IN_P Current Sourcing | Verify voltage at the pin | 4.8 | 5.4 | V | ALL |
| 5.3.1.9 SLOT_IN_0 Current Sourcing | Verify voltage at the pin | 4.8 | 5.4 | V | ALL |
| 5.3.1.10 SLOT_IN_1 Current Sourcing | Verify voltage at the pin | 4.8 | 5.4 | V | ALL |
| 5.3.1.11 SLOT_IN_2 Current Sourcing | Verify voltage at the pin | 4.8 | 5.4 | V | ALL |
| 5.3.1.12 SLOT_IN_3 Current Sourcing | Verify voltage at the pin | 4.8 | 5.4 | V | ALL |
| 5.3.1.13 SLOT_IN_4 Current Sourcing | Verify voltage at the pin | 4.8 | 5.4 | V | ALL |
| 5.3.1.14 SLOT_IN_P Current Sourcing | Verify voltage at the pin | 4.8 | 5.4 | V | ALL |
| 5.3.1.15 HEATER_DISABLE_IN_F Current Sourcing | Verify voltage at the pin | 4.8 | 5.4 | V | ALL |
| 5.3.1.16 NVIS_IN_F Current Sourcing | Verify voltage at the pin | 4.8 | 5.4 | V | ALL |
| 5.3.1.17 WOW_IN_F Current Sourcing | Verify voltage at the pin | 4.8 | 5.4 | V | ALL |
| 5.3.1.18 DATALOAD_IN_F Current Sourcing | Verify voltage at the pin | 4.8 | 5.4 | V | ALL |
| 5.3.1.19 DHM_SHOP_MODE_IN_F Current Sourcing | Verify voltage at the pin | 4.8 | 5.4 | V | ALL |
| 5.3.1.20 DHM_DATA_LOAD_IN_F Current Sourcing | Verify voltage at the pin | 4.8 | 5.4 | V | ALL |
| 5.3.1.21 HOMER_SIG_D Current Sourcing | Verify voltage at the pin | 1.7 | 2.4 | V | ALL |
| 5.3.1.22 ATT_REV Current Sourcing | Verify voltage at the pin | 1.7 | 2.4 | V | ALL |
| 5.3.1.23 HDG_REV Current Sourcing | Verify voltage at the pin | 1.7 | 2.4 | V | ALL |

| Test ID/Test Name | Measurement Description | LSL | USL | UNITS | Applicable |
| --- | --- | --- | --- | --- | --- |
| 5.3.1.24 ADC_REV Current Sourcing | Verify voltage at the pin | 1.7 | 2.4 | V | ALL |
| 5.3.1.25 FMS_REV Current Sourcing | Verify voltage at the pin | 1.7 | 2.4 | V | ALL |
| 5.3.1.26 DCV_REV Current Sourcing | Verify voltage at the pin | 1.7 | 2.4 | V | ALL |
| 5.3.1.27 ZEROIZE Current Sourcing | Verify voltage at the pin | 1.7 | 2.4 | V | ALL |
| 5.3.1.28 CUR_BUTN1 Current Sourcing | Verify voltage at the pin | 1.7 | 2.4 | V | ALL |
| 5.3.1.29 CUR_BUTN1 Current Sourcing | Verify voltage at the pin | 1.7 | 2.4 | V | ALL |
| 5.3.1.30 CUR_BUTN2 Current Sourcing | Verify voltage at the pin | 1.7 | 2.4 | V | ALL |
| 5.3.1.31 CUR_BUTN2 Current Sourcing | Verify voltage at the pin | 1.7 | 2.4 | V | ALL |
| 5.3.1.32 CUR_BUTN3 Current Sourcing | Verify voltage at the pin | 1.7 | 2.4 | V | ALL |
| 5.3.1.33 CUR_BUTN3 Current Sourcing | Verify voltage at the pin | 1.7 | 2.4 | V | ALL |
| 5.3.1.34 CUR_BUTN4 Current Sourcing | Verify voltage at the pin | 1.7 | 2.4 | V | ALL |
| 5.3.1.35 CUR_BUTN5 Current Sourcing | Verify voltage at the pin | 1.7 | 2.4 | V | ALL |
| 5.3.1.36 DISCIN_GND_OPEN_13 Current Sourcing | Verify voltage at the pin | 1.7 | 2.4 | V | ALL |
| 5.3.1.37 DISCIN_GND_OPEN_14 Current Sourcing | Verify voltage at the pin | 1.7 | 2.4 | V | ALL |
| 5.3.1.38 DISCIN_GND_OPEN_15 Current Sourcing | Verify voltage at the pin | 1.7 | 2.4 | V | ALL |
| 5.3.1.39 DISCIN_GND_OPEN_16 Current Sourcing | Verify voltage at the pin | 1.7 | 2.4 | V | ALL |
| 5.3.1.40 DISCIN_GND_OPEN_17 Current Sourcing | Verify voltage at the pin | 1.7 | 2.4 | V | ALL |
| 5.3.1.41 DISCIN_GND_OPEN_18 Current Sourcing | Verify voltage at the pin | 1.7 | 2.4 | V | ALL |
| 5.3.1.42 DISCIN_GND_OPEN_19 Current Sourcing | Verify voltage at the pin | 1.7 | 2.4 | V | ALL |
| 5.3.1.43 DISCIN_GND_OPEN_20 Current Sourcing | Verify voltage at the pin | 1.7 | 2.4 | V | ALL |
| 5.3.1.44 DISCIN_GND_OPEN_21 Current Sourcing | Verify voltage at the pin | 1.7 | 2.4 | V | ALL |
| 5.3.1.45 DISCIN_GND_OPEN_22 Current Sourcing | Verify voltage at the pin | 1.7 | 2.4 | V | ALL |
| 5.3.1.46 DISCIN_GND_OPEN_23 Current Sourcing | Verify voltage at the pin | 1.7 | 2.4 | V | ALL |
| 5.3.1.47 DISCIN_GND_OPEN_24 Current Sourcing | Verify voltage at the pin | 2.8 | 3.4 | V | ALL |
| 5.3.1.48 DISCIN_GND_OPEN_25 Current Sourcing | Verify voltage at the pin | 2.8 | 3.4 | V | ALL |
| 5.3.1.49 DISCIN_GND_OPEN_26 Current Sourcing | Verify voltage at the pin | 2.8 | 3.4 | V | ALL |
| 5.3.1.50 DISCIN_GND_OPEN_27 Current Sourcing | Verify voltage at the pin | 2.8 | 3.4 | V | ALL |
| 5.3.1.51 DISCIN_GND_OPEN_28 Current Sourcing | Verify voltage at the pin | 2.8 | 3.4 | V | ALL |

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

### Discrete In Low Test

Purpose

Verify inputs have no voltage detection defects that may be due to incorrect or missing parts and correctly detects a ground/open state.

Safety Related: No

Product State or Condition

STATE 3 - ENGINEERING RELEASE 2026-05-06

Apply power per section 3.4

Input(s) and Stimulus

Apply the stimulus listed to each input in the table with all others in the opposite state for each test.

Test ID/Test Name

      - AC_STRAP_1 Low

      - AC_STRAP_2 Low

      - AC_STRAP_3 Low

      - AC_STRAP_4 Low

      - AC_STRAP_5 Low

      - DISCIN_GND_OPEN_24 Low

      - DISCIN_GND_OPEN_25 Low

      - DISCIN_GND_OPEN_26 Low

      - DISCIN_GND_OPEN_27 Low

      - DISCIN_GND_OPEN_28 Low

      - POSITION_IN_0 Low

      - POSITION_IN_1 Low

Input(s) and Stimulus

Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin

Pass Criteria Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low

Applicable

ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL

Test ID/Test Name

      - POSITION_IN_P Low

      - SLOT_IN_0 Low

      - SLOT_IN_1 Low

      - SLOT_IN_2 Low

      - SLOT_IN_3 Low

      - SLOT_IN_4 Low

      - SLOT_IN_P Low

      - HEATER_DISABLE_IN_F Low

      - NVIS_IN_F Low

      - WOW_IN_F Low

      - DATALOAD_IN_F Low

      - DHM_SHOP_MODE_IN_F Low

      - DHM_DATA_LOAD_IN_F Low

      - HOMER_SIG_D Low

      - ATT_REV Low

      - HDG_REV Low

      - ADC_REV Low

      - FMS_REV Low

      - DCV_REV Low

      - ZEROIZE Low

      - CUR_BUTN1 J1 Low

      - CUR_BUTN1 J3 Low

      - CUR_BUTN2 J1 Low

      - CUR_BUTN2 J3 Low

      - CUR_BUTN3 J1 Low

      - CUR_BUTN3 J3 Low

      - CUR_BUTN4 Low

      - CUR_BUTN5 Low

      - DISCIN_GND_OPEN_13 Low

      - DISCIN_GND_OPEN_14 Low

      - DISCIN_GND_OPEN_15 Low

      - DISCIN_GND_OPEN_16 Low

      - DISCIN_GND_OPEN_17 Low

      - DISCIN_GND_OPEN_18 Low

      - DISCIN_GND_OPEN_19 Low

      - DISCIN_GND_OPEN_20 Low

      - DISCIN_GND_OPEN_21 Low

      - DISCIN_GND_OPEN_22 Low

      - DISCIN_GND_OPEN_23 Low

Input(s) and Stimulus

Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin Apply 3 (-0/+0.1) VDC to the pin

Pass Criteria Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low Discrete reads Low

Applicable

ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

### Discrete In High Test

Purpose

Verify inputs have no voltage detection defects that may be due to incorrect or missing parts and correctly detects a ground/open state.

Safety Related: No

Product State or Condition

Apply power per section 3.4

Input(s) and Stimulus

Apply the stimulus listed to each input in the table with all others in the opposite state for each test.

Test ID/Test Name

      - AC_STRAP_1 High

      - AC_STRAP_2 High

      - AC_STRAP_3 High

      - AC_STRAP_4 High

      - AC_STRAP_5 High

      - DISCIN_GND_OPEN_24 High

      - DISCIN_GND_OPEN_25 High

      - DISCIN_GND_OPEN_26 High

      - DISCIN_GND_OPEN_27 High

      - DISCIN_GND_OPEN_28 High

      - POSITION_IN_0 High

      - POSITION_IN_1 High

      - POSITION_IN_P High

      - SLOT_IN_0 High

      - SLOT_IN_1 High

      - SLOT_IN_2 High

      - SLOT_IN_3 High

      - SLOT_IN_4 High

      - SLOT_IN_P High

      - HEATER_DISABLE_IN_F High

      - NVIS_IN_F High

      - WOW_IN_F High

      - DATALOAD_IN_F High

      - DHM_SHOP_MODE_IN_F High

      - DHM_DATA_LOAD_IN_F High

      - HOMER_SIG_D High

      - ATT_REV High

      - HDG_REV High

      - ADC_REV High

      - FMS_REV High

      - DCV_REV High

      - ZEROIZE High

      - CUR_BUTN1 J1 High

      - CUR_BUTN1 J3 High

      - CUR_BUTN2 J1 High

      - CUR_BUTN2 J3 High

      - CUR_BUTN3 J1 High

      - CUR_BUTN3 J3 High

      - CUR_BUTN4 High

      - CUR_BUTN5 High

      - DISCIN_GND_OPEN_13 High

      - DISCIN_GND_OPEN_14 High

      - DISCIN_GND_OPEN_15 High

      - DISCIN_GND_OPEN_16 High

      - DISCIN_GND_OPEN_17 High

      - DISCIN_GND_OPEN_18 High

      - DISCIN_GND_OPEN_19 High

      - DISCIN_GND_OPEN_20 High

      - DISCIN_GND_OPEN_21 High

      - DISCIN_GND_OPEN_22 High

      - DISCIN_GND_OPEN_23 High

Input(s) and Stimulus Apply 14 (±0.2) VDC to the pin Apply 14 (±0.2) VDC to the pin Apply 14 (±0.2) VDC to the pin Apply 14 (±0.2) VDC to the pin Apply 14 (±0.2) VDC to the pin Apply 14 (±0.2) VDC to the pin Apply 14 (±0.2) VDC to the pin Apply 14 (±0.2) VDC to the pin Apply 14 (±0.2) VDC to the pin Apply 14 (±0.2) VDC to the pin Apply 9.5 (±0.2) VDC to the pin Apply 9.5 (±0.2) VDC to the pin Apply 9.5 (±0.2) VDC to the pin Apply 9.5 (±0.2) VDC to the pin Apply 9.5 (±0.2) VDC to the pin Apply 9.5 (±0.2) VDC to the pin Apply 9.5 (±0.2) VDC to the pin Apply 9.5 (±0.2) VDC to the pin

Apply 10.5 (±0.2) VDC to the pin Apply 10.5 (±0.2) VDC to the pin Apply 10.5 (±0.2) VDC to the pin Apply 10.5 (±0.2) VDC to the pin Apply 10.5 (±0.2) VDC to the pin Apply 10.5 (±0.2) VDC to the pin Apply 10.5 (±0.2) VDC to the pin Apply 6.5 (±0.2) VDC to the pin Apply 6.5 (±0.2) VDC to the pin Apply 6.5 (±0.2) VDC to the pin Apply 6.5 (±0.2) VDC to the pin Apply 6.5 (±0.2) VDC to the pin Apply 6.5 (±0.2) VDC to the pin Apply 6.5 (±0.2) VDC to the pin Apply 6.5 (±0.2) VDC to the pin Apply 6.5 (±0.2) VDC to the pin Apply 6.5 (±0.2) VDC to the pin Apply 6.5 (±0.2) VDC to the pin Apply 6.5 (±0.2) VDC to the pin Apply 6.5 (±0.2) VDC to the pin Apply 6.5 (±0.2) VDC to the pin Apply 6.5 (±0.2) VDC to the pin Apply 6.5 (±0.2) VDC to the pin Apply 6.5 (±0.2) VDC to the pin Apply 6.5 (±0.2) VDC to the pin Apply 6.5 (±0.2) VDC to the pin Apply 6.5 (±0.2) VDC to the pin Apply 6.5 (±0.2) VDC to the pin Apply 6.5 (±0.2) VDC to the pin Apply 6.5 (±0.2) VDC to the pin Apply 6.5 (±0.2) VDC to the pin Apply 6.5 (±0.2) VDC to the pin Apply 6.5 (±0.2) VDC to the pin

Pass Criteria Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high Discrete reads high

Applicable

STATE 3 - ENGINEERING RELEASE 2026-05-06

ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL ALL

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

### Discrete In V/O

### Discrete In 28V/O Current Sourcing

Purpose

Verify inputs have no resistor defects and is also likely to detect shorts to other IO types.

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

Safety Related: No

Product State or Condition

Apply power per section 3.4

Input(s) and Stimulus

Apply 5k Ohm pullup to 12VDC.

| Test ID/Test Name | Measurement Description | LSL | USL | UNITS | Applicable |
| --- | --- | --- | --- | --- | --- |
| 5.4.1.1 DISCIN_28V_OPEN_0 Current Sourcing | Verify voltage at the pin | 10.3 | 10.7 | V | ALL |
| 5.4.1.2 DISCIN_28V_OPEN_1 Current Sourcing | Verify voltage at the pin | 10.3 | 10.7 | V | ALL |
| 5.4.1.3 DISCIN_28V_OPEN_2 Current Sourcing | Verify voltage at the pin | 10.3 | 10.7 | V | ALL |
| 5.4.1.4 DISCIN_28V_OPEN_3 Current Sourcing | Verify voltage at the pin | 10.3 | 10.7 | V | ALL |
| 5.4.1.5 DISCIN_28V_OPEN_4 Current Sourcing | Verify voltage at the pin | 10.3 | 10.7 | V | ALL |
| 5.4.1.6 DISCIN_28V_OPEN_5 Current Sourcing | Verify voltage at the pin | 10.3 | 10.7 | V | ALL |

### Discrete In 28V/O Low Test

Purpose

Verify inputs have no defects on the input path.

Safety Related: No

Product State or Condition

Apply power per section 3.4

Input(s) and Stimulus

Apply 10V (+0.1V/-0V) per each input

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 5.4.2.1 DISCIN_28V_OPEN_0 Low | Verify a low is received | Discrete reads low | ALL |
| 5.4.2.2 DISCIN_28V_OPEN_1 Low | Verify a low is received | Discrete reads low | ALL |
| 5.4.2.3 DISCIN_28V_OPEN_2 Low | Verify a low is received | Discrete reads low | ALL |
| 5.4.2.4 DISCIN_28V_OPEN_3 Low | Verify a low is received | Discrete reads low | ALL |
| 5.4.2.5 DISCIN_28V_OPEN_4 Low | Verify a low is received | Discrete reads low | ALL |
| 5.4.2.6 DISCIN_28V_OPEN_5 Low | Verify a low is received | Discrete reads low | ALL |

### Discrete In 28V/O High Test

Purpose

Verify inputs have no defects on the input path.

Safety Related: No

Product State or Condition

Apply power per section 3.4

Input(s) and Stimulus

Apply 14V (+0V/-0.1V) per each input

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 5.4.3.1 DISCIN_28V_OPEN_0 High | Verify a high is received | Discrete reads high | ALL |
| 5.4.3.2 DISCIN_28V_OPEN_1 High | Verify a high is received | Discrete reads high | ALL |
| 5.4.3.3 DISCIN_28V_OPEN_2 High | Verify a high is received | Discrete reads high | ALL |
| 5.4.3.4 DISCIN_28V_OPEN_3 High | Verify a high is received | Discrete reads high | ALL |
| 5.4.3.5 DISCIN_28V_OPEN_4 High | Verify a high is received | Discrete reads high | ALL |
| 5.4.3.6 DISCIN_28V_OPEN_5 High | Verify a high is received | Discrete reads high | ALL |

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

### Discrete In 5V/G Current Sourcing

Purpose

Verify inputs have no resistor defects and is also likely to detect shorts to other IO types.

Safety Related: No

Product State or Condition

Apply power per section 3.4

Input(s) and Stimulus

Apply 5k Ohm pullup to 12VDC.

| Test ID/Test Name | Measurement Description | LSL | USL | UNITS | Applicable |
| --- | --- | --- | --- | --- | --- |
| 5.4.4.1 RAD_ALT_VALID Current Sourcing | Verify voltage at the pin | 9.6 | 10.0 | V | ALL |

### Discrete In 5V/G Low Test

Purpose

Verify inputs have no defects on the input path.

Safety Related: No

Product State or Condition

Apply power per section 3.4

Input(s) and Stimulus

Apply 1V (+0.1V/-0V) per each input

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 5.4.5.1 RAD_ALT_VALID Low | Verify a low is received | Input reads low | ALL |

### Discrete In 5V/G High Test

Purpose

Verify inputs have no defects on the input path.

Safety Related: No

Product State or Condition

Apply power per section 3.4

Input(s) and Stimulus

Apply 3V (+0V/-0.1V) per each input

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 5.4.6.1 RAD_ALT_VALID High | Verify a high is received | Input reads High | ALL |

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

Discrete Out

### Discrete Out Open Test

Purpose

Verify the discrete outputs have no defects along the signal path or with current sourcing(resistive).

Safety Related: No

Product State or Condition

Apply power per section 3.4

Input(s) and Stimulus

One at a time, for each pin, individually apply a pullup to +28VDC

Command only the specific UUT discrete output to open state and verify current output

| Test ID/Test Name | Measurement Description | LSL | USL | UNITS | Applicable |
| --- | --- | --- | --- | --- | --- |
| 5.5.1.1 DISCOUT_GND_OPEN_0 Open | Verify current at the pin | 0 | 1.5 | mA | ALL |
| 5.5.1.2 DISCOUT_GND_OPEN_1 Open | Verify current at the pin | 0 | 1.5 | mA | ALL |
| 5.5.1.3 DISCOUT_GND_OPEN_2 Open | Verify current at the pin | 0 | 1.5 | mA | ALL |
| 5.5.1.4 DISCOUT_GND_OPEN_3 Open | Verify current at the pin | 0 | 1.5 | mA | ALL |
| 5.5.1.5 DISCOUT_GND_OPEN_4 Open | Verify current at the pin | 0 | 1.5 | mA | ALL |
| 5.5.1.6 DISCOUT_GND_OPEN_5 Open | Verify current at the pin | 0 | 1.5 | mA | ALL |
| 5.5.1.7 DISCOUT_GND_OPEN_6 Open | Verify current at the pin | 0 | 1.5 | mA | ALL |
| 5.5.1.8 DISCOUT_GND_OPEN_7 Open | Verify current at the pin | 0 | 1.5 | mA | ALL |

### Discrete Out Active Test

Purpose

Verify the discrete outputs have no defects along the signal path or with current sourcing(resistive).

Safety Related: No

Product State or Condition

Apply power per section 3.4

Input(s) and Stimulus

Test each pin one at a time by applying a 187 (±1%) Ohm pullup to +28.6 (±1%) VDC Command the specific UUT discrete output to active state, all others in the opposite state

| Test ID/Test Name | Measurement Description | LSL | USL | UNITS | Applicable |
| --- | --- | --- | --- | --- | --- |
| 5.5.2.1 DISCOUT_GND_OPEN_0 Active | Verify voltage at the pin | 0 | 1 | V | ALL |
| 5.5.2.2 DISCOUT_GND_OPEN_1 Active | Verify voltage at the pin | 0 | 1 | V | ALL |
| 5.5.2.3 DISCOUT_GND_OPEN_2 Active | Verify voltage at the pin | 0 | 1 | V | ALL |
| 5.5.2.4 DISCOUT_GND_OPEN_3 Active | Verify voltage at the pin | 0 | 1 | V | ALL |
| 5.5.2.5 DISCOUT_GND_OPEN_4 Active | Verify voltage at the pin | 0 | 1 | V | ALL |
| 5.5.2.6 DISCOUT_GND_OPEN_5 Active | Verify voltage at the pin | 0 | 1 | V | ALL |
| 5.5.2.7 DISCOUT_GND_OPEN_6 Active | Verify voltage at the pin | 0 | 1 | V | ALL |
| 5.5.2.8 DISCOUT_GND_OPEN_7 Active | Verify voltage at the pin | 0 | 1 | V | ALL |

## ARINC 429 TX

### ARINC 429 TX Characteristics Test

Purpose

Verify the outputs have no defects along the signal path or in the transceiver circuit.

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

Safety Related: No

Product State or Condition

Apply power per section 3.4

Input(s) and Stimulus

Per each A429 TX output tested apply a load

-	load: 600 Ohm resistor and 0.0015μF capacitor in parallel across the TX pins. (it is understood station tolerances may affect these values up to 10%)

Command the UUT to output A429 data at high. (and/or low speed)

| Test ID/Test Name | Measurement Description | LSL | USL | UNITS | Applicable |
| --- | --- | --- | --- | --- | --- |
| 5.6.1.1 A429_OUT0 Low Speed Rise Time | Verify the pulse rise time (low speed) | 5.0 | 15.0 | uS | ALL |
| 5.6.1.2 A429_OUT0 High Speed Rise Time | Verify the pulse rise time (high speed) | 1.0 | 2.0 | uS | ALL |
| 5.6.1.3 A429_OUT0 High Speed Fall Time | Verify the pulse fall time (high speed) | 1.0 | 2.0 | uS | ALL |
| 5.6.1.4 A429_OUT0 High Speed Pulse Width Time | Verify the pulse width (high speed) | 4.75 | 5.25 | uS | ALL |
| 5.6.1.5 A429_OUT0 High Voltage | Verify the high voltage | +7.25 | +11.0 | VDC | ALL |
| 5.6.1.6 A429_OUT0 Null Voltage | Verify the null voltage | -0.5 | +0.5 | VDC | ALL |
| 5.6.1.7 A429_OUT0 Low Voltage | Verify the low voltage | -11.0 | -7.25 | VDC | ALL |
| 5.6.1.8 A429_OUT0 High Speed Bit Rate | Verify the bit rate (high speed) | 99 | 101 | kbps | ALL |
| 5.6.1.9 A429_OUT1 Low Speed Rise Time | Verify the pulse rise time (low speed) | 5.0 | 15.0 | uS | ALL |
| 5.6.1.10 A429_OUT1 High Speed Rise Time | Verify the pulse rise time (high speed) | 1.0 | 2.0 | uS | ALL |
| 5.6.1.11 A429_OUT1 High Speed Fall Time | Verify the pulse fall time (high speed) | 1.0 | 2.0 | uS | ALL |
| 5.6.1.12 A429_OUT1 High Speed Pulse Width Time | Verify the pulse width (high speed) | 4.75 | 5.25 | uS | ALL |
| 5.6.1.13 A429_OUT1 High Voltage | Verify the high voltage | +7.25 | +11.0 | VDC | ALL |
| 5.6.1.14 A429_OUT1 Null Voltage | Verify the null voltage | -0.5 | +0.5 | VDC | ALL |
| 5.6.1.15 A429_OUT1 Low Voltage | Verify the low voltage | -11.0 | -7.25 | VDC | ALL |
| 5.6.1.16 A429_OUT1 High Speed Bit Rate | Verify the bit rate (high speed) | 99 | 101 | kbps | ALL |
| 5.6.1.17 A429_OUT2 Low Speed Rise Time | Verify the pulse rise time (low speed) | 5.0 | 15.0 | uS | ALL |
| 5.6.1.18 A429_OUT2 High Speed Rise Time | Verify the pulse rise time (high speed) | 1.0 | 2.0 | uS | ALL |
| 5.6.1.19 A429_OUT2 High Speed Fall Time | Verify the pulse fall time (high speed) | 1.0 | 2.0 | uS | ALL |
| 5.6.1.20 A429_OUT2 High Speed Pulse Width Time | Verify the pulse width (high speed) | 4.75 | 5.25 | uS | ALL |
| 5.6.1.21 A429_OUT2 High Voltage | Verify the high voltage | +7.25 | +11.0 | VDC | ALL |
| 5.6.1.22 A429_OUT2 Null Voltage | Verify the null voltage | -0.5 | +0.5 | VDC | ALL |
| 5.6.1.23 A429_OUT2 Low Voltage | Verify the low voltage | -11.0 | -7.25 | VDC | ALL |
| 5.6.1.24 A429_OUT2 High Speed Bit Rate | Verify the bit rate (high speed) | 99 | 101 | kbps | ALL |
| 5.6.1.25 A429_OUT3 Low Speed Rise Time | Verify the pulse rise time (low speed) | 5.0 | 15.0 | uS | ALL |
| 5.6.1.26 A429_OUT3 High Speed Rise Time | Verify the pulse rise time (high speed) | 1.0 | 2.0 | uS | ALL |
| 5.6.1.27 A429_OUT3 High Speed Fall Time | Verify the pulse fall time (high speed) | 1.0 | 2.0 | uS | ALL |
| 5.6.1.28 A429_OUT3 High Speed Pulse Width Time | Verify the pulse width (high speed) | 4.75 | 5.25 | uS | ALL |
| 5.6.1.29 A429_OUT3 High Voltage | Verify the high voltage | +7.25 | +11.0 | VDC | ALL |
| 5.6.1.30 A429_OUT3 Null Voltage | Verify the null voltage | -0.5 | +0.5 | VDC | ALL |

| Test ID/Test Name | Measurement Description | LSL | USL | UNITS | Applicable |
| --- | --- | --- | --- | --- | --- |
| 5.6.1.31 A429_OUT3 Low Voltage | Verify the low voltage | -11.0 | -7.25 | VDC | ALL |
| 5.6.1.32 A429_OUT3 High Speed Bit Rate | Verify the bit rate (high speed) | 99 | 101 | kbps | ALL |
| 5.6.1.33 A429_OUT4 Low Speed Rise Time | Verify the pulse rise time (low speed) | 5.0 | 15.0 | uS | ALL |
| 5.6.1.34 A429_OUT4 High Speed Rise Time | Verify the pulse rise time (high speed) | 1.0 | 2.0 | uS | ALL |
| 5.6.1.35 A429_OUT4 High Speed Fall Time | Verify the pulse fall time (high speed) | 1.0 | 2.0 | uS | ALL |
| 5.6.1.36 A429_OUT4 High Speed Pulse Width Time | Verify the pulse width (high speed) | 4.75 | 5.25 | uS | ALL |
| 5.6.1.37 A429_OUT4 High Voltage | Verify the high voltage | +7.25 | +11.0 | VDC | ALL |
| 5.6.1.38 A429_OUT4 Null Voltage | Verify the null voltage | -0.5 | +0.5 | VDC | ALL |
| 5.6.1.39 A429_OUT4 Low Voltage | Verify the low voltage | -11.0 | -7.25 | VDC | ALL |
| 5.6.1.40 A429_OUT4 High Speed Bit Rate | Verify the bit rate (high speed) | 99 | 101 | kbps | ALL |
| 5.6.1.41 A429_OUT5 Low Speed Rise Time | Verify the pulse rise time (low speed) | 5.0 | 15.0 | uS | ALL |
| 5.6.1.42 A429_OUT5 High Speed Rise Time | Verify the pulse rise time (high speed) | 1.0 | 2.0 | uS | ALL |
| 5.6.1.43 A429_OUT5 High Speed Fall Time | Verify the pulse fall time (high speed) | 1.0 | 2.0 | uS | ALL |
| 5.6.1.44 A429_OUT5 High Speed Pulse Width Time | Verify the pulse width (high speed) | 4.75 | 5.25 | uS | ALL |
| 5.6.1.45 A429_OUT5 High Voltage | Verify the high voltage | +7.25 | +11.0 | VDC | ALL |
| 5.6.1.46 A429_OUT5 Null Voltage | Verify the null voltage | -0.5 | +0.5 | VDC | ALL |
| 5.6.1.47 A429_OUT5 Low Voltage | Verify the low voltage | -11.0 | -7.25 | VDC | ALL |
| 5.6.1.48 A429_OUT5 High Speed Bit Rate | Verify the bit rate (high speed) | 99 | 101 | kbps | ALL |
| 5.6.1.49 A429_OUT6 Low Speed Rise Time | Verify the pulse rise time (low speed) | 5.0 | 15.0 | uS | ALL |
| 5.6.1.50 A429_OUT6 High Speed Rise Time | Verify the pulse rise time (high speed) | 1.0 | 2.0 | uS | ALL |
| 5.6.1.51 A429_OUT6 High Speed Fall Time | Verify the pulse fall time (high speed) | 1.0 | 2.0 | uS | ALL |
| 5.6.1.52 A429_OUT6 High Speed Pulse Width Time | Verify the pulse width (high speed) | 4.75 | 5.25 | uS | ALL |
| 5.6.1.53 A429_OUT6 High Voltage | Verify the high voltage | +7.25 | +11.0 | VDC | ALL |
| 5.6.1.54 A429_OUT6 Null Voltage | Verify the null voltage | -0.5 | +0.5 | VDC | ALL |
| 5.6.1.55 A429_OUT6 Low Voltage | Verify the low voltage | -11.0 | -7.25 | VDC | ALL |
| 5.6.1.56 A429_OUT7 High Speed Bit Rate | Verify the bit rate (high speed) | 99 | 101 | kbps | ALL |
| 5.6.1.57 A429_OUT7 Low Speed Rise Time | Verify the pulse rise time (low speed) | 5.0 | 15.0 | uS | ALL |
| 5.6.1.58 A429_OUT7 High Speed Rise Time | Verify the pulse rise time (high speed) | 1.0 | 2.0 | uS | ALL |
| 5.6.1.59 A429_OUT7 High Speed Fall Time | Verify the pulse fall time (high speed) | 1.0 | 2.0 | uS | ALL |
| 5.6.1.60 A429_OUT7 High Speed Pulse Width Time | Verify the pulse width (high speed) | 4.75 | 5.25 | uS | ALL |
| 5.6.1.61 A429_OUT7 High Voltage | Verify the high voltage | +7.25 | +11.0 | VDC | ALL |
| 5.6.1.62 A429_OUT7 Null Voltage | Verify the null voltage | -0.5 | +0.5 | VDC | ALL |
| 5.6.1.63 A429_OUT7 Low Voltage | Verify the low voltage | -11.0 | -7.25 | VDC | ALL |

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

## ARINC 429 RX

### ARINC 429 RX Worse Case Input

Purpose

Verify the inputs have no defects affecting operation as well as detecting if a single leg is shorted/open due to A429 normally working under these circumstances.

Safety Related: No

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

Product State or Condition

Apply power per section 3.4

Input(s) and Stimulus

Per each A429 RX port

Apply known high-speed A429 data to the receiver port using the voltages shown below.

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 5.7.1.1 A429_IN0 Data | Verify the UUT received the expected data | Data received correctly | ALL |
| 5.7.1.2 A429_IN1 Data | Verify the UUT received the expected data | Data received correctly | ALL |
| 5.7.1.3 A429_IN2 Data | Verify the UUT received the expected data | Data received correctly | ALL |
| 5.7.1.4 A429_IN3 Data | Verify the UUT received the expected data | Data received correctly | ALL |
| 5.7.1.5 A429_IN4 Data | Verify the UUT received the expected data | Data received correctly | ALL |
| 5.7.1.6 A429_IN5 Data | Verify the UUT received the expected data | Data received correctly | ALL |
| 5.7.1.7 A429_IN6 Data | Verify the UUT received the expected data | Data received correctly | ALL |
| 5.7.1.8 A429_IN7 Data | Verify the UUT received the expected data | Data received correctly | ALL |
| 5.7.1.9 A429_IN8 Data | Verify the UUT received the expected data | Data received correctly | ALL |
| 5.7.1.10 A429_IN9 Data | Verify the UUT received the expected data | Data received correctly | ALL |
| 5.7.1.11 A429_IN10 Data | Verify the UUT received the expected data | Data received correctly | ALL |
| 5.7.1.12 A429_IN11 Data | Verify the UUT received the expected data | Data received correctly | ALL |
| 5.7.1.13 A429_IN12 Data | Verify the UUT received the expected data | Data received correctly | ALL |
| 5.7.1.14 A429_IN13 Data | Verify the UUT received the expected data | Data received correctly | ALL |
| 5.7.1.15 A429_IN14 Data | Verify the UUT received the expected data | Data received correctly | ALL |
| 5.7.1.16 A429_IN15 Data | Verify the UUT received the expected data | Data received correctly | ALL |

Note: Input Port 0 is configured to receive 429 data

## ARINC708

### ARINC708 Input

Purpose

Verify the inputs have no defects on the ARINC708 (WXR) input between Pins “X_P” and “X_N”

Safety Related: No

Product State or Condition

Apply power per section 3.4

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

Input(s) and Stimulus

Input a known radial pattern with a 75 Ohm resistor between pins

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 5.8.1.1 WXR_0 Data | Verify the MerVE input CRC (and the other input CRC is blank) | CRC is correct | ALL |
| 5.8.1.2 WXR_1 Data | Verify the MerVE input CRC (and the other input CRC is blank) | CRC is correct | ALL |

Note: Use a unique radial pattern for each input.

### ARINC708 Input termination

Purpose

To verify the ARINC 708 input terminations were installed correctly.

Safety Related: No

Product State or Condition

Apply power per section 3.4

Input(s) and Stimulus

None

| Test ID/Test Name | Measurement Description | LSL | USL | UNITS | Applicable |
| --- | --- | --- | --- | --- | --- |
| 5.8.2.1 WXR_0_Term_N Termination Test | Verify the resistance across input pin “0_P” and termination pin | 66 | 73 | Ohm | ALL |
| 5.8.2.2 WXR_1_Term_N Termination Test | Verify the resistance across input pin “1_P” and termination pin | 66 | 73 | Ohm | ALL |

### Ethernet

### Ethernet Data

Purpose

The purpose of this test is to verify Ethernet ports are defect free. The chosen method sends an amount of data and verifies no dropped packets, which is intended to use signal integrity to additionally test uncommon issues with magnetics.

Safety Related: Yes

Product State or Condition

Apply power per section 3.4

Input(s) and Stimulus

For each Ethernet port:

For every 62,850 bytes of data, verify 1 or less packets were lost.

- The operation shall be set to and verified running at maximum negotiation speed (to increase fault detection)

- If you send 125,700 bytes the fail limit would be two packets and so on.

- Must be done in both directions (if an echo test is done, it is considered to test both directions)

- Must be a minimum of 20 packets (reduces the probability of masking a failure, such as two byte drops in one packet to less than 5%)

- The data shall be pseudo random data (to increase fault detection)

- Intended for use with UDP protocol.

- Actual expectation is 1 bit or less lost every 502,800 bits, which allows for acceptable packet loss of UUT and/or test station.

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 5.9.1.1 ETH_1 Data | Verify 1 or less packet loss per above criteria | 1 or less packets lost | ALL |
| 5.9.1.2 ETH_1 Link Speed | Verify port is reporting a connection at maximum link speed (1GbE) | Correct link speed reported | ALL |
| 5.9.1.3 ETH_2 Data | Verify 1 or less packet loss per above criteria | 1 or less packets lost | ALL |
| 5.9.1.4 ETH_2 Link Speed | Verify port is reporting a connection at maximum link speed (1GbE) | Correct link speed reported | ALL |

## MIL-STD-1553

### 1553 Data and Voltage

Purpose

Verify the signal paths have no defects.

Safety Related: No

Product State or Condition

Apply power per section 3.4

Input(s) and Stimulus

Per each 1553 port

Communication Setup

20ft max

Coupling Transformer 1:1.4

UUT

58.5

58.5

58.5

58.5

Coupling Transformer 1.4:1

Transformer Coupled Test Equipment

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

Resistors should have +/-2% tolerance or less Cable impedance of 78 ohms

Voltage Measurement Setup

0

UUT

Measurement Point

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 5.10.1.1 1553_A Data | Send and receive known data Verify expected data was sent and received | Known data sent and received correctly | ALL |
| 5.10.1.2 1553_B Data | Send and receive known data Verify expected data was sent and received | Known data sent and received correctly | ALL |

| Test ID/Test Name | Measurement Description | LSL | USL | UNITS | Applicable |
| --- | --- | --- | --- | --- | --- |
| 5.10.1.3 1553_A Voltage | Verify the transmission voltage level while loaded (at the UUT transmitting pins) | 20.0 | 27.0 | Volts | ALL |

| Test ID/Test Name | Measurement Description | LSL | USL | UNITS | Applicable |
| --- | --- | --- | --- | --- | --- |
| 5.10.1.4 1553_B Voltage | Verify the transmission voltage level while loaded (at the UUT transmitting pins) | 20.0 | 27.0 | Volts | ALL |

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

### Video

### A818 Fiber Video In Data

Purpose

To verify there are no defects in the input path such as damaged, dirty or partially connected fiber optics.

Safety Related: Yes

Product State or Condition

Apply power per section 3.4

Input(s) and Stimulus

Send known data/image in with a signal level of -12dBm ±1dBm.

(The test must be implemented in a way to detect swapped A818 cables. This may be achieved by each port using unique data or testing each port one at a time)

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 5.11.1.1 A818_1_RX Data | Verify known image was received | Image received correctly | ALL |
| 5.11.1.2 A818_2_RX Data | Verify known image was received | Image received correctly | ALL |
| 5.11.1.3 A818_1 Video In No Faults | Verify with supplied images, no missing image fault | No missing video faults declared | ALL |
| 5.11.1.4 A818_2 Video In No Faults | Verify with supplied images, no missing image fault | No missing video faults declared | ALL |

* Verifying image received could be done with ‘onscreen CRC’ or ‘visible pixel CRC’ checks

internal to the UUT, by passing the image out another port or by other means.

### A818 Fiber Video Out Power and Data

Purpose

To verify there are no defects in the output path such as damaged, dirty or partially connected fiber optics.

Safety Related: Yes

Product State or Condition

Apply power per section 3.4

Input(s) and Stimulus

Send known data/image out per each port

(The test must be implemented in a way to detect swapped A818 cables. This may be achieved by each port using unique data or testing each port one at a time)

| Test ID/Test Name | Measurement Description | LSL | USL | UNITS | Applicable |
| --- | --- | --- | --- | --- | --- |
| 5.11.2.1 A818_1_TX On | Verify the power output | -8.5 | 0 | dBm | ALL |
| 5.11.2.2 A818_2_TX On | Verify the power output | -8.5 | 0 | dBm | ALL |
| 5.11.2.3 A818_1_TX Off | Disable the output (ODL) Verify the power output | N/A | -15 | dBm | ALL |
| 5.11.2.4 A818_2_TX Off | Disable the output (ODL) Verify the power output | N/A | -15 | dBm | ALL |

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 5.11.2.5 A818_1_TX Data | Verify correct data or image was output | Correct image transmitted | ALL |
| 5.11.2.6 A818_2_TX Data | Verify correct data or image was output | Correct image transmitted | ALL |

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

* Verifying image output could be done with ‘onscreen CRC’ or ‘visible pixel CRC’ checks, visually or by other means. Additionally, the ability to detect internally swapped cables must be considered based on the implementation.

### SMPTE Video In

Purpose

To verify there are no defects on the input path.

Safety Related: Yes

Product State or Condition

Apply power per section 3.4

Input(s) and Stimulus

Send known data/image in each port at 1080p video resolution.

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 5.11.3.1 SMPTE_IN0 Data | Verify the CRC matches or visually verify the image on a monitor. | CRC or image is correct | ALL |
| 5.11.3.2 SMPTE_IN1 Data | Verify the CRC matches or visually verify the image on a monitor. | CRC or image is correct | ALL |
| 5.11.3.3 SMPTE_IN0 No Faults | Verify with supplied images, no missing image fault | No missing video faults declared | ALL |
| 5.11.3.4 SMPTE_IN1 No Faults | Verify with supplied images, no missing image fault | No missing video faults declared | ALL |

Note: Image can be passed through to another output to be viewed or CRC checked, if not done internally.

### Missing Video Input Fault

Purpose

To verify there are no defects with the missing video input discrete path.

Safety Related: Yes

Product State or Condition

Apply power per section 3.4

Input(s) and Stimulus

Select the input listed in the table without providing an external video input.

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 5.11.4.1 A818_1_RX Missing Video In | Verify the UUT generates a missing video input fault. | Missing image fault received | ALL |
| 5.11.4.2 A818_2_RX Missing Video In | Verify the UUT generates a missing video input fault. | Missing image fault received | ALL |
| 5.11.4.3 SMPTE_IN0 Missing Video In | Verify the UUT generates a missing video input fault. | Missing image fault received | ALL |
| 5.11.4.4 SMPTE_IN1 Missing Video In | Verify the UUT generates a missing video input fault. | Missing image fault received | ALL |

Note: Image can be passed through to another output to be viewed or CRC checked, if not done internally.

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

Analog Video In – STANAG 3350

Purpose

To verify there are no defects in the analog video input path.

Safety Related: Yes

Product State or Condition

Apply power per section 3.4

Input(s) and Stimulus

Send a color bar image into the [type such as STANAG 3350] analog video input.

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 5.11.5.1 AVIN_0 Analog Video In | Verify image for correctness | Correct image displayed | ALL |
| 5.11.5.2 AVIN_1 Analog Video In | Verify image for correctness | Correct image displayed | ALL |
| 5.11.5.3 AVIN_3 Analog Video In | Verify image for correctness | Correct image displayed | ALL |

- Correctness is at the test operators’ discretion. Issues with colors or distortion should be the primary criteria. In the case of uncertainty, the manufacturing engineer or similar role should be contacted.

- The signal may be passed through the UUT to a video output to then display on a monitor.

- The source will provide the termination resistor or will be connected to the term pin

### Analog Video In – NTSC/PAL

Purpose

To verify there are no defects in the analog composite video input path.

Safety Related: Yes

Product State or Condition

Apply power per section 3.4

Input(s) and Stimulus

Send a color bar image into the analog video input using composite format (NTSC/PAL).

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 5.11.6.1 AVIN_0_B Image Test | Verify image for correctness | Image is correct | ALL |
| 5.11.6.2 AVIN_0_G Image Test | Verify image for correctness | Image is correct | ALL |
| 5.11.6.3 AVIN_0_R Image Test | Verify image for correctness | Image is correct | ALL |
| 5.11.6.4 AVIN_1_B Image Test | Verify image for correctness | Image is correct | ALL |
| 5.11.6.5 AVIN_1_G Image Test | Verify image for correctness | Image is correct | ALL |
| 5.11.6.6 AVIN_1_R Image Test | Verify image for correctness | Image is correct | ALL |
| 5.11.6.7 AVIN_3_B Image Test | Verify image for correctness | Image is correct | ALL |
| 5.11.6.8 AVIN_3_G Image Test | Verify image for correctness | Image is correct | ALL |
| 5.11.6.9 AVIN_3_R Image Test | Verify image for correctness | Image is correct | ALL |

- Correctness is at the test operators’ discretion. Issues with colors or distortion should be the primary criteria. In the case of uncertainty, the manufacturing engineer or similar role should be contacted.

- The signal may be passed through the UUT to a video output to then display on a monitor.

- The source will provide the termination resistor or will be connected to the term pin

### Analog Video Input termination

Purpose

To verify there are no defects between the analog video composite input pins.

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

Safety Related: No

Product State or Condition

Apply power per section 3.4

Input(s) and Stimulus

None

| Test ID/Test Name | Measurement Description | LSL | USL | UNITS | Applicable |
| --- | --- | --- | --- | --- | --- |
| 5.11.7.1 AVIN_0_B_N termination test | Verify the resistance across input pin and term pin | 71.25 | 78.75 | Ohm | ALL |
| 5.11.7.2 AVIN_0_G_N termination test | Verify the resistance across input pin and term pin | 71.25 | 78.75 | Ohm | ALL |
| 5.11.7.3 AVIN_0_R_N termination test | Verify the resistance across input pin and term pin | 71.25 | 78.75 | Ohm | ALL |
| 5.11.7.4 AVIN_1_B_N termination test | Verify the resistance across input pin and term pin | 71.25 | 78.75 | Ohm | ALL |
| 5.11.7.5 AVIN_1_G_N termination test | Verify the resistance across input pin and term pin | 71.25 | 78.75 | Ohm | ALL |
| 5.11.7.6 AVIN_1_R_N termination test | Verify the resistance across input pin and term pin | 71.25 | 78.75 | Ohm | ALL |
| 5.11.7.7 AVIN_3_B_N termination test | Verify the resistance across input pin and term pin | 71.25 | 78.75 | Ohm | ALL |
| 5.11.7.8 AVIN_3_G_N termination test | Verify the resistance across input pin and term pin | 71.25 | 78.75 | Ohm | ALL |
| 5.11.7.9 AVIN_3_R_N termination test | Verify the resistance across input pin and term pin | 71.25 | 78.75 | Ohm | ALL |

## RS422

### RS422 TX Voltage and Data

Purpose

To verify the output path is defect free.

Safety Related: No

Product State or Condition

Apply power per section 3.4

Input(s) and Stimulus

Apply a 120 ohm load across the output pins (or drive a 120 ohm receiver) Send known data out of the UUT at 115200 bit rate

| Test ID/Test Name | Measurement Description | LSL | USL | UNITS | Applicable |
| --- | --- | --- | --- | --- | --- |
| 5.12.1.1 RS422_485_TX1 Voltage | Verify voltage amplitude across TX pins | 2 | 6 | V | ALL |
| 5.12.1.2 RS422_485_TX2 Voltage | Verify voltage amplitude across TX pins | 2 | 6 | V | ALL |
| 5.12.1.3 RS422_485_TX3 Voltage | Verify voltage amplitude across TX pins | 2 | 6 | V | ALL |

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 5.12.1.4 RS422_485_TX1 Data | Verify expected data was transmitted | Correct data transmitted | ALL |
| 5.12.1.5 RS422_485_TX2 Data | Verify expected data was transmitted | Correct data transmitted | ALL |
| 5.12.1.6 RS422_485_TX3 Data | Verify expected data was transmitted | Correct data transmitted | ALL |

### RS422 RX Data

Purpose

To verify the input path is defect free.

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

Safety Related: No

Product State or Condition

Apply power per section 3.4

Input(s) and Stimulus

Send known data in each port at 115200 bit rate

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 5.12.2.1 RS422_485_RX1 Data | Verify expected data was received | Correct data received | ALL |
| 5.12.2.2 RS422_485_RX2 Data | Verify expected data was received | Correct data received | ALL |
| 5.12.2.3 RS422_485_RX3 Data | Verify expected data was received | Correct data received | ALL |

### RS422 RX Failsafe Resistor Check

Purpose

To verify the input failsafe resistor is installed correctly.

Safety Related: No

Product State or Condition

Apply power per section 3.4

Input(s) and Stimulus

120 Ohm resistor placed between high and low pins

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 5.12.3.1 RS422_485_RX1 Resistor | Verify voltage across input pins | Measured voltage of 0.4 VDC ± 0.1V | ALL |
| 5.12.3.2 RS422_485_RX2 Resistor | Verify voltage across input pins | Measured voltage of 0.4 VDC ± 0.1V | ALL |
| 5.12.3.3 RS422_485_RX3 Resistor | Verify voltage across input pins | Measured voltage of 0.4 VDC ± 0.1V | ALL |

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

Key Panel

### Key Panel Test

Purpose

To verify the keys and key circuit is defect free.

Safety Related: No

Product State or Condition

Apply power per section 3.4

Input(s) and Stimulus

Keypresses must be at least 25 ms in duration

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 5.13.1.1 Top Bezel Key – S1 | Verify pressing key results in the correct associated key detection and release | Key status is correct | ALL |
| 5.13.1.2 Top Bezel Key – S2 | Verify pressing key results in the correct associated key detection and release | Key status is correct | ALL |
| 5.13.1.3 Top Bezel Key – S3 | Verify pressing key results in the correct associated key detection and release | Key status is correct | ALL |
| 5.13.1.4 Top Bezel Key – S4 | Verify pressing key results in the correct associated key detection and release | Key status is correct | ALL |
| 5.13.1.5 Top Bezel Key – S5 | Verify pressing key results in the correct associated key detection and release | Key status is correct | ALL |
| 5.13.1.6 Top Bezel Key – S6 | Verify pressing key results in the correct associated key detection and release | Key status is correct | ALL |
| 5.13.1.7 Right Bezel Key – S9 | Verify pressing key results in the correct associated key detection and release | Key status is correct | ALL |
| 5.13.1.8 Right Bezel Key – S10 | Verify pressing key results in the correct associated key detection and release | Key status is correct | ALL |
| 5.13.1.9 Right Bezel Key – S11 | Verify pressing key results in the correct associated key detection and release | Key status is correct | ALL |
| 5.13.1.10 Right Bezel Key – S12 | Verify pressing key results in the correct associated key detection and release | Key status is correct | ALL |

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 5.13.1.11 Right Bezel Key – S13 | Verify pressing key results in the correct associated key detection and release | Key status is correct | ALL |
| 5.13.1.12 Right Bezel Key – S14 | Verify pressing key results in the correct associated key detection and release | Key status is correct | ALL |
| 5.13.1.13 Bottom Bezel Key – S17 | Verify pressing key results in the correct associated key detection and release | Key status is correct | ALL |
| 5.13.1.14 Bottom Bezel Key – S18 | Verify pressing key results in the correct associated key detection and release | Key status is correct | ALL |
| 5.13.1.15 Bottom Bezel Key – S19 | Verify pressing key results in the correct associated key detection and release | Key status is correct | ALL |
| 5.13.1.16 Bottom Bezel Key – S20 | Verify pressing key results in the correct associated key detection and release | Key status is correct | ALL |
| 5.13.1.17 Bottom Bezel Key – S21 | Verify pressing key results in the correct associated key detection and release | Key status is correct | ALL |
| 5.13.1.18 Bottom Bezel Key – S22 | Verify pressing key results in the correct associated key detection and release | Key status is correct | ALL |
| 5.13.1.19 Left Bezel Key – S25 | Verify pressing key results in the correct associated key detection and release | Key status is correct | ALL |
| 5.13.1.20 Left Bezel Key – S26 | Verify pressing key results in the correct associated key detection and release | Key status is correct | ALL |
| 5.13.1.21 Left Bezel Key – S27 | Verify pressing key results in the correct associated key detection and release | Key status is correct | ALL |
| 5.13.1.22 Left Bezel Key – S28 | Verify pressing key results in the correct associated key detection and release | Key status is correct | ALL |
| 5.13.1.23 Left Bezel Key – S29 | Verify pressing key results in the correct associated key detection and release | Key status is correct | ALL |
| 5.13.1.24 Left Bezel Key – S30 | Verify pressing key results in the correct associated key detection and release | Key status is correct | ALL |
| 5.13.1.25 Power Control On to Off | Verify toggling the power control turns the UUT off. | Unit responds correctly | ALL |
| 5.13.1.26 Power Control Off to On | Verify toggling the power control turns the UUT off. | Unit responds correctly | ALL |
| 5.13.1.27 Display Luminance Control | Verify pressing the rocker up and down visibly increases and decreases the display luminance | Unit responds correctly | ALL |
| 5.13.1.28 Video Brightness Control | Verify pressing of the up/down control results in the correct associated detection and release | Unit responds correctly | ALL |
| 5.13.1.29 Video Contrast Control | Verify pressing of the up/down control results in the correct associated detection and release | Unit responds correctly | ALL |

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

Note: This test may be accomplished by the test operator pressing the keys one at a time and seeing their associated detection on the UUT screen or by other means. This testing also needs to ensure that pressing one key doesn’t result in the detection of the wrong key or multiple keys.

### Key Panel Light Test

Purpose

To verify the lighting circuit is defect free.

Safety Related: No

Product State or Condition

Apply power per section 3.4

Input(s) and Stimulus

Max input power is 1 Watt at 5 VDC

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 5.13.2.1 Key Panel 5VDC Lighting Max | Apply 5VDC to 5VDC lighting input Verify the key panel and slip skid indicator have uniform lighting | Lighting is correct | ALL |
| 5.13.2.2 Key Panel 5VDC Reduced Lighting | Apply 5VDC to 5VDC lighting input Reduce the input voltage (enough to cause a noticeable decrease in light intensity) Verify the light intensity decreases. | Lighting is correct | ALL |
| 5.13.2.3 Key Panel Light Off Test | Remove all key panel voltage inputs. Verify the key panel light intensity is not visible. (at room ambient lighting) | Lighting is correct | ALL |

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

### Display

### Test Pattern

Purpose

The purpose is to verify the display and video circuit is defect free.

Safety Related: No

Product State or Condition

Apply power per section 3.4

Input(s) and Stimulus

Command UUT into Shop Mode

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 5.14.1.1 Multicolor Image Test | Display a known multicolor image. Verify image is as expected. | Image is displayed correctly | ALL |
| 5.14.1.2 Grayscale Image Test | Display a known grayscale image. Verify image is as expected. | Image is displayed correctly | ALL |

Note: Verification of image correctness is at the operators’ discretion. Characteristics such as:

- Missing lines/columns

- Missing colors

- Black, bright, or blinking can be considered failures on number and concentration (see inspection procedure from Table 3)

- Glass defects observed at 30 inches distance

In the case of uncertainty, the manufacturing engineer should be contacted.

### Light Leak

Purpose

The purpose is to verify there are no display defects that result in bright spots on the display.

Safety Related: No

Product State or Condition

Apply power per section 3.4

Input(s) and Stimulus

Set luminance to maximum value Set UUT to NVIS mode.

Set brightness to a level that is convenient for observation in a production environment.

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 5.14.2.1 Light Leak Test | Using NVIS Monocular and Class A filter installed in a dark room. Visually verify there are no intense bright spots in comparison with the filtered illuminated features. There should also be no excessive light spots around the button or display. | NVIS mode correct without light leaks | ALL |

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

### Luminance Test

Purpose

The purpose is to verify the luminance circuit is defect free.

Safety Related: No

Product State or Condition

Apply power per section 3.4

Input(s) and Stimulus

See table

| Test ID/Test Name |  | Measurement Description | LSL | USL | UNITS | Applicable |
| --- | --- | --- | --- | --- | --- | --- |
| 5.14.3.1 | Day Mode Maximum Brightness | Set UUT to max brightness in Day mode using a white screen Verify brightness level at the center of the display. | 250 | N/A | FL | ALL |
| 5.14.3.2 | Day Mode Minimum Brightness | Set UUT to min brightness in Day using a white screen. Verify brightness level at the center of the display. | 0 | 0.05 | FL | ALL |
| 5.14.3.3 | NVIS Mode Maximum Brightness | Set UUT to max brightness in NVIS mode using a white screen Verify brightness level at the center of the display. | 1.7 | 2.7 | FL | ALL |
| 5.14.3.4 | NVIS Mode Minimum Brightness | Set UUT to min brightness in NVIS using a white screen. Verify brightness level at the center of the display. | 0 | 0.05 | FL | ALL |

### DEP Chroma Tolerance

Purpose

This test verifies the UUT display Chroma Tolerance has not changed from initial calibration during testing.

Safety Related: No

Product State or Condition

Apply power per section 3.4

Input(s) and Stimulus

See table and use the following to calculate chromaticity

| Color Coordinates |  |  |
| --- | --- | --- |
| Color | u' | v' |
| Red | 0.412 | 0.531 |
| Green | 0.124 | 0.574 |
| Blue | 0.159 | 0.198 |
| White | 0.19 | 0.493 |

DEP Chroma Tolerance is defined as the variance from the specified nominal values measured normal to the display at the maximum luminance setting, measured in an ambient illumination of less than 0.5 fL. The equation for DEP Chroma Tolerance is:

###### 𝐷𝐸𝑃 𝐶ℎ𝑟𝑜𝑚𝑎 𝑇𝑜𝑙𝑒𝑟𝑎𝑛𝑐𝑒 = ∆𝐶∗	= 13𝐿∗  ∆𝑢′ 𝐷𝐸𝑃 2 + ∆𝑣′ 𝐷𝐸𝑃 2

𝐷𝐸𝑃

∆𝑢′ 𝐷𝐸𝑃 = Difference between specified u’ values and u’ measured at the display center

###### ∆𝑣′ 𝐷𝐸𝑃  = Difference between specified v’ values and v’ measured at the display center

𝐿∗

3

###### = 116

𝐿𝑚𝑒𝑎𝑠𝑢𝑟𝑒𝑑

𝑌𝑛

###### − 16 𝑓𝑜𝑟 𝐿𝑚𝑒𝑎𝑠𝑢𝑟𝑒𝑑 ≥ 1𝑓𝐿

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

𝐿∗ = 9.03 𝐿𝑚𝑒𝑎𝑠𝑢𝑟𝑒𝑑  𝑓𝑜𝑟 𝐿𝑚𝑒𝑎𝑠𝑢𝑟𝑒𝑑  < 1𝑓𝐿

###### Where Yn = Measured luminance of Display White reference at screen center taken from DEP

| Test ID/Test Name | Measurement Description | LSL | USL | UNITS | Applicable |
| --- | --- | --- | --- | --- | --- |
| 5.14.4.1 Day Mode DEP Chroma Tolerance | Set UUT to max brightness in Day mode using a white screen Measure Luminance and Chromaticity from the DEP at the center of the display. Calculate DEP Chroma value as defined. | 0 | 26 | N/A | ALL |
| 5.14.4.2 NVIS Mode DEP Chroma Tolerance | Set UUT to max brightness in NVIS mode using a white screen. Measure Luminance and Chromaticity from the DEP at the center of the display. Calculate DEP Chroma value as defined. | 0 | 26 | N/A | ALL |

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

Analog I/O Verification

Processing Output

Filter and

Diff Amp

Filter and

Diff Amp

Cursor 2

Filter and Diff Amp

Excitation Monitor

Cursor 1

-7.5 V

Regulator

Filter and

Diff Amp

+7.5 V

Regulator

Analog Switches and ADCs

Excitation Output

IO Card

Analog Type 1 Output Excitation +

Cursor Reference Return Output

Analog Type 1 Output Excitation -

Analog Type 5A Input

Cursor 1 Y

Analog Type 5A Input

Cursor 1 X

Analog Type 3 Input

Excitation + Monitor

Analog Type 3 Input

Excitation - Monitor

Analog Type 5B Input Cursor 2 Y

Analog Type 5B Input

Cursor 2 X

Analog Type 5B Input

Reference

Figure 1: MFD Analog Circuit Design

### Differential Analog Input Tests

Purpose

This test validates that differential analog inputs operate error free.

Safety Related: No

Product State or Condition

Apply power per section 3.4.2

| Test ID/Test Name | Stimulus | Measurement | LSL | USL | Units | Applicable |
| --- | --- | --- | --- | --- | --- | --- |
| 5.15.1.1 HOMER_SIG 0.2V Differential | 0.2V Differential | 0.2 | 0.16 | 0.24 | V | ALL |
| 5.15.1.2 HOMER_SIG -0.2V Differential | -0.2V Differential | -0.2 | -0.24 | -0.16 | V | ALL |
| 5.15.1.3 HOMER_STEER 0.2V Differential | 0.2V Differential | 0.2 | 0.16 | 0.24 | V | ALL |
| 5.15.1.4 HOMER_STEER -0.2 Differential | -0.2V Differential | -0.2 | -0.24 | -0.16 | V | ALL |
| 5.15.1.5 RAD_ALT Full Range 0V Differential | 0V Differential | 0 | -0.015 | 0.015 | V | ALL |

| Test ID/Test Name | Stimulus | Measurement | LSL | USL | Units | Applicable |
| --- | --- | --- | --- | --- | --- | --- |
| 5.15.1.6 RAD_ALT Limited Range 0V Differential | 0V Differential | 0 | -0.015 | 0.015 | V | ALL |
| 5.15.1.7 RAD_ALT Full Range - 0.7V Differential | -0.7V Differential | -0.7 | -0.718 | -0.682 | V | ALL |
| 5.15.1.8 RAD_ALT Limited Range -0.7V Differential | -0.7V Differential | -0.7 | -0.718 | -0.682 | V | ALL |
| 5.15.1.9 RAD_ALT Full Range -2V Differential | -2.0V Differential | -2.0 | -2.023 | -1.977 | V | ALL |
| 5.15.1.10 RAD_ALT Full Range -10V Differential | -10.0V Differential | -10.0 | -10.053 | -9.947 | V | ALL |
| 5.15.1.11 RAD_ALT Full Range -12V Differential | -12.0V Differential | -12.0 | -12.06 | -11.94 | V | ALL |
| 5.15.1.12 RAD_ALT Full Range -14V Differential | -14.0V Differential | -14.0 | -14.068 | -13.932 | V | ALL |
| 5.15.1.13 CURSOR_DRV_IN 13V Differential | 13.0V Differential | 13.0 | 12.85 | 13.15 | V | ALL |
| 5.15.1.14 CURSOR_X_2 6V Differential | 6.0V | 6.0 | 5.925 | 6.075 | V | ALL |
| 5.15.1.15 CURSOR_X_2 2V Differential | 2.0V | 2.0 | 1.925 | 2.075 | V | ALL |
| 5.15.1.16 CURSOR_X_2 -6V Differential | -6.0V | -6.0 | -6.075 | -5.925 | V | ALL |
| 5.15.1.17 CURSOR_X_2 -2V Differential | -2.0V | -2.0 | -2.075 | -1.925 | V | ALL |
| 5.15.1.18 CURSOR_Y_2 6V Differential | 6.0V | 6.0 | 5.925 | 6.075 | V | ALL |
| 5.15.1.19 CURSOR_Y_2 2V Differential | 2.0V | 2.0 | 1.925 | 2.075 | V | ALL |
| 5.15.1.20 CURSOR_Y_2 -6V Differential | -6.0V | -6.0 | -6.075 | -5.925 | V | ALL |
| 5.15.1.21 CURSOR_Y_2 -2V Differential | -2.0V | -2.0 | -2.075 | -1.925 | V | ALL |

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

### Single Ended Analog Input Tests

Purpose

This test validates that the single ended analog inputs operate error free.

Safety Related: No

Product State or Condition

Apply power per section 3.4.2

| Test ID/Test Name | Stimulus | Measurement | LSL | USL | Units | Applicable |
| --- | --- | --- | --- | --- | --- | --- |
| 5.15.2.1 CURSOR_X J1 Single Ended 7V | 7.0V | 7.0 | 6.925 | 7.075 | V | ALL |
| 5.15.2.2 CURSOR_X J1 Single Ended 2V | 2.0V | 2.0 | 1.925 | 2.075 | V | ALL |
| 5.15.2.3 CURSOR_X J1 Single Ended -7V | -7.0V | -7.0 | -7.075 | -6.925 | V | ALL |
| 5.15.2.4 CURSOR_X J1 Single Ended -2V | -2.0V | -2.0 | -2.075 | -1.925 | V | ALL |
| 5.15.2.5 CURSOR_Y J1 Single Ended 7V | 7.0V | 7.0 | 6.925 | 7.075 | V | ALL |
| 5.15.2.6 CURSOR_Y J1 Single Ended 2V | 2.0V | 2.0 | 1.925 | 2.075 | V | ALL |
| 5.15.2.7 CURSOR_Y J1 Single Ended -7V | -7.0V | -7.0 | -7.075 | -6.925 | V | ALL |
| 5.15.2.8 CURSOR_Y J1 Single Ended -2V | -2.0V | -2.0 | -2.075 | -1.925 | V | ALL |
| 5.15.2.9 CURSOR_X J3 Single Ended 7V | 7.0V | 7.0 | 6.925 | 7.075 | V | ALL |
| 5.15.2.10 CURSOR_X J3 Single Ended 2V | 2.0V | 2.0 | 1.925 | 2.075 | V | ALL |
| 5.15.2.11 CURSOR_X J3 Single Ended -7V | -7.0V | -7.0 | -7.075 | -6.925 | V | ALL |
| 5.15.2.12 CURSOR_X J3 Single Ended -2V | -2.0V | -2.0 | -2.075 | -1.925 | V | ALL |
| 5.15.2.13 CURSOR_Y J3 Single Ended 7V | 7.0V | 7.0 | 6.925 | 7.075 | V | ALL |
| 5.15.2.14 CURSOR_Y J3 Single Ended 2V | 2.0V | 2.0 | 1.925 | 2.075 | V | ALL |
| 5.15.2.15 CURSOR_Y J3 Single Ended -7V | -7.0V | -7.0 | -7.075 | -6.925 | V | ALL |
| 5.15.2.16 CURSOR_Y J3 Single Ended -2V | -2.0V | -2.0 | -2.075 | -1.925 | V | ALL |

### Analog Output Tests

Purpose

This test validates that the analog outputs operate error free.

Safety Related: No

Product State or Condition

Apply power per section 3.4.2

Input(s) and Stimulus

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

Connect the output pin to ground thru a 125 Ohm resistor

| Test ID/Test Name | Measurement Description | LSL | USL | UNITS | Applicable |
| --- | --- | --- | --- | --- | --- |
| 5.15.3.1 MSC_PWR_P7_5 J3-108 | Measure output pin voltage | 7.43 | 7.57 | V | ALL |
| 5.15.3.2 MSC_PWR_N7_5 J3-109 | Measure output pin voltage | -7.43 | -7.57 | V | ALL |
| 5.15.3.3 MSC_PWR_P7_5 J1-110 | Measure output pin voltage | 7.43 | 7.57 | V | ALL |
| 5.15.3.4 MSC_PWR_N7_5 J1-111 | Measure output pin voltage | -7.43 | -7.57 | V | ALL |

## RS232

Purpose

This section lists the RS232 debug ports for completeness, but are not specifically tests as they are used during the rest of the ATP.

| Port Name | Pin Name | Con | Pin |
| --- | --- | --- | --- |
| DEBUG_RS232_1 | RS232_EUT_RXA | J1 | 55 |
|  | DIGITAL GND | J1 | 54 |
|  | RS232_EUT_TXA | J1 | 65 |
|  | DIGITAL GND | J1 | 64 |
| DEBUG_RS232_2 | RS232_EUT_RXB | J2 | 44 |
|  | DIGITAL GND | J2 | 45 |
|  | RS232_EUT_TXB | J2 | 56 |
|  | DIGITAL GND | J1 | 57 |

### Temperature Monitor Function

Purpose

This test validates that UUT temperature monitors function correctly.

Safety Related: Yes

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 5.17.1.1 LCD Top Temp Ambient | Verify temperature reading at factory ambient for erroneously min/max values. | Temperature sensor reads between 0 DegC to 40 DegC | ALL |
| 5.17.1.2 LCD Center Ambient | Verify temperature reading at factory ambient for erroneously min/max values. | Temperature sensor reads between 0 DegC to 40 DegC | ALL |
| 5.17.1.3 LCD Bottom Ambient | Verify temperature reading at factory ambient for erroneously min/max values. | Temperature sensor reads between 0 DegC to 40 DegC | ALL |
| 5.17.1.4 LCD Board Ambient | Verify temperature reading at factory ambient for erroneously min/max values. | Temperature sensor reads between 0 DegC to 40 DegC | ALL |
| 5.17.1.5 Backlight Board Ambient | Verify temperature reading at factory ambient for erroneously min/max values. | Temperature sensor reads between 0 DegC to 40 DegC | ALL |
| 5.17.1.6 GPX MM Ambient | Verify temperature reading at factory ambient for erroneously min/max values. | Temperature sensor reads between 0 DegC to 40 DegC | ALL |
| 5.17.1.7 GPX U21 Ambient | Verify temperature reading at factory ambient for erroneously min/max values. | Temperature sensor reads between 0 DegC to 40 DegC | ALL |
| 5.17.1.8 GPX GPU Ambient | Verify temperature reading at factory ambient for erroneously min/max values. | Temperature sensor reads between 0 DegC to 40 DegC | ALL |
| 5.17.1.9 SCX3 HM Ambient | Verify temperature reading at factory ambient for erroneously min/max values. | Temperature sensor reads between 0 DegC to 40 DegC | ALL |

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 5.17.1.10 SCX3 CCA Ambient | Verify temperature reading at factory ambient for erroneously min/max values. | Temperature sensor reads between 0 DegC to 40 DegC | ALL |
| 5.17.1.11 SCX3 T2080 Ambient | Verify temperature reading at factory ambient for erroneously min/max values. | Temperature sensor reads between 0 DegC to 40 DegC | ALL |
| 5.17.1.12 IOX3 MM0 Ambient | Verify temperature reading at factory ambient for erroneously min/max values. | Temperature sensor reads between 0 DegC to 40 DegC | ALL |
| 5.17.1.13 IOX3 MM1 Ambient | Verify temperature reading at factory ambient for erroneously min/max values. | Temperature sensor reads between 0 DegC to 40 DegC | ALL |
| 5.17.1.14 IOX3 U47 Ambient | Verify temperature reading at factory ambient for erroneously min/max values. | Temperature sensor reads between 0 DegC to 40 DegC | ALL |
| 5.17.1.15 IOX3 U48 Ambient | Verify temperature reading at factory ambient for erroneously min/max values. | Temperature sensor reads between 0 DegC to 40 DegC | ALL |
| 5.17.1.16 IOX3 U50 Ambient | Verify temperature reading at factory ambient for erroneously min/max values. | Temperature sensor reads between 0 DegC to 40 DegC | ALL |
| 5.17.1.17 IOX3 U51 Ambient | Verify temperature reading at factory ambient for erroneously min/max values. | Temperature sensor reads between 0 DegC to 40 DegC | ALL |
| 5.17.1.18 VMX3 MM Ambient | Verify temperature reading at factory ambient for erroneously min/max values. | Temperature sensor reads between 0 DegC to 40 DegC | ALL |
| 5.17.1.19 VMX3 U42 Ambient | Verify temperature reading at factory ambient for erroneously min/max values. | Temperature sensor reads between 0 DegC to 40 DegC | ALL |
| 5.17.1.20 VMX3 U47 Ambient | Verify temperature reading at factory ambient for erroneously min/max values. | Temperature sensor reads between 0 DegC to 40 DegC | ALL |
| 5.17.1.21 VMX3 U48 Ambient | Verify temperature reading at factory ambient for erroneously min/max values. | Temperature sensor reads between 0 DegC to 40 DegC | ALL |
| 5.17.1.22 VMX3 U88 Ambient | Verify temperature reading at factory ambient for erroneously min/max values. | Temperature sensor reads between 0 DegC to 40 DegC | ALL |
| 5.17.1.23 VMX3 U89 Ambient | Verify temperature reading at factory ambient for erroneously min/max values. | Temperature sensor reads between 0 DegC to 40 DegC | ALL |
| 5.17.1.24 VMX3 MRV Ambient | Verify temperature reading at factory ambient for erroneously min/max values. | Temperature sensor reads between 0 DegC to 40 DegC | ALL |
| 5.17.1.25 RIO-M ODL Ambient | Verify temperature reading at factory ambient for erroneously min/max values. | Temperature sensor reads between 0 DegC to 40 DegC | ALL |
| 5.17.1.26 RIO-M ADC Ambient | Verify temperature reading at factory ambient for erroneously min/max values. | Temperature sensor reads between 0 DegC to 40 DegC | ALL |
| 5.17.1.27 RIO-D Ambient | Verify temperature reading at factory ambient for erroneously min/max values. | Temperature sensor reads between 0 DegC to 40 DegC | ALL |

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

SUB ASSEMBLY TEST

This section describes the sub assembly tests that shall be performed. All tests listed within this section shall be performed on all units listed in Table 1 in the manufacturing environment unless otherwise specified. Each test for each sub-assembly will have its own section number 6.N with the associated table.

Not Applicable

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

CONFIGURATION TEST

### Configuration Verification

This section describes the test(s) which will verify the final configuration of the product (specifically the software and firmware loaded on the unit). The ATP is not the controlling document for determining correct software loads, therefore, BOMs, HSIDs and other product documents control what is to be considered correct and identified here.

Purpose

Verify the final configuration of the UUT.

Product State or Condition

Apply power per section 3.4.2

| Test ID/Test Name | Measurement Description | Pass Criteria | Applicable |
| --- | --- | --- | --- |
| 7.1.1.1 Serial Number Load | Send Serial Number Load command to UUT along with the UUT’s serial number. Verify serial number loaded. | Serial number is correct | ALL |
| 7.1.1.2 Fault Log Clear | Send Fault Log Clear Call command to UUT. Verify fault log cleared. | Cleared fault log | ALL |
| 7.1.1.3 Operational Software | Install and verify operational software loaded. | Correct SW loaded | ALL |

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

SUPPLEMENTAL INFORMATION

This section contains “For Reference Only” information provided as a convenience, such as

figures or UUT block diagrams.

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

TEST DATA SHEET

This section serves as the information that is required to be captured in the artifact of test during the determination of acceptance for the specified unit under test. It should line up with the rest of the test procedure, with test section numbering, titles, and values matching. All data should be captured in a quantitative manner whenever possible. If the measured data is a large data array/package, modify the test procedure accordingly. If this ATP is implemented in a completely manual test setup, a full template will need to be created in the respective ATM to capture all the tests implemented in that ATM.

The following information will be captured for each unit tested

- UUT Part Number

- MCN/Serial Number

- Tested By

- ATP Part Number

- ATM Part Number

- Test Software Part Number

- UUT Revision

- Date Tested

- Test Procedure Revision

- Test Method Revision

- Test Software Revision

- Test ID/Test Name

- USL/LSL or Pass Criteria

- Actual result

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

INTERFACE TEST COVERAGE

#### The purpose of this section is to help reduce escape risk by identifying each signal being tested or not tested. All physical input and output pins shall be listed (regardless of being tested or not) and shall be correlated to at least one specific test ID or heading number in section 5 and/or 6. If not tested, comments such as ‘No Test’ should be used with a few more words to describe why (No Test-Spare, No Test-Test Signal, No Test-No Connect). Ground pins should be identified as GND.

| Connector/Pin | Signal Name (ICD) | ATP Test ID/Section |
| --- | --- | --- |
| J1-1 | POSITION_IN_P | 5.3 |
| J1-2 | DISCIN_28V_OPEN_1 | 5.4.1 |
| J1-3 | MILBUS0_A_DP | 5.10.1 |
| J1-4 | MILBUS0_A_DN | 5.10.1 |
| J1-5 | MILBUS0_B_DP | 5.10.1 |
| J1-6 | MILBUS0_B_DN | 5.10.1 |
| J1-7 | DIMMING_IN_N | GND |
| J1-8 | DISCIN_28V_OPEN_2 | 5.4.1 |
| J1-9 | SPARE | No Test-No Connect |
| J1-10 | CURSOR_X | 5.15 |
| J1-11 | CHASSIS GROUND | GND |
| J1-12 | CHASSIS GROUND | GND |
| J1-13 | 115VAC_L | 5.1.1 |
| J1-14 | DIMMING_IN_N | 5.13 |
| J1-15 | SPARE | No Test-Spare |
| J1-16 | POSITION_IN_1 | 5.3 |
| J1-17 | SLOT_IN_P | 5.3 |
| J1-18 | STRAP_RETURN | GND |
| J1-19 | SPARE | No Test-Spare |
| J1-20 | CURSOR_Y | 5.15 |
| J1-21 | DISCIN_28V_OPEN_0 | 5.4.1 |
| J1-22 | NVIS_IN_F | 5.3 |
| J1-23 | CHASSIS GROUND | GND |
| J1-24 | CHASSIS GROUND | GND |
| J1-25 | SPARE | No Test-Spare |
| J1-26 | SPARE | No Test-Spare |
| J1-27 | 115VAC_H | 5.1.1 |
| J1-28 | SLOT_IN_3 | 5.3 |
| J1-29 | AC_STRAP_3 | 5.3 |
| J1-30 | SPARE | No Test-Spare |
| J1-31 | DISCIN_GND_OPEN_18 | 5.3 |
| J1-32 | SPARE | No Test-Spare |
| J1-33 | CHASSIS GROUND | 5.1.1 |
| J1-34 | SPARE | No Test-Spare |
| J1-35 | DISCOUT_GND_OPEN_7 | 5.5 |
| J1-36 | SPARE | No Test-Spare |
| J1-37 | SPARE | No Test-Spare |

| Connector/Pin | Signal Name (ICD) | ATP Test ID/Section |
| --- | --- | --- |
| J1-38 | SPARE | No Test-Spare |
| J1-39 | AC_STRAP_4 | 5.3 |
| J1-40 | AC_STRAP_2 | 5.3 |
| J1-41 | AC_STRAP_1 | 5.3 |
| J1-42 | STRAP_RETURN | GND |
| J1-43 | SPARE | No Test-Spare |
| J1-44 | SPARE | No Test-Spare |
| J1-45 | CHASSIS GROUND | GND |
| J1-46 | SPARE | No Test-Spare |
| J1-47 | DISCOUT_GND_OPEN_6 | 5.5 |
| J1-48 | SPARE | No Test-Spare |
| J1-49 | SLOT_IN_1 | 5.3 |
| J1-50 | DISCIN_GND_OPEN_16 | 5.3 |
| J1-51 | AC_STRAP_5 | 5.3 |
| J1-52 | SPARE | No Test-Spare |
| J1-53 | SPARE | No Test-Spare |
| J1-54 | DIGITAL GND | GND |
| J1-55 | RS232_EUT_RXA | No Test – Test Input |
| J1-56 | CHASSIS GROUND | GND |
| J1-57 | SPARE | No Test-Spare |
| J1-58 | SPARE | No Test-Spare |
| J1-59 | CUR_BUTN3 | 5.3 |
| J1-60 | SPARE | No Test-Spare |
| J1-61 | POSITION_IN_0 | 5.3 |
| J1-62 | DISCIN_GND_OPEN_17 | 5.3 |
| J1-63 | SPARE | No Test-Spare |
| J1-64 | DIGITAL GND | GND |
| J1-65 | RS232_EUT_TXA | No Test – Test Input |
| J1-66 | SPARE | No Test-Spare |
| J1-67 | SPARE | No Test-Spare |
| J1-68 | CHASSIS GROUND | GND |
| J1-69 | CHASSIS GROUND | GND |
| J1-70 | CHASSIS GROUND | GND |
| J1-71 | SPARE | No Test-Spare |
| J1-72 | SLOT_IN_2 | 5.3 |
| J1-73 | DISCIN_GND_OPEN_22 | 5.3 |
| J1-74 | SPARE | No Test-Spare |
| J1-75 | SPARE | No Test-Spare |
| J1-76 | SPARE | No Test-Spare |
| J1-77 | SPARE | No Test-Spare |
| J1-78 | SPARE | No Test-Spare |
| J1-79 | CHASSIS GROUND | GND |
| J1-80 | SPARE | No Test-Spare |
| J1-81 | DISCOUT_GND_OPEN_5 | 5.5 |
| J1-82 | CUR_BUTN2 | 5.3 |

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

| Connector/Pin | Signal Name (ICD) | ATP Test ID/Section |
| --- | --- | --- |
| J1-83 | SPARE | No Test-Spare |
| J1-84 | DISCIN_GND_OPEN_21 | 5.3 |
| J1-85 | SPARE | No Test-Spare |
| J1-86 | SPARE | No Test-Spare |
| J1-87 | STRAP_RETURN | GND |
| J1-88 | RESERVED | No Test-No Connect |
| J1-89 | RESERVED | No Test-No Connect |
| J1-90 | RESERVED | No Test-No Connect |
| J1-91 | CHASSIS GROUND | GND |
| J1-92 | SPARE | No Test-Spare |
| J1-93 | DISCOUT_GND_OPEN_3 | 5.5 |
| J1-94 | SLOT_IN_4 | 5.3 |
| J1-95 | CHASSIS GROUND | GND |
| J1-96 | SLOT_IN_0 | 5.3 |
| J1-97 | DATALOAD_IN_F | 5.3 |
| J1-98 | SPARE | No Test-Spare |
| J1-99 | SPARE | No Test-Spare |
| J1-100 | SPARE | No Test-Spare |
| J1-101 | SPARE | No Test-Spare |
| J1-102 | CHASSIS GROUND | GND |
| J1-103 | CHASSIS GROUND | GND |
| J1-104 | DISCOUT_GND_OPEN_4 | 5.5 |
| J1-105 | SPARE | No Test-Spare |
| J1-106 | SPARE | No Test-Spare |
| J1-107 | SPARE | No Test-Spare |
| J1-108 | RESERVED | No Test-No Connect |
| J1-109 | HEATER_DISABLE_IN_F | 5.3 |
| J1-110 | MSC_PWR_P7_5V | 5.15.3 |
| J1-111 | MSC_PWR_N7_5V | 5.15.3 |
| J1-112 | SPARE | No Test-Spare |
| J1-113 | CHASSIS GROUND | GND |
| J1-114 | CHASSIS GROUND | GND |
| J1-115 | CUR_BUTN1 | 5.3 |
| J1-116 | WOW_IN_F | 5.3 |
| J1-117 | RESERVED | No Test-No Connect |
| J1-118 | CHASSIS GROUND | GND |
| J1-119 | DISCIN_GND_OPEN_19 | 5.3 |
| J1-120 | STRAP_RETURN | GND |
| J1-121 | DIMMING_IN_P | 5.13 |
| J1-122 | DISCIN_GND_OPEN_20 | 5.3 |
| J1-123 | RESERVED | No Test-No Connect |
| J1-124 | SPARE | No Test-Spare |
| J1-125 | SPARE | No Test-Spare |
| J1-126 | SPARE | No Test-Spare |
| J1-127 | SPARE | No Test-Spare |

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

| Connector/Pin | Signal Name (ICD) | ATP Test ID/Section |
| --- | --- | --- |
| J1-128 | DIMMING_IN_P | 5.13 |
| J2-1 | AVIN_0_B_TERM | 5.11.5 |
| J2-2 | AVIN_1_B_P | 5.11.5 |
| J2-3 | AVIN_1_B_TERM | 5.11.5 |
| J2-4 | AVIN_3_B_P | 5.11.5 |
| J2-5 | AVIN_0_G_N | 5.11.5 |
| J2-6 | AVIN_G_TERM | 5.11.5 |
| J2-7 | AVIN_1_G_P | 5.11.5 |
| J2-8 | AVIN_0_B_N | 5.11.5 |
| J2-9 | AVIN_1_B_N | 5.11.5 |
| J2-10 | AVIN_3_B_N | 5.11.5 |
| J2-11 | AVIN_3_B_TERM | 5.11.5 |
| J2-12 | AVIN_0_G_P | 5.11.5 |
| J2-13 | AVIN_1_G_N | 5.11.5 |
| J2-14 | AVIN_3_G_N | 5.11.5 |
| J2-15 | AVIN_0_R_N | 5.11.5 |
| J2-16 | AVIN_0_R_P | 5.11.5 |
| J2-17 | AVIN_0_B_P | 5.11.5 |
| J2-18 | SPARE | No Test-Spare |
| J2-19 | SPARE | No Test-Spare |
| J2-20 | HOMER_SIG+ | 5.15.1 |
| J2-21 | HOMER_SIG- | 5.15.1 |
| J2-22 | AVIN_1_G_TERM | 5.11.5 |
| J2-23 | AVIN_3_G_P | 5.11.5 |
| J2-24 | AVIN_3_G_TERM | 5.11.5 |
| J2-25 | AVIN_1_R_P | 5.11.5 |
| J2-26 | AVIN_0_R_TERM | 5.11.5 |
| J2-27 | HOMER_SIG_D | 5.15.1 |
| J2-28 | SPARE | No Test-Spare |
| J2-29 | CHASSIS GROUND | GND |
| J2-30 | SPARE | No Test-Spare |
| J2-31 | SPARE | No Test-Spare |
| J2-32 | HOMER_STEER- | 5.15.1 |
| J2-33 | HOMER_STEER+ | 5.15.1 |
| J2-34 | A429_IN1_H | 5.7 |
| J2-35 | A429_IN1_L | 5.7 |
| J2-36 | AVIN_1_R_TERM | 5.11.5 |
| J2-37 | AVIN_1_R_N | 5.11.5 |
| J2-38 | WXR_0_N | 5.8 |
| J2-39 | WXR_0_P | 5.8 |
| J2-40 | WXR_0_TERM_N | 5.8 |
| J2-41 | CHASSIS GROUND | GND |
| J2-42 | CHASSIS GROUND | GND |
| J2-43 | CHASSIS GROUND | GND |
| J2-44 | RS232_EUT_RXB | No Test – Test input |

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

| Connector/Pin | Signal Name (ICD) | ATP Test ID/Section |
| --- | --- | --- |
| J2-45 | DIGITAL GND | GND |
| J2-46 | CHASSIS GROUND | GND |
| J2-47 | A429_OUT1_H | 5.6 |
| J2-48 | AVIN_3_R_P | 5.11.5 |
| J2-49 | DISCIN_GND_OPEN_27 | 5.3 |
| J2-50 | DISCIN_28V_OPEN_3 | 5.4.1 |
| J2-51 | CHASSIS GROUND | GND |
| J2-52 | DISCIN_28V_OPEN_4 | 5.4.1 |
| J2-53 | ATT_REV | 5.3 |
| J2-54 | A429_IN2_H | 5.7 |
| J2-55 | A429_IN2_L | 5.7 |
| J2-56 | RS232_EUT_TXB | No Test – Test input |
| J2-57 | DIGITAL GND | GND |
| J2-58 | A429_OUT1_L | 5.6 |
| J2-59 | AVIN_3_R_TERM | 5.11.5 |
| J2-60 | AVIN_3_R_N | 5.11.5 |
| J2-61 | SPARE | No Test-Spare |
| J2-62 | DISCOUT_GND_OPEN_1 | 5.3 |
| J2-63 | CHASSIS GROUND | GND |
| J2-64 | WXR_1_N | 5.8 |
| J2-65 | WXR_1_P | 5.8 |
| J2-66 | WXR_1_TERM_N | 5.8 |
| J2-67 | A429_IN3_L | 5.7 |
| J2-68 | A429_IN3_H | 5.7 |
| J2-69 | SPARE | No Test-Spare |
| J2-70 | SPARE | No Test-Spare |
| J2-71 | DISCIN_GND_OPEN_23 | 5.3 |
| J2-72 | DISCIN_GND_OPEN_24 | 5.3 |
| J2-73 | DISCOUT_GND_OPEN_2 | 5.5 |
| J2-74 | DISCOUT_GND_OPEN_0 | 5.5 |
| J2-75 | HDG_REV | 5.3 |
| J2-76 | A429_IN4_H | 5.7 |
| J2-77 | A429_IN4_L | 5.7 |
| J2-78 | A429_IN7_L | 5.7 |
| J2-79 | A429_IN7_H | 5.7 |
| J2-80 | ADC_REV | 5.3 |
| J2-81 | FMS_REV | 5.3 |
| J2-82 | SPARE | No Test-Spare |
| J2-83 | SPARE | No Test-Spare |
| J2-84 | DCV_REV | 5.3 |
| J2-85 | SPARE | No Test-Spare |
| J2-86 | SPARE | No Test-Spare |
| J2-87 | SPARE | No Test-Spare |
| J2-88 | DISCIN_GND_OPEN_25 | 5.3 |
| J2-89 | DISCIN_GND_OPEN_28 | 5.3 |

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

| Connector/Pin | Signal Name (ICD) | ATP Test ID/Section |
| --- | --- | --- |
| J2-90 | SPARE | No Test-Spare |
| J2-91 | SPARE | No Test-Spare |
| J2-92 | SPARE | No Test-Spare |
| J2-93 | SPARE | No Test-Spare |
| J2-94 | SPARE | No Test-Spare |
| J2-95 | DISCIN_GND_OPEN_26 | 5.3 |
| J2-96 | SPARE | No Test-Spare |
| J2-97 | SPARE | No Test-Spare |
| J2-98 | SPARE | No Test-Spare |
| J2-99 | SPARE | No Test-Spare |
| J2-100 | SPARE | No Test-Spare |
| J2-101 | SPARE | No Test-Spare |
| J2-102 | SPARE | No Test-Spare |
| J2-103 | SPARE | No Test-Spare |
| J2-104 | RS-422_485_RX3_L | 5.12 |
| J2-105 | SPARE | No Test-Spare |
| J2-106 | SPARE | No Test-Spare |
| J2-107 | SPARE | No Test-Spare |
| J2-108 | SPARE | No Test-Spare |
| J2-109 | SPARE | No Test-Spare |
| J2-110 | SPARE | No Test-Spare |
| J2-111 | SPARE | No Test-Spare |
| J2-112 | DISCIN_28V_OPEN_5 | 5.4 |
| J2-113 | A429_IN9_H | 5.7 |
| J2-114 | RS-422_485_RX3_H | 5.12 |
| J2-115 | RS-422_485_TX3_L | 5.12 |
| J2-116 | A429_OUT4_H | 5.6 |
| J2-117 | A429_OUT4_L | 5.6 |
| J2-118 | CHASSIS GROUND | GND |
| J2-119 | A429_IN15_L | 5.7 |
| J2-120 | A429_IN15_H | 5.7 |
| J2-121 | A429_IN9_L | 5.7 |
| J2-122 | RS-422_485_TX3_H | 5.12 |
| J2-123 | RS-422_485_TX1_L | 5.12 |
| J2-124 | RS-422_485_TX1_H | 5.12 |
| J2-125 | RS-422_485_RX1_L | 5.12 |
| J2-126 | RS-422_485_RX1_H | 5.12 |
| J2-127 | RESERVED | No Test-No Connect |
| J2-128 | SPARE | No Test-Spare |
| J3-1 | SPARE | No Test-Spare |
| J3-2 | SPARE | No Test-Spare |
| J3-3 | RAD_ALT_N | 5.15.1 |
| J3-4 | SPARE | No Test-Spare |
| J3-5 | RAD_ALT_P | 5.15.1 |
| J3-6 | SPARE | No Test-Spare |

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

| Connector/Pin | Signal Name (ICD) | ATP Test ID/Section |
| --- | --- | --- |
| J3-7 | RAD_ALT_VALID | 5.4.4 |
| J3-8 | CURSOR_DRV_IN+ | 5.15.1 |
| J3-9 | SPARE | No Test-Spare |
| J3-10 | SPARE | No Test-Spare |
| J3-11 | SPARE | No Test-Spare |
| J3-12 | SPARE | No Test-Spare |
| J3-13 | SPARE | No Test-Spare |
| J3-14 | SPARE | No Test-Spare |
| J3-15 | SPARE | No Test-Spare |
| J3-16 | SPARE | No Test-Spare |
| J3-17 | SPARE | No Test-Spare |
| J3-18 | SPARE | No Test-Spare |
| J3-19 | CURSOR_DRV_IN- | 5.15.1 |
| J3-20 | SPARE | No Test-Spare |
| J3-21 | SPARE | No Test-Spare |
| J3-22 | SPARE | No Test-Spare |
| J3-23 | SPARE | No Test-Spare |
| J3-24 | SPARE | No Test-Spare |
| J3-25 | SPARE | No Test-Spare |
| J3-26 | SPARE | No Test-Spare |
| J3-27 | SPARE | No Test-Spare |
| J3-28 | RESERVED | No Test-No Connect |
| J3-29 | CHASSIS GND | GND |
| J3-30 | RS-422_485_TX2_L | 5.12 |
| J3-31 | RS-422_485_TX2_H | 5.12 |
| J3-32 | A429_IN10_L | 5.7 |
| J3-33 | A429_IN10_H | 5.7 |
| J3-34 | A429_IN11_H | 5.7 |
| J3-35 | A429_IN11_L | 5.7 |
| J3-36 | SPARE | No Test-Spare |
| J3-37 | SPARE | No Test-Spare |
| J3-38 | SPARE | No Test-Spare |
| J3-39 | SPARE | No Test-Spare |
| J3-40 | SPARE | No Test-Spare |
| J3-41 | DHM_SHOP_MODE_IN_F | 5.3 |
| J3-42 | CHASSIS GND | GND |
| J3-43 | A429_IN12_H | 5.7 |
| J3-44 | A429_OUT3_L | 5.6 |
| J3-45 | A429_OUT3_H | 5.6 |
| J3-46 | A429_IN13_H | 5.7 |
| J3-47 | A429_IN13_L | 5.7 |
| J3-48 | SPARE | No Test-Spare |
| J3-49 | A429_IN14_L | 5.6 |
| J3-50 | A429_IN14_H | 5.6 |
| J3-51 | RESERVED | No Test-No Connect |

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

| Connector/Pin | Signal Name (ICD) | ATP Test ID/Section |
| --- | --- | --- |
| J3-52 | RESERVED | No Test-No Connect |
| J3-53 | RESERVED | No Test-No Connect |
| J3-54 | A429_IN12_L | 5.6 |
| J3-55 | A582_IN0_L | 5.6 |
| J3-56 | A582_IN0_H | 5.6 |
| J3-57 | SPARE | No Test-Spare |
| J3-58 | SPARE | No Test-Spare |
| J3-59 | A429_OUT0_L | 5.6 |
| J3-60 | A429_OUT0_H | 5.6 |
| J3-61 | SPARE | No Test-Spare |
| J3-62 | A429_IN6_H | 5.7 |
| J3-63 | A429_IN6_L | 5.7 |
| J3-64 | RESERVED | No Test-No Connect |
| J3-65 | A429_IN5_L | 5.7 |
| J3-66 | A429_IN5_H | 5.7 |
| J3-67 | SPARE | No Test-Spare |
| J3-68 | SPARE | No Test-Spare |
| J3-69 | DHM_DATA_LOAD_IN_F | 5.3 |
| J3-70 | DISCIN_GND_OPEN_13 | 5.3 |
| J3-71 | A429_IN8_H | 5.7 |
| J3-72 | A429_IN8_L | 5.7 |
| J3-73 | CURSOR_RTN_IN | GND |
| J3-74 | CURSOR_RTN | GND |
| J3-75 | CHASSIS GND | GND |
| J3-76 | SPARE | No Test-Spare |
| J3-77 | SPARE | No Test-Spare |
| J3-78 | CUR_BUTN4 | 5.3 |
| J3-79 | RESERVED | No Test-No Connect |
| J3-80 | SPARE | No Test-Spare |
| J3-81 | SPARE | No Test-Spare |
| J3-82 | CHASSIS GND | GND |
| J3-83 | SPARE | No Test-Spare |
| J3-84 | CURSOR_X | 5.15 |
| J3-85 | CURSOR_Y | 5.15 |
| J3-86 | CHASSIS GND | GND |
| J3-87 | CURSOR_X_2 | 5.15 |
| J3-88 | SPARE | No Test-Spare |
| J3-89 | CUR_BUTN5 | 5.3 |
| J3-90 | ZEROIZE | 5.3 |
| J3-91 | CURSOR_Y_2 | 5.15 |
| J3-92 | SPARE | No Test-Spare |
| J3-93 | DISCIN_GND_OPEN_14 | 5.3 |
| J3-94 | SPARE | No Test-Spare |
| J3-95 | SPARE | No Test-Spare |
| J3-96 | SPARE | No Test-Spare |

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

| Connector/Pin | Signal Name (ICD) | ATP Test ID/Section |
| --- | --- | --- |
| J3-97 | SPARE | No Test-Spare |
| J3-98 | RS-422_485_RX2_L | 5.12 |
| J3-99 | RS-422_485_RX2_H | 5.12 |
| J3-100 | CUR_BUTN3 | 5.3 |
| J3-101 | CUR_BUTN2 | 5.3 |
| J3-102 | DIGITAL GND | GND |
| J3-103 | CUR_BUTN1 | 5.3 |
| J3-104 | DISCIN_GND_OPEN_15 | 5.3 |
| J3-105 | SPARE | No Test-Spare |
| J3-106 | SPARE | No Test-Spare |
| J3-107 | SPARE | No Test-Spare |
| J3-108 | MSC_PWR_P7_5V | 5.15.3 |
| J3-109 | MSC_PWR_N7_5V | 5.15.3 |
| J3-110 | SPARE | No Test-Spare |
| J3-111 | DHM_RS485_N | No Test – Test Input |
| J3-112 | DHM_RS485_P | No Test – Test Input |
| J3-113 | SPARE | No Test-Spare |
| J3-114 | SPARE | No Test-Spare |
| J3-115 | A429_OUT2_H | 5.6 |
| J3-116 | SPARE | No Test-Spare |
| J3-117 | SPARE | No Test-Spare |
| J3-118 | SPARE | No Test-Spare |
| J3-119 | SPARE | No Test-Spare |
| J3-120 | RESERVED | No Test-No Connect |
| J3-121 | A429_OUT6_L | 5.6 |
| J3-122 | A429_OUT2_L | 5.6 |
| J3-123 | A429_OUT5_H | 5.6 |
| J3-124 | A429_OUT5_L | 5.6 |
| J3-125 | A429_OUT7_H | 5.6 |
| J3-126 | A429_OUT7_L | 5.6 |
| J3-127 | RESERVED | No Test-No Connect |
| J3-128 | A429_OUT6_H | 5.6 |
| J4-A | 28VDC | 5.1.1 |
| J4-B | NO CONNECTION | No Test-No Connect |
| J4-C | NO CONNECTION | No Test-No Connect |
| J4-D | SAFETY GROUND | GND |
| J4-E | 28V RETURN | GND |
| J4-F | NO CONNECTION | No Test-No Connect |
| J5-A1 | ENET_1_B+ | 5.9 |
| J5-A2 | ENET_1_B- | 5.9 |
| J5-A3 | ENET_1_A+ | 5.9 |
| J5-A4 | ENET_1_A- | 5.9 |
| J5-A5 | GND | GND |
| J5-A6 | GND | GND |
| J5-B1 | ENET_2_B+ | 5.9 |

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06

| Connector/Pin | Signal Name (ICD) | ATP Test ID/Section |
| --- | --- | --- |
| J5-B2 | ENET_2_B- | 5.9 |
| J5-B3 | ENET_2_A+ | 5.9 |
| J5-B4 | ENET_2_A- | 5.9 |
| J5-B5 | GND | GND |
| J5-B6 | GND | GND |
| J5-C1 | ENET_2_D+ | 5.9 |
| J5-C2 | ENET_2_D- | 5.9 |
| J5-C3 | ENET_2_C+ | 5.9 |
| J5-C4 | ENET_2_C- | 5.9 |
| J5-C5 | GND | GND |
| J5-C6 | GND | GND |
| J5-D1 | ENET_1_D+ | 5.9 |
| J5-D2 | ENET_1_D- | 5.9 |
| J5-D3 | ENET_1_C+ | 5.9 |
| J5-D4 | ENET_1_C- | 5.9 |
| J5-D5 | GND | GND |
| J5-D6 | GND | GND |
| J6 | SMPTE_IN0 | 5.11.3 |
| J6 | SMPTE_SHIELD | 5.11.3 |
| J7 | SMPTE_IN1 | 5.11.3 |
| J7 | SMPTE_SHIELD | 5.11.3 |
| J8-A | A818_2_RX | 5.11.1 |
| J8-B | A818_1_RX | 5.11.1 |
| J8-C | A818_1_TX | 5.11.1 |
| J8-D | A818_2_TX | 5.11.1 |

U.S. Export Classification: EAR 9E610.a

Uncontrolled: Subject to change without notice. This export classification marking supersedes any and all other export classifications and export markings which may be contained in this document.

STATE 3 - ENGINEERING RELEASE 2026-05-06