ID :  C4B_ATR-3

Test_type :

Test_Venue :

ID :  C4B_ATR-4

Test_type :

Test_Venue :

ID :  C4B_ATR-6

# 1 Scope

Test_type :

Test_Venue :

ID :  C4B_ATR-7

This document defines Acceptance Test Requirements and Environmental Stress Screen (ESS) requirements for the MFD-268C4B Platform

Test_type :

Test_Venue :

ID :  C4B_ATR-8

# 2 References

Test_type :

Test_Venue :

ID :  C4B_ATR-153

946-46U3-001 Platform Requirements Specification for the Multifunction Display 268C4B

Test_type :

Test_Venue :

ID :  C4B_ATR-9

# 3 Product Description

Test_type :

Test_Venue :

ID :  C4B_ATR-10

The MFD-268C4B is a heads down display developed as an upgrade to the existing MFD-268C4A and is designed primarily for rotorcraft platforms.

Test_type :

Test_Venue :

ID :  C4B_ATR-11

# 4 Test Requirement

Test_type :

Test_Venue :

ID :  C4B_ATR-120

4.1 Test_type :

Test_Venue :

ID :  C4B_ATR-121

The MFD shall monitor the following Aircraft Interfaces for erroneous operation:

ARINC 429 Inputs and Outputs

RS-422/485

MIL-STD-1553

Analog Inputs, except for the Bezel Lighting Input

Analog Outputs

Discrete Inputs

Discrete Outputs

Test_type :  Loopback

Test_Venue :  HASS

Functional

ID :  C4B_ATR-143

The MFD shall perform start up tests of internal hardware devices to ensure device logic and interfaces are operational.

Test_type :  BIT

Test_Venue :  HASS

Functional

ID :  C4B_ATR-145

The MFD shall perform CBIT to ensure that it is operating properly and can respond to failure.

Test_type :  BIT

Test_Venue :  HASS

Functional

ID :  C4B_ATR-12

## 4.2 Discrete Input Requirements

Test_type :

Test_Venue :

ID :  C4B_ATR-32

### 4.2.1 Ground/Open Inputs

Test_type :

Test_Venue :

ID :  C4B_ATR-23

The MFD shall provide a minimum of twenty eight (28) General Purpose Ground/Open Discrete Inputs.

Test_type :  Loopback

Test_Venue :  HASS

Functional

ID :  C4B_ATR-28

The MFD shall provide five (5) Aircraft Type Straps as Ground/Open Discrete Inputs.

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-29

The MFD shall provide a Day/NVIS Select Ground/Open Discrete input

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-30

The MFD shall provide a Dataload Enable Ground/Open Discrete Input

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-31

The MFD shall provide a Heater Disable Ground/Open Discrete Input

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-33

### 4.2.2 Voltage/Open Inputs

Test_Venue :

ID :  C4B_ATR-34

The MFD shall provide a minimum of six (6) General Purpose 28V/Open Discrete Inputs.

Test_type :  Loopback

Test_Venue :  HASS

Functional

ID :  C4B_ATR-35

The MFD shall provide one (1) Radar Altitude Valid 5V/GND Discrete input.

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-13

## 4.3 Discrete Output Requirements

Test_Venue :

ID :  C4B_ATR-36

The MFD shall provide eight (8) Ground/Open Discrete Outputs.

Test_type :  Loopback

Test_Venue :  HASS

Functional

ID :  C4B_ATR-14

## 4.4 Strapping Requirements

Test_Venue :

ID :  C4B_ATR-24

The MFD shall provide five (5) Slot ID straps and one (1) Parity strap to determine the MFD position on the aircraft.

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-25

The MFD shall consider the Slot ID to be valid if the sum of the Slot Strapping inputs (ID and Parity) indicated as Closed (True) is odd.

Commentary: Odd parity is used for Slot ID validity

Test_type :  Direct

ID :  C4B_ATR-26

The MFD shall provide two MFD Position ID straps and one (1) MFD Position Parity strap to determine the MFD position on the aircraft.

Test_Venue :  HASS

Functional

ID :  C4B_ATR-27

The MFD shall consider the Position ID to be valid if the sum of the MFD Position Strapping (ID and Parity) inputs indicated as Closed (True) is odd.

Commentary: Odd parity is used for MFD Position ID validity

Test_type :  Direct

ID :  C4B_ATR-15

## 4.5 DC Analog Requirements

Test_type :

Test_Venue :

ID :  C4B_ATR-37

### 4.5.1 Inputs

Test_type :

ID :  C4B_ATR-42

#### 4.5.1.1 Type 2

Test_Venue :

ID :  C4B_ATR-43

The MFD shall provide two (2) Analog Type 2 Inputs.

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-44

Each Analog Type 2 Input shall have a measurement range of ±0.4 milliamps (mA).

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-45

Each Analog Type 2 Input shall have an accuracy of ±0.04 mA.

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-46

#### 4.5.1.2 Type 3

Test_type :

Test_Venue :

ID :  C4B_ATR-47

The MFD shall provide one (1) Analog Type 3 Input.

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-48

The Analog Type 3 Input shall have a measurement range of 0-16VDC.

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-49

The Analog Type 3 Input shall have an accuracy of ±150 mV.

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-50

#### 4.5.1.3 Type 4

Test_type :

Test_Venue :

ID :  C4B_ATR-51

The MFD shall provide one (1) Analog Type 4 Input.

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-52

The Analog Type 4 Input shall have a differential measurement range of -15 VDC to 0 VDC.

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-53

The Analog Type 4 Input shall have a total accuracy of ± (B0 + Bx) VDC with an M of .375 and A of .1 where B0 and Bx are defined as follows:

B0 = Offset Error = A/100 * (Full Scale Input Range)

Bx = Gain Error = M/100 * (Input Voltage)

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-54

#### 4.5.1.4 Type 5A

Test_type :

Test_Venue :

ID :  C4B_ATR-55

The MFD shall provide 2 Analog Type 5A Inputs (Cursor X & Y)

Test_Venue :  HASS

Functional

ID :  C4B_ATR-56

Each Analog Type 5A Input shall have a measurement range of ±7.5 VDC

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-57

Each Analog Type 5A Input shall have an accuracy of ±75 mV.

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-58

#### 4.5.1.5 Type 5B

Test_Venue :

ID :  C4B_ATR-59

Test_type :  Direct

Test_Venue :  HASS

Functional

Each Analog Type 5B Input shall have a measurement range of ±7.5 VDC

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-62

Each Analog Type 5A Input shall have an accuracy of ±75 mV.

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-38

### 4.5.2 Outputs

Test_type :

Test_Venue :

ID :  C4B_ATR-39

The MFD shall provide one (1) Analog Type 1 Output.

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-40

The Analog Type 1 Output shall consist of two fixed voltage DC outputs of 7.5 VDC and -7.5 VDC.

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-41

The accuracy of each fixed voltage DC output shall be ±75 mV.

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-16

## 4.6 Bezel Requirements

Test_type :

Test_Venue :

ID :  C4B_ATR-61

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-63

The Bezel Lighting Input shall have a differential input range of 0 VDC - 5 VDC plus a common mode input voltage range of ± 1VDC.

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-106

The MFD shall provide two state ON-OFF mechanical control to control the operating state of the MFD.

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-107

The MFD shall provide an illuminated mechanical slip indicator centered along the bottom edge of the display bezel.

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-110

The MFD shall provide 24 illuminated tactile feel push button controls with 6 controls on either side of the display, 6 above the display, and 6 below display.

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-146

The MFD shall provide an illuminated 3 position Up/Down/Neutral mechanical control to control the display luminance.

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-108

The MFD shall increase the display luminance when the luminance control is pressed into the Up position.

Test_type :  Direct

Test_Venue :  Functional

Luminance

ID :  C4B_ATR-109

The MFD shall decrease the display luminance when the luminance control is pressed into the Down position.

Test_type :  Direct

Test_Venue :  Functional

Luminance

ID :  C4B_ATR-147

The MFD shall provide two illuminated 3 position Up/Down/Neutral mechanical controls for controlling Video Brightness and Video Contrast.

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-17

## 4.7 Graphics Requirements

Test_type :

Test_Venue :

ID :  C4B_ATR-114

The MFD shall monitor the status of the 2D graphics capability for erroneous behavior.

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-115

The MFD shall monitor the status of the 3D graphics capability for erroneous behavior.

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-116

The MFD shall detect erroneous operation of the display output.

Test_Venue :  HASS

Functional

ID :  C4B_ATR-140

The MFD shall report a loss of signal to the Hosted Applications when an enabled video input is lost.

Test_type :  BIT

Test_Venue :  HASS

Functional

ID :  C4B_ATR-84

Test_type :

Test_Venue :

ID :  C4B_ATR-85

The MFD shall provide 3 Analog Video Inputs.

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-86

When configured to accept RGB component inputs the Analog Video input shall accept video signals compliant to the following standards:

- STANAG 3350 Class A, B, and C color video

- XGA Progressive Scan (Sync on Green)

- RS-170 Monochrome video signal on green input

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-87

When configured to accept composite inputs the Analog Video input shall accept video signals compliant to the following standards:

- NTSC composite color video in accordance with SMPTE 170M.

- PAL-I composite color video in accordance with BT.470-6.

- RS-170 Monochrome video signal on green input

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-119

The MFD shall report a loss of sync fault to the Hosted Applications within 5 field times of losing synchronization with the input signal.

Test_type :  BIT

Test_Venue :  HASS

Functional

ID :  C4B_ATR-88

### 4.7.2 SMPTE 292

Test_type :

Test_Venue :

ID :  C4B_ATR-89

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-90

Each SMPTE 292 Digital Video Input shall support video signals in 720p, 1080i, and 1080p video formats.

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-118

The MFD shall report a fault to the Hosted Applications if a loss of signal is detected on an enable SMPTE 292 Input

Test_type :  BIT

Test_Venue :  HASS

Functional

ID :  C4B_ATR-91

### 4.7.3 ARINC 818

Test_type :

Test_Venue :

ID :  C4B_ATR-94

Test_type :

Test_Venue :

ID :  C4B_ATR-92

The MFD shall provide two (2) ARINC 818 Inputs.

Test_type :  Loopback

Test_Venue :  HASS

Functional

ID :  C4B_ATR-93

Each ARINC 818 Input shall be configurable by the Hosted Applications to operate at following bit rates, with a tolerance of ±100 ppm:

FC 3X (3.1875 Gbps)

FC 4X (4.25 Gbps)

Commentary: not all resolutions are support by each bit rate. It is the responsibility of the System Integrator to ensure the Hosted Applications configure the interface to an appropriate resolution for the selected bit rate.

Test_type :  Loopback

Test_Venue :  HASS

Functional

ID :  C4B_ATR-117

The MFD shall report a fault to the Hosted Applications if a loss of optical signal occurs on an enabled ARINC 818 video inputs.

Test_type :  BIT

Test_Venue :  HASS

Functional

ID :  C4B_ATR-95

#### 4.7.3.2 Output

Test_type :

Test_Venue :

ID :  C4B_ATR-96

The MFD shall provide two (2) ARINC 818 Outputs.

Test_type :  Loopback

Test_Venue :  HASS

Functional

ID :  C4B_ATR-97

Each ARINC 818 Output shall be configurable by the Hosted Applications to operate at following bit rates, with a tolerance of ±100 ppm:

FC 3X (3.1875 Gbps)

FC 4X (4.25 Gbps)

Commentary: not all resolutions are support by each bit rate. It is the responsibility of the system integrator to ensure the Hosted Applications configure the interface to an appropriate resolution for the selected bit rate.

Test_type :  Loopback

Test_Venue :  HASS

Functional

ID :  C4B_ATR-18

## 4.8 ARINC 708 (WXR) Requirements

Test_type :

Test_Venue :

ID :  C4B_ATR-78

The MFD shall provide two ARINC 708 Inputs.

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-79

Each ARINC 708 Input shall be compliant to the ARINC 708-6 standard.

Test_type :  Direct

Test_Venue :  HASS

Functional

## 4.9 ARINC 429 Requirements

Test_type :

Test_Venue :

ID :  C4B_ATR-67

### 4.9.1 Inputs

Test_type :

Test_Venue :

ID :  C4B_ATR-68

The MFD shall provide a minimum of sixteen (16) ARINC 429 Inputs compatible with the characteristics of ARINC 429P1-19.

Test_type :  Loopback

Test_Venue :  HASS

Functional

ID :  C4B_ATR-69

Each ARINC 429 input shall be independently configurable to operate in either high speed (100 Kbps) or low speed (12.5 Kbps) mode.

Test_type :  Loopback

Test_Venue :  HASS

Functional

ID :  C4B_ATR-70

Each ARINC 429 input shall be independently configurable to operate with odd or even parity.

Test_type :  Loopback

Test_Venue :  HASS

Functional

ID :  C4B_ATR-71

4.9.2 Test_type :

Test_Venue :

ID :  C4B_ATR-72

The MFD shall provide eight (8) ARINC 429 Outputs.

Test_type :  Loopback

Test_Venue :  HASS

Functional

ID :  C4B_ATR-73

Each A429 Output shall be independently configurable to operate in either high speed (100 Kbps) or low speed (12.5 Kbps) mode.

Test_type :  Loopback

Test_Venue :  HASS

Functional

ID :  C4B_ATR-74

Each ARINC 429 output shall be independently configurable to operate with odd, even, or none parity.

Test_type :  Loopback

Test_Venue :  HASS

Functional

ID :  C4B_ATR-65

## 4.10 RS-422/485 Requirements

Test_type :

Test_Venue :

ID :  C4B_ATR-75

The MFD shall provide three (3) RS-422/485 interfaces.

Test_type :  Loopback

Test_Venue :  HASS

Functional

ID :  C4B_ATR-76

Each RS-422/485 interface shall support full duplex operation.

Commentary: Full duplex operation requires dedicated TX and RX signal pairs.

Test_type :  Loopback

ID :  C4B_ATR-77

Each RS-422/485 interface shall support the following data rates, in bits per second:

- 9600

- 14400

- 19200

- 28800

- 38400

- 57600

- 76800

- 115200

- 153600

- 230400

- 460800

- 921600

- 1843200

Test_type :  Loopback

Test_Venue :  HASS

Functional

ID :  C4B_ATR-66

## 4.11 Ethernet Requirements

Test_type :

Test_Venue :

ID :  C4B_ATR-80

The MFD shall provide 2 Ethernet Interfaces.

Test_type :  Loopback

Test_Venue :  HASS

Functional

ID :  C4B_ATR-81

Each Ethernet interface shall support 10/100/1000Base-T operation.

Test_type :  Loopback

Test_Venue :  HASS

Functional

ID :  C4B_ATR-142

The MFD shall communicate with external applications using the MFD Ethernet interfaces.

Test_type :  Direct

ID :  C4B_ATR-19

Test_type :

Test_Venue :

ID :  C4B_ATR-82

The MFD shall provide 1 dual redundant (Bus A and Bus B) MIL-STD-1553 Interface.

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-83

The MIL-STD-1553 Interface shall operate in accordance with MIL-STD-1553B.

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-98

## 4.13 RS-232

Test_type :

Test_Venue :

ID :  C4B_ATR-99

The MFD shall provide a two RS-232 ports connected to the MFD processor.

Commentary: the intended use of the RS-232 ports is as a maintenance and debugging interface.

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-20

## 4.14 Input Power Requirements

Test_type :

Test_Venue :

ID :  C4B_ATR-138

Primary power returns shall be isolated from the MFD chassis ground by at least 5 MOhms at 28VDC.

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-139

The MFD power connector shall have a safety ground pin connected to chassis ground with a resistance less than 2.5 milliohms.

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-122

The worst case, steady state Primary power consumed by the MFD, excluding heater power, shall be 195 Watts or less.

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-144

The MFD shall operate from 16 to 29VDC.

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-123

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-124

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-125

The MFD shall meet all requirements specified herein when receiving multiple Primary Power interruptions spaced two or more seconds apart.

Commentary: Multiple interruptions spaced less than two seconds apart may result in either the MFD operating through the condition or reset with a warm start after the interruptions cease.

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-21

Test_type :

Test_Venue :

ID :  C4B_ATR-127

### 4.15.1 Day Mode

Test_type :

Test_Venue :

ID :  C4B_ATR-100

Test_type :  Direct

Test_Venue :  Luminance

ID :  C4B_ATR-101

When set to Day Mode and the minimum luminance input setting, the white (full-on) luminance at the center of the display as measured from the DEP shall be less than or equal 0.05 fL.

Test_type :  Direct

Test_Venue :  Luminance

ID :  C4B_ATR-102

In day mode the display shall have a luminance uniformity less than or equal to 0.3 when measured using a five by seven point grid centered on the display and extending to within 0.5 inches of the edge of the active area on the top and bottom.

Test_type :  Direct

Test_Venue :  Luminance

ID :  C4B_ATR-128

Test_type :

Test_Venue :

ID :  C4B_ATR-103

In NVIS Mode the display shall have a luminance uniformity less than or equal to 0.3 when measured using a five by seven point grid centered on the display and extending to within 0.5 inches of the edge of the active area on the top and bottom.

Test_type :  Direct

Test_Venue :  Luminance

ID :  C4B_ATR-131

The MFD shall be free of light leaks visible with the naked eye or a NVIS Class A viewing device. Light leaks are emissions from areas other than the display or intentionally illuminated bezel areas.

Test_type :  Direct

Test_Venue :  Luminance

ID :  C4B_ATR-130

### 4.15.3 Chromaticity

Test_type :

Test_Venue :

ID :  C4B_ATR-104

The display shall have a DEP Chroma Tolerance less than or equal to 26 in both Day Mode and NVIS Mode using the nominal color coordinates provided below:

Test_type :  Direct

Test_Venue :  Luminance

ID :  C4B_ATR-129

### 4.15.4 Bezel

Test_type :

Test_Venue :

ID :  C4B_ATR-135

The luminance of each bezel marking and illuminated control shall be 1.0 ± .5 fL when the Bezel Lighting Input is provided with 5 ± 0.1 VDC.

Test_type :  Direct

Test_Venue :  Luminance

ID :  C4B_ATR-136

The luminance of each bezel marking and illuminated control shall be less than 0.05 fL when the Bezel Lighting Input is provided with 0 ± 0.1 VDC.

Test_type :  Direct

Test_Venue :  Luminance

ID :  C4B_ATR-105

The MFD shall adjust the luminance of the Bezel Lighting based on the input voltage supplied to the Bezel Lighting Input following the voltage to output curve typical of M6363/5-8 lamps.

Test_type :  Direct

Test_Venue :  Luminance

ID :  C4B_ATR-22

## 4.16 Fan Requirements

Test_type :

Test_Venue :

ID :  C4B_ATR-113

The MFD shall monitor the cooling fan is operating properly.

Test_type :  BIT

Test_Venue :  HASS

Functional

ID :  C4B_ATR-126

The MFD shall receive 115 VAC power compliant to MIL-STD-704A Single Phase, 400 Hz, 115 VAC, Category B to power the fan.

Test_Venue :  HASS

Functional

ID :  C4B_ATR-111

4.17 Test_type :

Test_Venue :

ID :  C4B_ATR-112

The MFD shall operate at temperatures of -40°C to 55°C Continuous, 71°C short duration operation of 30 minutes minimum at sea level.

Test_type :  Direct

Test_Venue :  HASS

ID :  C4B_ATR-141

Test_type :  Direct

Test_Venue :  HASS

Functional

ID :  C4B_ATR-132

## 4.18 Defect Requirements

Test_type :

Test_Venue :

The display shall contain no more than 6 defects classified as bright dots

Test_type :  Inspections

Test_Venue :  Luminance

ID :  C4B_ATR-149

The display shall contain no more than 20 defects classified as black dots.

Test_type :  Inspections

Test_Venue :  Luminance

ID :  C4B_ATR-150

The display shall contain no more than two bright dots within a 5 mm diameter circle, and no more than 3 circles of this type.

Test_type :  Inspections

Test_Venue :  Luminance

ID :  C4B_ATR-151

The display shall contain no more than two black dots within a 5 mm diameter circle, and no more than 5 circles of this type.

Test_type :  Inspections

Test_Venue :  Luminance

ID :  C4B_ATR-152

The display may contain a single instance of 3 black dots within a 5 mm diameter circle. A circle of this type is included in the allowed number of 5mm black dot circles.

Test_type :  Inspections

Test_Venue :  Luminance

ID :  C4B_ATR-133

The display shall be free of failures of entire rows or columns of display elements.

Test_type :  Inspections

Test_Venue :  HASS

Functional

ID :  C4B_ATR-134

The display glass active area shall have no readily discernible defects or blemishes (air bubbles, cracks, discoloration, scratches, and extraneous matter) that result in false or misleading information when viewed at thirty inches anywhere in the primary viewing envelope, when illuminated to at least 100fL under ambient lighting of at least 500fc.

Commentary: The term "readily discernible" is defined as a luminance difference of greater than or equal to 20% relative to the surrounding background.

Test_type :  Inspections

Test_Venue :  HASS

Functional